| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the A-Preamble boosting levels for multi-carrier mode (16.3.6.1) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Youngbo Cho, Seonghyun Chae, Chiwoo Lim, Ki-chun Cho, Sung-Eun Park, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | youngbo.cho@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m |  |
| Abstract | The contribution proposes the text changes to A-Preamble boosting levels for multi-carrier mode of the 802.16m draft amendment, IEEE P802.16m/D7. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to A-Preamble boosting levels for multi-carrier mode (16.3.6.1)

Chiwoo Lim, Ki-chun Cho, Sung-Eun Park, Jaeweon Cho, Hokyu Choi, Heewon Kang
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of the A-Preamble boosting levels for multi-carrier to be included in the 802.16m amendment. In the current draft amendment [1], there is no description of A-Preamble boosting levels for multi-carrier mode. In calculation of boosting level of regular bandwidth, it’s chosen such that the maximum peak value of PA-Preamble/SA-Preamble in time domain should not exceed the peak value of 16e preamble which has 9dB boosting. However, in the multi-carrier, the peak powers of PA-Preamble/SA-Preamble with the present boosting levels are exceeded the dynamic range of ABS’s amplifier (which is same as 98%-tile value that is CDF of the peak values of data symbols). So, new reference values in multi-carrier mode are required.

- References

[1] IEEE P802.16m/D6, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-05-22

- Rationale of modifications

In D6 [1], A-Preamble boosting levels are chosen such that the maximum peak value of PA-Preamble/ SA-Preamble in the single-carrier case. However, in the multi-carrier case, the peak powers of PA-Preamble/ SA-Preamble with the present boosting levels are exceeded the dynamic range of ABS’s amplifier (which is same as 98%-tile value that is CDF of the peak values of data symbols).

Table 1 shows the 98%-tile CDF peak values of the data symbols

Table 1. 98%-tile CDF peak power values of data symbols (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | 0.68 | 1.01 | 1.19 |
| 1k | 0.95 | 1.27 | 1.29 |
| 2k | 1.22 | 1.50 | 1.52 |

Pilot randomization is applied for the simulation.

Table 2 shows the PA-Preamble peak power with present boosting levels.

Table 2. Boosted PA-Preamble Peak Power (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.08 | 1.49 | 3.13 |
| 1k | -1.09 | 1.78 | 3.41 |
| 2k | -0.66 | 2.35 | 3.99 |

Table 3-a, 3-b, 3-c, and 3-d present the SA-Preamble peak power with present boosting levels at the 1, 2, 3, and 4 antennas, respectively.

Table 3-a. Boosted SA-Preamble Peak Power with 1 antenna (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.06 | 1.93 | 3.57 |
| 1k | -0.99 | 1.83 | 3.47 |
| 2k | -0.36 | 2.64 | 4.28 |

Table 3-b. Boosted PA-Preamble Peak Power with 2 antennas (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.08 | 1.49 | 3.13 |
| 1k | -1.09 | 1.78 | 3.47 |
| 2k | -0.66 | 2.35 | 3.99 |

Table 3-c. Boosted PA-Preamble Peak Power with 4 antennas (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.06 | 1.83 | 3.49 |
| 1k | -1.00 | 2.01 | 3.64 |
| 2k | 0.72 | 3.71 | 5.35 |

Table 3-d. Boosted PA-Preamble Peak Power with 8 antennas (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.05 | 1.95 | 3.59 |
| 1k | -1.02 | 1.93 | 3.63 |
| 2k | -0.83 | 2.17 | 3.81 |

Simulation results show that the peak powers of PA-Preamble/SA-Preamble with the present boosting levels are exceeded the 98%-tile value that is CDF of the peak values of data symbols in the multi-carrier. Therefore, the modification of boosting values is required. Table 4-a~4-e present the modified boosting levels for PA-Preamble/ SA-Preamble which are chosen by following rules.

- The maximum peak value of PA-Preamble/SA-Preamble is same as 98%-tile value that is CDF of the peak values of data symbols.

- For 512 and 1024 FFT cases, the modified value is chosen at the carrier aggregation mode with 3 carriers.

- For 2048 FFT case, the modified value is chosen at the carrier aggregation mode with 2 carriers.

Table 4-a. Modified PA-Preamble boosting levels

| FFT | Present PA-Preamble boosting level | Modified PA-Preamble boosting level | Difference (dB) |
| --- | --- | --- | --- |
| 512 | 2.3999 (7.60 dB) | 1.9216 (5.67 dB) | -1.93 |
| 1k | 3.4143 (10.67 dB) | 2.6731 (8.54 dB) | -2.13 |
| 2k | 5.132 (14.21 dB) | 4.6511 (13.35 dB) | -0.85 |

Table 4-b. Modified SA-Preamble boosting levels with 1 antenna

| FFT | Present SA-Preamble boosting level with 1 ant | Modified PA-Preamble boosting level with 1 ant | Difference (dB) |
| --- | --- | --- | --- |
| 512 | 2.87 (5.44 dB) | 1.42 (3.06 dB) | -2.38 |
| 1k | 1.75 (4.86 dB) | 1.36 (2.69 dB) | -2.18 |
| 2k | 1.73 (4.76 dB) | 1.52 (3.62 dB) | -1.14 |

Table 4-c. Modified SA-Preamble boosting levels with 2 antennas

| FFT | Present SA-Preamble boosting level with 2 ant | Modified PA-Preamble boosting level with 2 ant | Difference (dB) |
| --- | --- | --- | --- |
| 512 | 2.51 (7.99 dB) | 1.96 (5.83 dB) | -2.17 |
| 1k | 2.33 (7.35 dB) | 1.78 (5.01 dB) | -2.34 |
| 2k | 2.43 (7.71 dB) | 2.16 (6.69 dB) | -1.02 |

Table 4-d. Modified SA-Preamble boosting levels with 4 antennas

| FFT | Present SA-Preamble boosting level with 4 ant | Modified PA-Preamble boosting level with 4 ant | Difference (dB) |
| --- | --- | --- | --- |
| 512 | 4.38 (12.83 dB) | 3.36 (10.53 dB) | -2.30 |
| 1k | 3.56 (11.03 dB) | 2.72 (8.68 dB) | -2.35 |
| 2k | 3.98 (12.00 dB) | 3.08 (9.78 dB) | -2.21 |

Table 4-e. Modified SA-Preamble boosting levels with 8 antennas

| FFT | Present SA-Preamble boosting level with 8 ant | Modified PA-Preamble boosting level with 8 ant | Difference (dB) |
| --- | --- | --- | --- |
| 512 | 8.67 (18.76 dB) | 6.58 (16.36 dB) | -2.40 |
| 1k | 6.25 (15.92 dB) | 4.78 (13.59 dB) | -2.33 |
| 2k | 5.13 (14.20 dB) | 4.75 (13.53 dB) | -0.67 |

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Modify the following text on page 499 line 53 as following text]

The magnitude boosting levels in single carrier mode for different FFT size are shown in Table 797.

The magnitude boosting levels for different FFT size are shown in Table XXX.

[Remedy-2: Modify the table 824 on page 499]

Table 824XXX – PA-Preamble boosting levels

| 512 | 1k | 2k |
| --- | --- | --- |
| 2.39991.9216 | 3.41432.6731 | 5.13204.6511 |

[Remedy-3: Modify the following text on page 508 line 46-48 as following text]

The magnitude boosting levels in single carrier mode for different FFT size and number of antennas are shown in Table 799.

The magnitude boosting levels for different FFT size and number of antennas are shown in Table YYY.

[Remedy-4: Modify the table 827 on page 508]

Table 827YYY – SA-Preamble boosting levels

| Ant\FFT | 512 | 1k | 2k |
| --- | --- | --- | --- |
| 1 | 1.87 1.42 | 1.751.36 | 1.731.52 |
| 2 | 2.511.96 | 2.331.78 | 2.432.16 |
| 4 | 4.383.36 | 3.562.72 | 3.983.08 |
| 8 | 8.676.58 | 6.254.78 | 5.134.75 |

---------------------------------------------------  Text End  ---------------------------------------------------