| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for paging method for E-MBS AMS in multi-carrier system (16.2.8.2) |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com |
| Re: | Sponsor Ballot comment on IEEE p802.16m D6 |  |
| Abstract | This contribution proposes the texts related to paging method for E-MBS AMS in multi-carrier system |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for paging method for E-MBS AMS in multi-carrier system (16.2.3.1/16.2.8.2/16.9.2.3)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, and Jin Sam Kwak

- LG Electronics

# Introduction

Paging carrier configuration in 16m/D6 multicarrier system [1]

As described in the Section 16.2.8.2.10.2 in 16m Draft [1], the paging carrier of the Idle mode AMS which does not receive E-MBS service is decided based on  DID of the AMS and N value (the number of carriers per PGID in an ABS that are used for transmitting paging message for idle mode AMS(s)) as follows:

Paging carrier index = DID modulo N – equation (1)

And the paging carrier for E-MBS AMS is the same as the carrier which provides the E-MBS.

E-MBS scenarios corresponding to accounting (or charging) methods

1) In Time/Traffic based accounting

- For correct charging an E-MBS AMS should inform E-MBS controller server how many or how long the AMS receives E-MBS traffic. Upper layer signaling (Joining/leaving procedure) will be used for this purpose. It means that an ABS can initiate DSx (DSA/DSD) procedure based on the upper layer signaling when the E-MBS AMS starts or finishes receiving the E-MBS service. That is, ABS can know if the AMS subscribing E-MBS currently receives E-MBS service. And, the AMS will switch from primary carrier to E-MBS carrier after DSA procedure in order to receive E-MBS service.

2) In subscription based accounting

- In this scenario, an E-MBS AMS does not need to inform E-MBS controller server of the E-MBS reception status. An E-MBS AMS gets the E-MBS connection information through the DSA procedure when an E-MBS AMS subscribes the E-MBS service or through the provisional DSA procedure after the AMS performs the initial network entry procedure. Joining/leaving procedure will not be used in this scenario and therefore the ABS cannot initiate the DSx procedure when the E-MBS AMS starts or finishes receiving the E-MBS service.. That is, ABS cannot know if the AMS currently receives E-MBS service. The method for informing the ABS of the start/end of E-MBS reception needs to be defined additionally. The AMS will switch from primary carrier to E-MBS carrier after informing ABS of the start of E-MBS reception.

Why should an ABS know if the AMS which subscribes E-MBS service currently receives E-MBS service for paging carrier configuration?

If an ABS does not know if an E-MBS AMS currently receives E-MBS service, the paging carrier for the AMS is not selected properly when an Idle mode AMS starts/finishes receiving the E-MBS as shown figure 1.

Figure 1.

Therefore, in idle mode the AMS which subscribes the E-MBS service should inform ABS of the start/end of E-MBS reception. If the AMS subscribing E-MBS currently receives E-MBS data, the AAI_PAG-ADV of the AMS is transmitted in the carrier carrying the E-MBS data. Otherwise, the AAI_PAG-ADV is transmitted in the carrier indicated by DID % N.

# Proposal

1] E-MBS reception status report

During DSA procedure an ABS notifies an E-MBS AMS if the AMS in idle mode informs the ABS of the start/end of the E-MBS data reception or not.

Table 1. AAI_DSA-REQ/RSP

| M/O | Attributes/Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … |  |
| O | EMBS reception status report required | 1 | Indicates if it is required for an AMS to inform ABS of the E-MBS reception status (the start/end of E-MBS reception). - 0b0: The AMS is not required to inform ABS of the E-MBS reception status. - 0b1: The AMS is required to inform ABS of the E-MBS reception status. The E-MBS AMS in idle mode performs the location update procedure in order to report the start/end of E-MBS reception. In connected and carrier switching mode an E-MBS AMS informs the ABS of the start/end of E-MBS reception and the ABS allocates the unicast resource to the ABS based on the report. | It will be present when the AMS has subscribed the E-MBS service. |

- EMBS reception status report required == 0b0

  - In this case the DSA procedure implicitly indicates the start of E-MBS reception and DSD procedure indicates the end of E-MBS reception.

  - Therefore, in idle mode an AMS subscribing E-MBS service does not need to inform an ABS of the start/end of E-MBS reception explicitly.

  - After DSA procedure an AMS in carrier switching mode switches to the E-MBS carrier

  - After DSA procedure an AMS starts receiving the E-MBS service through E-MBS carrier.

- EMBS reception status report required  == 0b1

  - An AMS subscribing E-MBS gets only E-MBS connection information during DSA procedure.

  - In idle mode when an AMS in idle mode starts or finishes receiving E-MBS service, the AMS informs an ABS of it.

  - When an AMS wants to receive E-MBS, the AMS starts receiving the E-MBS service after the AMS informs an ABS of the start of E-MBS reception.

  - After DSA procedure an AMS in carrier switching mode stays in primary carrier until the AMS informs ABS of the start of E-MBS reception. That is, the AMS switches to the E-MBS carrier after informing ABS of the start of E-MBS reception.

  - During de-registration procedure, an AMS will inform ABS if the AMS currently receives E-MBS data

2] Reporting the status of E-MBS reception in Idle mode in case of subscription based accounting

When an AMS in Idle state starts or finishes receiving the E-MBS service, the AMS needs to notify ABS of it for ABS to select the paging carrier of the AMS properly. Location update procedure will be used for it. If the ABS receives the location update message (AAI_RNG-REQ) which indicates the start of E-MBS reception, the ABS sets the paging carrier of the AMS to the carrier which provides the E-MBS and transmits the AAI_PAG-ADV for the AMS through the E-MBS carrier. If the ABS receives the location update message (AAI_RNG-REQ) which indicates the end of receiving E-MBS, the ABS sets the paging carrier of the AMS to the carrier index calculated based on DID of the AMS and N and transmits the AAI_PAG-ADV for the AMS through the E-MBS carrier.

Table 2. AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | Ranging purpose Indication | 4 | 0b0000 = it indicates that the AMS is currently attempting HO reentry, or, in combination with a Paging Controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If this value is set and Paging Controller ID is absent, it indicates that the AMS is currently attempting HO reentry. If this value is set and Paging Controller ID is present, it indicates that the AMS is attempting network reentry from idle mode to the ABS. …. 0b0111 = it indicates that the AMS is currently experiencing “femto interference” 0b1000 - 0b1111 = reserved 0b1000= it indicates that the AMS is attempting to perform the location update process to notify ABS of the start of receiving E-MBS service. In this case the paging carrier is updated to the carrier which provides E-MBS. 0b1001= it indicates that the AMS is performing the location update process to notify the ABS of the end of receiving E-MBS service. In this case the paging carrier of this AMS is updated to the paging carrier index calculated by the first equation (Paging carrier index = DID modulo N) defined in Section 16.2.8.2.10.2. 0b1010~0b1111: reserved |  |
| … | … | … | … | … |

During De-registration procedure an AMS which subscribes E-MBS service notifies an ABS whether the AMS currently receives the E-MBS service when the AMS is not in carrier switching mode with EMBS reception status report required set to 0b0.

Table 3. AAI_DREG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | E-MBS reception status | 1 | Indicates whether the AMS currently receives E-MBS service or not 0b0: The AMS currently receives E-MBS service 0b1: The AMS does not receives E-MBS service currently | It will be present when the AMS is not in carrier switching mode with EMBS reception status report required set to 0b0. |
| … | … | … | … | … |

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Text Start------------------------------------------------------------------------

[Remedy #1: Modify the Section 16.2.8.2.10.2 as follows, on page 310]

16.2.8.2.10.2 Idle mode

A multicarrier supporting AMS in idle state follows the same procedures defined in 16.2.17.

In a multi-carrier system the PGID_Info message is transmitted in all fully configured carriers. The AAI_PAG-ADV message for an AMS shall be transmitted in only one of the carriers. An idle mode AMS determines the carrier index for monitoring the paging message within AMS's paging-listening interval where its paging message is transmitted using the following equations:

Paging carrier index = DID modulo N (5)

The value of N is the number of carriers per PGID in an ABS that are used for transmitting paging message for idle mode AMS(s). The Paging Carrier Index corresponds to the ascending order of physical carrier index of the carriers which are used for transmitting paging message in the ABS. Paging carrier indication bit is used to specify if a carrier is a paging carrier or not. When the Paging carrier indication =1, then the corresponding carrier is paging carrier. The Paging carrier indication of different carriers is included in the PGID_Info message and may be included in the AAI_NBR-ADV message or the AAI_MC-ADV message.

When an idle mode AMS moves to a new ABS and determines that the SFH change count of the new ABS is same as the information it received through the AAI_NBR-ADV message, then the AMS may use the paging carrier indication received through AAI_NBR-ADV message if this paging carrier indication is included in the AAI_NBR-ADV message.

The AAI_PAG-ADV message is transmitted in one or more of the frames starting from the second sub-frame in the super-frame.

For an E-MBS AMS which subscribes E-MBS service the AAI_PAG-ADV message may be is transmitted in the same carrier as the dedicated carrier for where E-MBS service is provided. In this case, the AMS does not use the equation (5).

An AMS may perform Location Update process to acquire its preferred carrier for the idle mode support when AMS cannot find the paging carrier. If different carriers have different coverage area, then the preferred carrier is used for transmitting paging message. In this case, the AMS does not follow equation (25) to derive paging carrier index.

--------------Proposed Text #1 ---------------------------------------------------------------------------