| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for flow mapping for persistent scheduling (16.2.7) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com |
| Re: | Sponsor Ballot comment on IEEE p802.16m D6 |  |
| Abstract | This contribution proposes the texts related to UL persistent allocation |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for flow mapping for persistent scheduling (16.2.7)

Jeongki Kim, Youngsoo Yuk, and Jin Sam Kwak

- LG Electronics

# Introduction

In 16m Draft [1] persistent scheduling is used for connections with periodic traffic pattern and with relatively fixed size. As mentioned in [2], the current UL Persistent allocation method does not support per-connection allocation.

We propose the simple method to support the per-connection allocation for UL persistent allocation.

# Proposal

During the connection setup procedure for the flow which uses persistent allocation an ABS allocates the 1-bit Persistent allocation (PA) ID for the flow to an AMS in order to uniquely indentify the PA connection among multiple PA flows and the 1-bit PA ID is included in the UL persistent allocation A-MAP IE. When an AMS receives the UL Persistent Allocation A-MAP IE, the AMS can know which flow the UL persistent allocation is for.

- References

[1] IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] C802.16m-10_0098r1, “Intended Flow Information for UL PA Allocations”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested Remedy #1: Add the following rows into Table 786]

Table 786 – Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| … | … | … |
| PA ID | 1 | Persistent allocation Identifier Identify the flow which uses the persistent allocation within an AMS |

[Suggested Remedy #2: Add the following row into Table 742]

Table 742 – AAI_DSA-REQ message

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| ..... | ……….. | .... | ……… | ……….. |
| O | PA ID | 1 | Persistent allocation Identifier Identify the flow which uses the persistent allocation within an AMS | Present when the persistent allocation is used for this flow |

[Suggested Remedy #3: Add the following row into Table 743]

Table 743 – AAI_DSA-RSP message

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| ..... | ……….. | .... | ……… | ……….. |
| O | PA ID | 1 | Persistent allocation Identifier Identify the flow which uses the persistent allocation within an AMS | Present when the persistent allocation is used for this flow |

[Suggested Remedy #4: Modify Table 860 as follows:]

Table 860 - UL Persistent Allocation A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE () { |  |  |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| PA ID | 1 | Persistent allocation ID |
| Allocation Period | 2 | Period of persistent allocation. If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ | - | - |
| … | … | … |
| Reserved | 16 5 |  |
| } else if (Allocation period !=0b00) { |  |  |
| … |  |  |
| Reserved | 1 |  |
| } |  |  |
| } |  |  |

--------------Proposed Texts  ---------------------------------------------------------------------------