| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for mobility information in AAI_RNG-REQ/AAI_DREG-REQ message (16.2.3) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Giwon Park, Yongho Kim, Kiseon Ryu, and Jin Sam Kwak LG Electronics | E-mail: giwon.park@lge.com |
| Re: | Sponsor Ballot comment on IEEE p802.16m D6 |  |
| Abstract | This contribution proposes the texts related to mobility information in the idle mode of the IEEE 802.16m |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for mobility information in AAI_RNG-REQ/AAI_DREG-REQ message (16.2.3)

Giwon park, Yongho Kim, Kiseon Ryu, and Jin Sam Kwak

- LG Electronics

# Introduction

In 16m Draft [1] there is no clear criteria about the fast/medium/slow in mobility information of AAI_RNG-REQ/AAI_DREG-REQ message. We propose the criteria of fast/medium/slow in mobility information.  For example, Preferred BS changing can be used to decide the mobility speed of AMS. That is, AMS decides its mobility speed via the number of the times which it changed preferred BS. Moreover, when the AMS calculates the number of preferred BS changing, it only considers the macro BS changing.

# References

[1] IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

---------------------------------------------------------Start of the Proposed Text ----------------------------------------------

[Change texts in section 16.2.3.1 AAI_RNG-REQ Message as follows:]

16.2.3.1 AAI_RNG-REQ Message

Table 679 - AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Mobility information | 2 | Used to indicate MS's mobility level 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved AMS decides its mobility according to the following condition; If the number of times AMS changes preferred BS is below Low mobility threshold (see Table 716) during the AMS mobility measurement interval (see Table 554), AMS decides its mobility speed to slow. If the number of times AMS changes preferred BS is above Low mobility threshold (see Table 716) and is below High mobility threshold (see Table 716) during the AMS mobility measurement interval (see Table 554), AMS decides its mobility speed to medium. If the number of times AMS changes preferred BS is above High mobility threshold (see Table 716) during the AMS mobility measurement interval (see Table 554), AMS decides its mobility speed to fast. | May be present when De-registration_Request_Code is set to 0x01 |

[Change texts in section 16.2.3.20 AAI_DREG-REQ Message as follows:]

16.2.3.20 AAI_DREG-REQ Message

Table 706 - AAI_DREG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Mobility information | 2 | Used to indicate MS's mobility level 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved AMS decides its mobility according to the following condition; If the number of times AMS changes preferred BS is below Low mobility threshold (see Table 716) during the AMS mobility measurement interval (see Table 554), AMS decides its mobility speed to slow. If the number of times AMS changes preferred BS is above Low mobility threshold (see Table 716) and is below High mobility threshold (see Table 716) during the AMS mobility measurement interval (see Table 554), AMS decides its mobility speed to medium. If the number of times AMS changes preferred BS is above High mobility threshold (see Table 716) during the AMS mobility measurement interval (see Table 554), AMS decides its mobility speed to fast. | May be present when De-registration_Request_Code is set to 0x01 |

[Add texts in section 16.2.3.20 AAI_SCD Message as follows:]

16.2.3.20 AAI_System Configuration Descriptor (SCD) Message

Table 716 - AAI_SCD message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | AMS mobility threshold | Low mobility threshold |  | This threshold is used to decide the mobility speed of AMS. If the number of times AMS changes preferred BS is below this threshold during the predefined time duration, AMS decides its mobility speed to slow. When the AMS calculates the number of preferred BS changing, it only considers the macro BS changing. | N.A. |
|  | AMS mobility threshold | High mobility threshold |  | This threshold is used to decide the mobility speed of AMS. If the number of times AMS changes preferred BS is above this threshold during the predefined time duration, AMS decides its mobility speed to fast. When the AMS calculates the number of preferred BS changing, it only considers the macro BS changing. | N.A. |

[Insert a new parameter into Table 554 in line 43 of page 40 as follows:]

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| AMS | AMS mobility measurement interval | Duration for AMS’s mobility measurement (i.e. count of the number of preferred BS change). AMS reflects the measured value (i.e. the number of preferred BS change) which was measured during this interval to its mobility speed. | 60sec | 600sec | 1800sec |

-----------------------------------------------------------End of the Proposed Text   -----------------------------------------------------