| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Adding the negotiation field for sounding antenna switching (16.2.3.7) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Chiwoo Lim, Taeyoung Kim, Keunchul Hwang, Jaeweon Cho, Hokyu Choi Samsung Electronics | chiwoo.lim@samsung.com |
| Re: | Sponsor ballot comment on P802.16m/D6 Target topic: “IEEE P802.16m/D6, section 16.2.3.7”. |  |
| Abstract | The contribution proposes the Adding the negotiation field for sounding antenna switching of the 802.16m draft amendment, IEEE P802.16m/D6. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Adding the negotiation field for sounding antenna switching 
(16.2.3.7)

Chiwoo Lim, Taeyoung Kim, Keunchul Hwang, Jaeweon Cho, Hokyu Choi 
Samsung Electronics

- Introduction

The contribution proposes to add the negotiation field for sounding antenna switching in the 802.16m amendment. In the current draft amendment [1], there is no negotiation field for sounding antenna switching. However, ABS can command the AMS to switch the physical transmit antenna(s) for sounding transmission for antenna switching capable AMS and multi-antenna AMS in section 16.3.9.2.3.2. Moreover, in case of AMS with 2Rx/1Tx, ABS (with 4Tx) can know only 4*1 channels without sounding antenna switching. Actually, there are 4*2 channels (ABS: 4Tx, AMS: 2Rx). It causes the performance loss due to channel mismatch. With sounding antenna switching, ABS can know 4*2 channels and there is no performance loss. Therefore, we want to add the negotiation field for sounding antenna switching in AAI_REG_REQ message.

- References

[1] IEEE P802.16m/D6, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, May 2010

- Proposed changes for 802.16m draft amendment

--------------------------------------------------- Text Start ---------------------------------------------------

[Remedy-1: Insert the following field in Table 687 on page 98 line 26]

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O |  |  | Capacity for sounding antenna switching support | 1 | 0b0: no support 0b1: support |  |

--------------------------------------------------- Text End ---------------------------------------------------