| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of indexing alphabet in Feedback Channels (16.3.8.3.3.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Chiwoo Lim, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics | chiwoo.lim@samsung.com |
| Re: | Sponsor ballot comment on P802.16m/D6 Target topic: “IEEE P802.16m/D6, section 16.3.8.3.3.2”. |  |
| Abstract | The contribution proposes the clarification of indexing alphabet in Feedback Channels of the 802.16m draft amendment, IEEE P802.16m/D6. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of indexing alphabet in Feedback Channels

(16.3.8.3.3.2)

Chiwoo Lim, Hwasun Yoo, Jaeweon Cho, Hokyu Choi, Heewon Kang
Samsung Electronics

- Introduction

The contribution proposes the clarification of alphabet indexing in Feedback Channels in the 802.16m amendment. In the current draft amendment [1], there are some confusing alphabet indexes. The alphabet k is already used as the HARQ feedback channel index.

In addition, there is no explain of K in LFB-K*floor(LHFB/6). K is the number of HARQ feedback regions in an UL AAI subfame.

- References

[1] IEEE P802.16m/D6, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, May 2010

- Proposed changes for 802.16m draft amendment

--------------------------------------------------- Text Start ---------------------------------------------------

[Remedy-1: Modify the texts on page 683 line 1~14 as following]

In FDD, there is one HARQ feedback region in each UL AAI subframe. In TDD, with notation in Table 753 in 16.2.14.2.2.2.1, for HARQ feedback region in UL AAI subframe n, the associated DL subframes can start from a set of DL AAI subframe indices denoted by M = {m0,m1,...,mK-1mR-1}, where m0 < m1 < ... mK-1mR-1. The number of HARQ feedback regions in an UL AAI subfame is equal to K R according to the number of the associated DL subframes, i.e the size of set M. For DL bursts starting at AAI subframe mkmr, the index of the associated HARQ Feedback region is the order of mkmr in set M, with index 0 corresponding to the 1st HARQ feedback region. Within each HARQ feedback region, the index for HARQ feedback channel is calculated as follows.

[Remedy-2: Modify the text on page 683 line 51~52 as following]

It is allocated after HARQ feedback regions and the total number of the fast feedback channels is LFB-KR*floor(LHFB/6), where R is the number of HARQ feedback regions in an UL AAI subfame.

--------------------------------------------------- Text End ---------------------------------------------------