| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for multicast operation in IEEE 802.16m |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Seho Kim, Sangheon Kim, Hyunkyu Yu, Jung Je Son, Heewon Kang Samsung Electronics Co., Ltd. | seho42.kim@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m |  |
| Abstract | The contribution provides the text for multicast operation in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Multicast operation in IEEE 802.16m

Seho Kim, Sangheon Kim, Hyunkyu Yu, Jung Je Son, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

IEEE 802.16e support both multicast service and the multicast and broadcast services which utilize MBS specific features. Even if E-MBS service is supported in IEEE 802.16m, multicast service is also required to support simple multicasting service in a BS. This contribution provides the description of multicast operation in IEEE 802.16m. The following terms are added and modified to support multicast service in IEEE 802.16m.

- Table 865-Broadcast Assignment A-MAP IE is modified to add multicast assignment information.

- Table 850-Description of the Masking Code for type indicator 001 are modified to add decimal value of Multicast.

- Multicast Identifier is added to describe ID of group that receives multicast burst.

- AAI_DSA-REQ/RSP message are modified to assign Multicast service and Multicast STID.

- AAI_DSD-REQ message is modified to delete Multicast STID.

Removed text

Added text

- Proposed Text Changes

[Remedy #1, Modify the text in Table 865, page 606, section 16.3.6.5.2.4.13 as follows,]

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 865 describes the fields in a broadcast assignment A-MAP IE.

Table 865 – Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers ranging channel allocation information |
| If(Function Index == 0){ |  |  |
| Multicast STID | 12 | ID of a group that receives multicast assignment If all bits of Multicast STID are set to ‘1’s, this A-MAP IE allocates resource for a broadcast burst. Otherwise, this A-MAP IE allocates resource for a multicast burst for a group having this Multicast STID |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 142 | Reserved bits |
| }else{ |  |  |
| Reserved | 16 | Reserved bits |
| } |  |  |
| }else if(Function Index == 1){ |  |  |
| Number of Ranging Opportunities (N) | 2 |  |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| } |  |  |
| Reserved |  | Reserved bits |
| } |  |  |
| } |  |  |

* A 16 bit CRC is generated based on the randomized contents of the Multicast A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 849. If Function index==0b00 or 0b01, the CRC is masked by the 16-bit CRC mask with Decimal value=0. If Function index == 0b10, the CRC is masked by the 16-bit CRC mask with Decimal value=129.

Multicast STID: ID of a group that receives multicast burst. Multicast STID is assigned by AAI_DSA-REQ/RSP message and deleted by AAI_DSD-REQ message.

[Remedy #1, Modify the text in line 45, page 555, section 16.3.6.5.2.4 as follows,]

Table 848 – Assignment A-MAP IE Types

| A-MAP IE Type | Usage | Property |
| --- | --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE | Unicast |
| 0b0001 | UL Basic Assignment A-MAP IE | Unicast |
| 0b0010 | DL Subband Assignment A-MAP IE | Unicast |
| 0b0011 | UL Subband Assignment A-MAP IE | Unicast |
| 0b0100 | Feedback Allocation A-MAP IE | Unicast |
| 0b0101 | UL Sounding Command A-MAP IE | Unicast |
| 0b0110 | CDMA Allocation A-MAP IE | Unicast |
| 0b0111 | DL Persistent Allocation A-MAP IE | Unicast |
| 0b1000 | UL Persistent Allocation A-MAP IE | Unicast |
| 0b1001 | Group Resource Allocation A-MAP IE | Multicast |
| 0b1010 | Feedback Polling A-MAP IE | Unicast |
| 0b1011 | BR-ACK A-MAP IE | Multicast |
| 0b1100 | Broadcast Assignment A-MAP IE | Broadcast/Multicast |
| 0b1101 | Reserved | NA. |
| 0b1110 | Reserved | NA. |
| 0b1111 | Extended Assignment A-MAP IE | NA. |

[Remedy #1, Modify the text in Table 850, page 556, section 16.3.6.5.2.4 as follows,]

Table 850 – Description of the Masking Code for type indicator 001

| Decimal Value | Description |
| --- | --- |
| 0 | Used to mask Broadcast Assignment A-MAP IE for broadcast and ranging channel assignment |
| 1 | Used to mask BR-ACK A-MAP IE |
| 2-128 | Used to mask Group Resource Allocation A-MAP IE(group ID) |
| 129 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment |

[Remedy #2, Insert the following text after line 42, page 48 as follows,]

16.2.1.2.6 Multicast Station Identifier

A 12-bit value that is used to uniquely indentify a multicast flow in the domain of the ABS. All ‘1’ shall not be used.

[Remedy #3, Insert the following text after line 41, page 187, section 16.2.3.46.1 as follows,]

When an ABS commences Multicast service flow, the following parameters shall be included in the AAI_DSA-REQ message.

- Multicast Service: Indicates whether the Multicast service is being requested or provided for the connection that is being setup.

- Multicast STID whose connection is associated the service flow in DSA-REQ.

[Remedy #3, Insert the following text in Table 742, page 197, section 16.2.3.46.1 as follows,]

Table 742 – AAI_DSA-REQ

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Multicast Service | 1 | Indicates whether the Multicast service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. | Present when both ABS/AMS initiates AAI_DSA-REQ |
| O | Multicast STID | 12 | ID of a group to which the flow is added. | Present when Multicast Service is set to 1 |

[Remedy #4, Insert the following text after line 29, page 198, section 16.2.3.46.2 as follows,]

When an AMS commences Multicast service flow, the ABS shall include the following parameters in the AAI_DSA-RSP message.

- Multicast Service: Indicates whether the Multicast service is being requested or provided for the connection that is being setup.

- Multicast STID whose connection is associated the service flow in DSA-RSP.

[Remedy #4, Insert the following text in Table 743, page 199, section 16.2.3.46.2 as follows,]

Table 743 – AAI_DSA-RSP

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Multicast Service | 1 | Indicates whether the Multicast service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. | Present when both ABS/AMS initiates AAI_DSA-REQ |
| O | Multicast STID | 12 | ID of a group to which the flow is added | Present when Multicast Service is set to 1 |

[Remedy #5, Insert the following text after line 10, page 209, section 16.2.3.46.7 as follows,]

When an AAI_DSD-REQ message is sent by an AMS or ABS to delete a multicast service flow, the following parameters may be included in the AAI_DSD-REQ message

- Multicast ID of the connection to be deleted.

[Remedy #5, Insert the following text in Table 748, page 209, section 16.2.3.46.7 as follows,]

Table 748 – AAI_DSD-REG

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Multicast STID | 12 | ID of a group to which the flow is deleted | Present when ABS/AMS initiates AAI_DSD-REQ |