| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE (16.3.6) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Taeyoung Kim, Sangheon Kim Samsung Electronics | ty33.kim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes the texts related to feedback polling A-MAP IE. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the feedback polling A-MAP IE (16.3.6)

Taeyoung Kim, Sangheon Kim

Samsung Electronics Co., Ltd.

# Introduction

This contribution proposes several changes related to Feedback Polling A-MAP IE since some descriptions are still ambiguous and the procedure for MIMO feedback is a little bit complicated. The main concerns are as follows.

- Feedback resource allocation with Feedback Polling A-MAP IE with Polling _sub_type=0b1.

- Uplink HARQ retransmission for MIMO feedback

- Transmission timing of Feedback Polling A-MAP IE when ABS want to change only feedback contents without reallocation of feedback resource and change of feedback period.

# Problems and proposed solutions

1. Feedback resource allocation with Feedback Polling A-MAP IE with Polling_sub_type=0b1

Currently Feedback Polling A-MAP IE with Polling_sub_type=0b1 has no information on the dedicated UL allocation for MIMO feedback message. Instead, several methods are defined in [1]. First way is to use other Assignment A-MAP IE such as UL Basic Assignment A-MAP IE and UL Subband Assignment A-MAP IE to allocate the dedicated UL feedback resource. Second way is to transmit the additional Feedback Polling A-MAP IE with Polling_sub_type=0b0.

However, using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE has some ambiguity in the AMS’s perspective because it’s not clearly defined which resources assigned by other assignment A-MAP IE are associated with MIMO feedback indicated by Feedback Polling A-MAP IE. To define the association between allocated resources and MIMO feedback, it’s necessary to modify many things and newly define some rule. But, in our opinion, it’s not good approach and solution because it might cause the increase of complexity. Therefore, we propose to remove this method using other assignment A-MAP IE such as UL Basic Assignment A-MAP IE and UL Subband Assignment A-MAP IE. Instead, we prefer to use additional Feedback Polling A-MAP IE with Polling_sub_type=0b0 in the same subframe.

2. Uplink HARQ retransmission for MIMO feedback

Generally, HARQ retransmission procedure is that if the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it. In current 16m/D6, however, new HARQ retransmission procedure for MIMO feedback message is defined to prevent the expected performance degradation when long period feedback message is not transmitted. If long period feedback message is not transmitted to ABS, the CQI or PMI mismatch can be occurred even though short period feedback message is transmitted without error.

But, it causes the increase of implementation complexity and the performance degradation can be negligible because of three reasons; 1) the probability of missing the long period feedback message is not so high, 2) using scheduling method, we can prevent it to indicate the a little bit long period for short period message, 3) based on our simulation results which have the worst condition because we assumed that CQI/PMI mismatch are always occurred, the performance loss is not severe. Please see the below results. Therefore, we’d like to suggest to removing the additional HARQ retransmission procedure.

|  | Best 12 |  | Best 6 |  |
| --- | --- | --- | --- | --- |
|  | SE | cell-edge user SE | SE | cell-edge user SE |
| w/o PMI/CQI mismatch | 100% | 100% | 100% | 100% |
| w/ PMI/CQI mismatch | 95.4% | 94.1% | 98.3% | 96.5% |

Note) Urban Micro, 3km/h, 10MHz, 4x2 MIMO (CL-MU, transformed), SLRU only

3. Transmission timing for Feedback Polling A-MAP IE

After receiving Feedback Polling A-MAP IE, the first allocation shall start two frames later, as defined in Table 862 [1]. When ABS want to change only feedback contents without reallocation of feedback resource and change of feedback period by transmitting Feedback Polling A-MAP IE, the transmission timing setting is important because two frame offset might cause the shifting the feedback allocation. Therefore, in this contribution we propose the transmission timing for Feedback Polling A-MAP IE when the ABS transmits a Feedback Polling A-MAP IE to change the only feedback contents without reallocation of feedback resources and changing the period of feedback.

- References

[1] Draft Amendment, P802.16m/D6, May 2010

# Text proposal for inclusion in the 802.16m amendment

<Remedy-1: Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 45 page 593 as follows>

-------------------------------  Start of Text Proposal-1 ---------------------------------------------------

When the Polling_sub_type bit is set to 0b0, a dedicated UL allocation is included in this Feedback Polling A-MAP IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this Feedback Polling A-MAP IE. When the Polling_sub_type bit is set to 0b1, no dedicated UL allocation is included. Instead, the ABS shall transmit an additional Feedback Polling A-MAP IE with Polling_sub_type=0b0 in the same AAI subframe for a dedicated UL allocation and at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall compose the feedback using Feedback Polling A-MAP IE with Polling_sub_type=0b0 or 0b1, and transmit feedback in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE with Polling_sub_type=0b0 in the same AAI subframe with feedback periods designating the same transmission frames..

-------------------------------  End of Text Proposal-1 ---------------------------------------------------

<Remedy-2: Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 39 page 593 as follows>

-------------------------------  Start of Text Proposal-2 ---------------------------------------------------

The information element shown in Table 862 is used by the ABS to schedule MMO MIMO feedback transmission by the AMS. In response to Feedback Polling A-MAP IE, The the AMS sends the MIMO feedback using a MAC control message or a MAC signaling header AAI_SingleBS_MIMO_FBK message, AAI_MultiBS_MIMO_FBK message, MIMO Feedback Header (MFH) or Correlation Matrix Feedback Header (CMFH), depending on the requested feedback content and number of transmit antennas at the ABS.

When the Polling_sub_type bit is set to 0b0, a dedicated UL allocation is included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Polling_sub_type bit is set to 0b1, no dedicated UL allocation is included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall transmit in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE in the same AAI subframe with feedback periods designating the same transmission frames.

When MIMO_feedback_IE_type bit is set to 0b0,Tthe AMS shall send the feedback in a MAC signaling header MIMO Feedback Header when the following feedback information wideband information for any combinations of MIMO feedback modes 0, 4 and 7 with q = 0 is requested.

•wideband information for any combinations of MIMO feedback modes 0, 4 and 7 with q = 0.

The AMS shall send the feedback for all other MFM combinations in a MAC control message AAI_SingleBS_MIMO_FBK message. When MIMO_feedback_IE_type bit is set to 0b1, the AMS shall send the feedback in the AAI_MultiBS_MIMO_FBK message

The coefficients of the quantized transmit correlation matrix shouldshall be feedback fed back in Correlation Matrix Feedback Header (CMFH) whenif no AAI_SingleBS_MIMO_FBK message is sent in the same AAI subframe designated by the Feedback Polling A-MAP IE packet and if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix shouldshall be feedback fed back in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback fed back in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

If the transmit correlation matrix is previously allocated and the AMS is allocated the ABS indicates to transmit the feedback content with q>0, the previous feedback resources previously assigned for the transmit correlation matrix shall be de-allocated with a Feedback Polling A-MAP IE.

-------------------------------  End of Text Proposal-2 ---------------------------------------------------

<Remedy-3: Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 44 page 602 as follows>

-------------------------------  Start of Text Proposal-3 ---------------------------------------------------

Uplink HARQ retransmission for MIMO feedback shall be done as described in 16.2.14.2.1.2. Retransmissions shall obey the following rules:

- Except the retransmission process for the previous HARQ burst contains the long-period feedback, iIf the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it.

- If the retransmission process for the previous HARQ burst reporting long-period feedback is not finished before a new HARQ burst reporting short-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting long-period feedback is carried on, and the new HARQ burst reporting short-period feedback is not transmitted.

-------------------------------  End of Text Proposal-3 ---------------------------------------------------

<Remedy-4: Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 45 page 603 as follows>

-------------------------------  Start of Text Proposal-4 ---------------------------------------------------

For the single-BS MIMO operation, if an AMS is allocated multiple types of MIMO feedback mode by two Feedback Polling A-MAP IEs with Polling_sub_type = 0b0 and 0b1 in the same subframe, the values of short period (p) and long period (q) shall be consistent in the multipletwo IEs. p and q shall only take one same value in two IEs.

If the ABS transmits a Feedback Polling A-MAP IE to change only the feedback contents without reallocation of feedback resources and without changing the period of feedback, the ABS shall transmit the Feedback Polling A-MAP IE in the second AAI frame prior to the AAI frame designated by the previous IE.

-------------------------------  End of Text Proposal-4 ---------------------------------------------------

<Remedy-5: Modify section 16.3.6.5.2.4.11 on line 1 page 597 as follows>

-------------------------------  Start of Text Proposal-4 ---------------------------------------------------

Table 862 – Feedback Polling A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Period | 4 | Resource is allocated at frames designated by every short and long period. The short feedback period is p frames. The long feedback period is q superframes. The first allocation shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The feedback of MIMO feedback modes in MFM_allocation_index is allocated on the short period. The feedback of the transmit correlation matrix is allocated on the long period if q > 0. Short and long period reports shall start at the first allocation. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. … |
| … | … | … |

-------------------------------  End of Text Proposal-4 ---------------------------------------------------

<Remedy-6: Modify section 16.3.6.5.2.4.11 on line 1 page 599 as follows>

-------------------------------  Start of Text Proposal-4 ---------------------------------------------------

Table 862 – Feedback Polling A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Period | 4 | Resource is allocated at frames designated by every short and long period. The short feedback period is p frames. The long feedback period is q superframes. The first allocation shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The feedback of MIMO feedback modes in MFM_allocation_index is allocated on the short period. The feedback of the transmit correlation matrix is allocated on the long period if q > 0. Short and long period reports shall start at the first allocation. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. … |
| … | … | … |

-------------------------------  End of Text Proposal-4 ---------------------------------------------------