| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modification for supporting E-MBS on the dedicated carrier |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Taeyoung Kim, Hyunjeong Kang, Jaejeong(Brian) Shim Samsung Electronics | ty33.kim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes the texts related to supporting E-MBS on the dedicated carrier. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification for supporting E-MBS on the dedicated carrier

Taeyoung Kim, Hyunjeong Kang, Jaejeong(Brian) Shim

Samsung Electronics Co., Ltd.

# Introduction

In current 16m/D6[1], there are two methods to support E-MBS service using a dedicated carrier to E-MBS traffic only and a carrier that is not dedicated to E-MBS traffic. When the dedicated carrier to E-MBS traffic is considered in 16m/D6, the E-MBS operation scenario is not clear and some parameters needed are not included in AAI_SCD message for completing the E-MBS operation. So, this contribution proposes the modification of the current text and AAI_SCD message.

# Proposed solution

In current 16m/D6[1], it said that the AMS learns the E-MBS_Zone_ID(s) to which the serving and neighboring ABS belongs through AAI_E-MBS_CFG message. The AMS needs to get the E-MBS_Zone_IDs those are served in the ABS. By having the information, the AMS can identify whether it can receive certain E-MBS content at the ABS or not. Using acquired E-MBS_Zone_ID(s), the ABS or AMS can initiate the DSA procedure for multicast and broadcast connections. It is assumed that the AAI_E-MBS_CFG message is provided through a carrier where E-MBS is served. In case of the carrier that is not dedicated to E-MBS traffic, the AAI_E-MBS_CFG message shall be transmitted through the corresponding carrier and the AMS can acquire the E-MBS_Zone_ID(s) at the carrier.

However, in case of the carrier that is dedicated to E-MBS service where multicarrier feature is supported by the AMS and the ABS, this procedure using AAI_E-MBS_CFG message to obtain E-MBS_Zone_ID(s) is not working. In case that the AMS is connected to the primary carrier and E-MBS service is provided through the dedicated carrier, the AAI_E-MBS_CFG message shall not be transmitted on the primary carrier, so the AMS cannot acquire E-MBS_Zone_ID(s) to initiate the DSA procedure before carrier switching.

To resolve this problem, we propose to add the E-MBS_Zone_ID(s) information into an AAI_SCD message. For this case, an AAI_MC-ADV message also includes E-MBS related parameters (e.g., E-MBS_Zone_ID(s)) contained in AAI_SCD message of other carriers. Therefore, if the E-MBS traffic is served through other carrier, the AMS decodes the AAI_MC-ADV message before initiating the DSA procedure and can obtain the E-MBS_Zone_ID(s) of other carrier.

- References

[1] Draft Amendment, P802.16m/D6, May. 2010

# Text proposal for inclusion in the 802.16m amendment

<Remedy-1: Modify section 16.2.3.12 on line 39 page 117 as follows>

-------------------------------  Start of Text Proposal-1 ---------------------------------------------------

Delta information is encoded as a sequence of TLVs. The encoding of the one-byte type field for S-SFH elements can be found in Table 693—. Parameters from Type 60 to Type 9197 shall only be included in AAI_MC-ADV. The length field should equal the element length as specified in the S-SFH, rounded up to an integer number of bytes.

The delta information field should be padded so its length will be an integer number of bytes

Table 693 – Type values for delta SFH encodiidng in AAI_NBR-ADV and AAI_MC-ADV

| Type | Name |
| --- | --- |
| 1 | LSBs of 48 bit ABS MAC ID |
| 2 | Number of UL ACK/NACK channels per HARQ feedback region |
| … |  |
| 91 | 16m_Relay_zone_AMS_allocation_indicator |
| 92 | Zone_Allocation-Bit-MAP |
| 93 | ZF |
| 94 | MSI Length |
| 95 | E-MBS AAI frame offset |
| 96 | Num of E-MBS Zone ID |
| 97 | E-MBS_Zone_ID |

-------------------------------  End of Text Proposal-1 ---------------------------------------------------

<Remedy-2: Modify section 16.2.3.30 on line 42 page 160 as follows>

-------------------------------  Start of Text Proposal-2 ---------------------------------------------------

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| O | E-MBS Configuration Parameters | Zone_Allocation-Bit-MAP | DSACMAX -1 | See section 16.9.3.1 | Present when E-MBS is configured |
|  | E-MBS Configuration Parameters | ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1 |  |
|  | E-MBS Configuration Parameters | MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40ms (NMSI=2) 0b01: 4 superframes, 80ms (NMSI=4) 0b10: 8 superframes, 160ms (NMSI=8) 0b11: 16 superframes, 320ms (NMSI=16) |  |
|  | E-MBS Configuration Parameters | E-MBS AAI frame offset | variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3 bits MSI length == 0b01: 4 bits MSI length == 0b10: 5 bits MSI length == 0b11: 6 bits |  |
|  | E-MBS Configuration Parameters | Num of E-MBS Zone ID | 3 | Number of E-MBS zone ID to add |  |
|  | E-MBS Configuration Parameters | fullNumE-MBSIdArr[1…8] |  |  |  |
|  | E-MBS Configuration Parameters | A) E-MBS_Zone_ID | 7 | Indicates an E-MBS zone ID |  |

-------------------------------  End of Text Proposal-2 ---------------------------------------------------

<Remedy-3: Modify section 16.9.2.1 on line 23 page 847 as follows>

-------------------------------  Start of Text Proposal-3 ---------------------------------------------------

The AMS learns the E-MBS_Zone_ID(s) to which the serving and neighboring ABS belongs through AAI_E-MBS_CFG or AAI_SCD or AAI_MC-ADV message.

-------------------------------  End of Text Proposal-3 ---------------------------------------------------

<Remedy-4: Modify section 16.9.3.1 on line 12 page 849 as follows>

-------------------------------  Start of Text Proposal-4 ---------------------------------------------------

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and E-MBS ID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI_E-MBS-CFG shall be advertised in the superframe before the beginning of the largest MSI specified in an AAI_SCD, on the carrier E-MBS is served (16 superframes).

-------------------------------  End of Text Proposal-4 ---------------------------------------------------