| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Uplink Power Control Clarification (16.3.9.4.7) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Jeongho Park, Hokyu Choi Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D6 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Uplink Power Control Clarification (16.3.9.4.7)

Jeongho Park, Hokyu Choi

Samsung Electronics

- References

[1] IEEE P802.16m/D6

- Introduction

This contribution includes several text proposal or modification for clarification.

- Scope of Suggested Remedies

A. PTN Clarification

In page 162 [1], there is a parameter named PTN which represents the thermal noise power density in 0oC.

It would be better to keep exact the equation and value for it.

Since thermal noise is the function of temperature (Kelvin) and Bolzmann constant, we can easily obtain the value in 0oC like the equation below.

PTN = Kb * T

Where

- T is Kelvin temparature which is 0 (oC) + 273.15 [K]

- Kb is Bolzmann constant which is 1.3806504 e-23 [JK-1]

Therefore

PTN = 1.38e-23 * 273.15

and

PTN [dBm] = 10*log10(PTN * 1000) = -174.2352 [dBm]

B. SINRTarget Clarification in PSR (power status report)

One of conditions for MS’s PSR is event triggering based on the difference between the last value of M and current value of M where M is defined in page 746 [1] as

M = L + SINRTarget (Reported)

However this SINRTarget could be different according to the transmission channel. It is necessary to define which transmission channel is referred to this parameter.

Based on the contents and theory of PSR, this SINR is for data transmission which includes the parameter of SIRDL.

C. Clarification of OffsetControl and OffsetData in AAI_HO-CMD message

In page 748 [1], there is a simple sentence about usage of power offset values included in AAI_HO-CMD message like below.

“if CDMA_RNG_FLAG in message AAI_HO-CMD is set to 0, CDMA based ranging will be skipped, the power offset values for target ABS will be included the AAI_HO-CMD, as shown in Table 690”

i) Table number is not correct. It should be 691.

ii) There is no description about what power offset values are used for. Their main purpose is to compensate  difference of uplink NI level between serving and target ABS.

- Proposed Text

Remedy #1: page 162, Modify line 59~63 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

PTN  is the thermal noise power density in 0oC which has value of -174.2352 dBm.

f  is the subcarrier spacing (Hz).

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #2: page 746, Modify line 61~65 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

L is the pathloss defined in Equation (291)

SINRTarget is defined in Equation (292) with beta=0 and SINRTarget(Reported) is the value measured at the moment of ‘Reported’

nlast is the frame index when the last AMS status reporting message was sent

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #3: page 748, Modify line 12~26 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

16.3.9.4.9 Uplink Power Control in Handover

During the handover processing, if CDMA_RNG_FLAG in message AAI_HO-CMD is set to 0, CDMA based ranging will be skipped, the power offset values for target ABS will be included the AAI_HO-CMD, as shown in Table 690691. Other uplink power control parameters for target ABS are set by using the serving ABS value.

If CDMA_RNG_FLAG in message AAI_HO-CMD is set to 1, CDMA based ranging shall be performed, the power control parameters of target ABS will be got as the process defined in 16.3.9.4.8.

-------------------------------------------------Text Change END -----------------------------------------------------