| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for the number of PC-A-MAP IE in case of D:U = 1:3 (16.3.6.5.1.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Jeongho Park, Ji-Yun Seol, Hokyu Choi Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D6 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for the number of PC-A-MAP IE in case of D:U = 1:3 (16.3.6.5.1.2)

Jeongho Park, Ji-Yun Seol, Hokyu Choi

Samsung Electronics

- References

[1] IEEE 802.16m/D6

- Introduction and Problem Definition

This contribution provides a proposal to limit the number of total PC-A-MAP IE in case of D:U = 1:3.

In 16e/16m mixed modes, with high probability, the number of AMS would be smaller than 16m only modes. This is because some of terminal would be supported by 16e modes. Neverthless, AMS needs to prepare all cases about PC-A-MAP IE sizes even like NPC-A-MAP has ceil(44 * U/D). According to [1], total number of PC-A-MAP in case of D:U = 1:3 would be 132. Since the mixed case is expected to have smaller AMS, such amount of PC-A-MAP will not be necessary.

In this contribution, therefore, we propose to limit the number of PC-A-MAP IE in D:U=1:3 case to smaller number.

- The Scope of Suggested Remedies

1. UL_FEEDBACK_SIZE

Since PC-A-MAP IE size is relevant to the Fast FBCH in uplink, it is necessary to limit the resource size for UL control channel itself which is UL_FEEDBACK_SIZE. This is included in S-SFH SP3 IE (page 548, Table 842) which has 4 bit size representing 1 ~ 16 LRUs.

Frame structure D:U=1:3 (Frame configuration index 20, 21 for 5/10/20MHz BW) has 50% resource portion out of whole resource. Therefore, it would be natural to limit resources for UL control by 50%, which is 1~8 LRUs.

2. Power control channel resource size indicator

This is the field name with 2bit included in S-SFH SP1 IE, which represents the total number of PC-A-MAP IE per subframe. According to the field value, total 4 cases are defined for PC-A-MAP IE size.

0b00 : 0 (No use of PC-A-MAP)

0b01 : ceil (14 * U/D)

0b10 : ceil (28 * U/D)

0b11 : ceil (44 * U/D)

If we limit the maximum size of UL control channel up to 8 LRUs in case of D:U = 1:3, maximum number of F-FBCH per subframe would be 23 which could be shown in Table 1.

(Table 1)

| # of LRU for FBCH | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of FBCH | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
|  | Number of channels for FFBCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # of H-FBCH in 5MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 0b01 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b10 | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 0b11 | -1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| # of H-FBCH in 10MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 0b01 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b10 | -1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 0b11 | -2 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| # of H-FBCH in 20MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0b00 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 0b01 | -1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 0b10 | -5 | -2 | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 0b11 | -7 | -4 | -1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |

If maximum number of F-FBCH per subframe is 23, it means it could be covered by 0b10 of “Power control channel resource size indicator” because ceil (28 * U/D ) is enough. We can rule out the usage of 0b11 there.

Therefore it would be better to include description text for this limitation.

- Proposed Text

Remedy #1:page 548, Modify line 5~8 in table 842 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

Table 842 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL_FEEDBACK_SIZE | 4 | Number of distributed LRUs for UL feedback channels per a UL AAI sub-frame (Refer to 16.3.8.3.3.2) ranging from 1 LRU (0b0000) to 16 LRUs (0b1111). In case of frame configuration index 20 and 21 for 5/10/20MHz BW with 1/8 CP, MSB is not used. In this case the range of size is from 1 LRU(0b0000) to 8 LRUs (0b0111). |

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #2:page 542, Modify line 23~29 in table 840 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

Table 840 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Power control channel resource size indicator. | 2 | Total number of PC-A-MAP IE, NPC-A-MAP 0b00 : 0 (No use of PC-A-MAP) 0b01 : ceil(14 * U/D) 0b10 : ceil(28 * U/D) 0b11 : ceil(44 * U/D) Here U and D are the number of available subframe number per a frame in uplink and downlink respectively. In case of frame configuration index 20 and 21 for 5/10/20MHz BW with 1/8 CP, 0b11 is not used.. |

-------------------------------------------------Text Change END -----------------------------------------------------