| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PGID in AAI_MC-ADV and AAI_NBR-ADV (16.2.3.12) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Hyunjeong Kang, Jaehyuk Jang, Jungje Son Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | SB_16m on P802.16m/D6 |  |
| Abstract | The contribution proposes to specify PGID for carriers in AAI_MC-ADV and AAI_NBR-ADV messages. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

PGID in AAI_MC-ADV and AAI_NBR-ADV

Hyunjeong Kang, Jaehyuk Jang, Jungje Son

Samsung Electronics

- Introduction

When an idle mode AMS in a multi-carrier system determines a paging carrier, it uses the equation (5) Paging carrier index = DID modulo N. The N is the number of carriers per paging group in an ABS that are used as paging carrier. So, the AMS shall know the paging group for each carrier in an ABS and whether the carrier is a paging carrier or not to find the value of N.

According to current 16m/D6, the paging carrier indication of each carrier is given through a PGID_Info, an AAI_MC-ADV or an AAI_NBR-ADV message. The paging group identifier for carrier(s) is provided through the PGID_Info message. However the paging group identifier for carrier(s) is not specified in an AAI_MC-ADV message and an AAI_NBR-ADV message.

Therefore this contribution proposes to provide paging group identifier for carriers in a serving ABS and carriers in neighboring ABS(s) through an AAI_MC-ADV message and an AAI_NBR-ADV message, respectively.

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Add a new field for PGID after ‘Physical carrier index’ on line 64 of page 115 as follows:]

| M | PGID | 16 | Indicate Paging group identifier where the carrier belongs | Shall be included for each carrier of each neighbor BS |
| --- | --- | --- | --- | --- |

[Remedy 2: Add a new field for PGID before ‘Paging carrier indicator’ on line 53 of page 220 as follows:]

| O | PGID | 16 | Indicate Paging group identifier where the carrier belongs | Shall be present when Serving BS carrier number is more than 1 |
| --- | --- | --- | --- | --- |

-------------------------------  Text End  ---------------------------------------------------