| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Broadcast A-A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Hyunkyu Yu, Ji-Yun Seol, Hyunjeong Kang, Jaeweon Cho, Rakesh Taori Samsung Electronics Co., Ltd. Youngsoo Yuk, Jinsoo Choi, Jin Sam Kwak LG Electronics Ming-Hung Tao, Ying-Chuan Hsiao ITRI | hk.yu@samsung.com youngsoo.yuk@lge.com mhtao@itri.org.tw |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution proposes to delete time domain repetition in the broadcast assignment A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Broadcast A-A-MAP IE (Section 16.3.6.5)

Hyunkyu Yu, Ji-Yun Seol, Hyunjeong Kang, Jaeweon Cho, Rakesh Taori

Samsung Electronics

Youngsoo Yuk, Jinsoo Choi, Jin Sam Kwak

LG Electronics

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

In order to support the case where the available resources in a subframe are not enough for transmitting a long broadcast MAC control message, time domain repetition was proposed and adopted in point of PHY transmission structure. Because, in general, MAC control messages can be fragmented, we have now two options in D6: message fragmentation and time domain repetition.

We should only allow more than one option if the schemes bring particularly great benefits in some situations. Especially for something as basic as the reception of broadcast message, which every single terminal has to implement, it is beneficial to avoid options.

Message fragmentation has to be supported anyways irrespective of time domain repetition because any control message can be fragmented. Time domain repetition, however, is highly expected to give a large implementation burden rather than any significant benefits.

The followings are the reasons why we recommend removing time domain repetition.

- Implementation complexity in AMS side

- Time domain repetition requires larger buffer size or increases the complexity in buffer management

- Scheduling complexity in ABS side

- Some messages (e.g. AAI_TRF-IND, AAI_PAG-ADV and AAI_NBR-ADV) whose transmission method and transmission time are specified according to its usage are not appropriate to be transmitted using time domain repetition. Thus, an ABS scheduler firstly needs to decide whether a message can be transmitted repetitively in time domain or not whenever the scheduler generates a broadcast burst. To find out that the message is appropriate to be included in a broadcast burst using time domain repetition, the ABS scheduler communicates with an ABS processor which handles MAC layer operation and it may cause delay on the message processing.

- Because time-repeated burst should be transmitted with the periodicity of one frame, the scheduling priority for the broadcast burst needs to be very high. In downlink, if many resources are reserved for periodic transmission, e.g. PA, and GRA, additional resource reservation for broadcast burst may increase scheduling complexity.

- Restriction on resource usage

- Support of two methods (fragmentation, time domain repetition) can increase scheduling flexibility because we can choose between two things. Time domain repetition, however, requires a certain number of repetitions, each of which should have the same amount of resources. From this aspect, we may say that time domain repetition, on the contrary, decreases the flexibility on resource utilization.

- Small gain in point of signaling overhead

- Time domain repetition is motivated from the case when the message size is large. However, reduction in signaling overhead (2-bytes MAC header) yields very small benefit, especially in case of long message size.

- Proposed Text Changes

[Remedy-1: Change the following text, at line 20 in page 607, section 16.3.6.5.2.4.13, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 865 describes the fields in a broadcast assignment A-MAP IE.

Table 865 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers ranging channel allocation information |
| If (Function Index == 0){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries of Table 956 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD If number of Dl AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator=0b1 |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 14 | Reserved bits |
| } else{ |  |  |
| Reserved | 167 | Reserved bits |
| } |  |  |
| } else if (Function Index == 1){ |  |  |
| Number of Ranging Opportunities (N) | 2 |  |
| For (i=0; i<N; i++){ |  |  |
| Subframe index | 3 |  |
| } |  |  |
| Reserved | TBD | Reserved bits |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 849.

The broadcast burst signaled by the Broadcast Assignment A-MAP IE is always transmitted using SFBC as the MIMO encoder format and QPSK as the modulation scheme.

The periodicity of the time domain repetition is one frame. During the period when a broadcast burst is transmitted repetitively in time, other broadcast bursts can only be transmitted without time domain repetition. For each broadcast burst transmission with time domain repetition, the ABS shall transmit a broadcast A-A-MAP IE with decreasing the value of the remaining repetition number by 1 per each transmission. That is, if the ABS performs the first transmission at the k-th frame with signaling the remaining repetition number as N-1, the transmission shall be completed at the (k+N-1)-th frame. The value of Repetition field shall be decreased by 1 per each transmission. For other fields, only the resource index can be changed during the period when the time domain repetition is performed. SPID is always 0 for broadcast burst transmission.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Change the following text, at line 50 in page 432, section 16.3.3.1, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

ABS shall not allocate more than 1 time repeated broadcast data burst per frame and 1 non-time repeated broadcast data burst per AAI subframe (using Broadcast Assignment A-Map IE) and …

-------------------------------------------------End proposed text----------------------------------------------------------------