| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for CDMA Allocation A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Hyunkyu Yu, Youngkyo Baek, Jaeweon Cho Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies HARQ signaling and MIMO scheme for the burst indicated through CDMA Allocation A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for CDMA Allocation A-MAP IE (Section 16.3.6.5)

Hyunkyu Yu, Youngkyo Baek, Jaeweon Cho

Samsung Electronics

- Introduction

(1) Proposals on transmission scheme

- In D6, there’s no description about which MIMO scheme is applied for the burst signaled via CDMA allocation A-MAP IE. Considering anonymous transmission, we recommend SFBC in DL and vertical encoding with Mt=1 in UL.

- HARQ process needs to be specified separately for DL and UL.

(2) Fragmentation of AAI_RNG-RSP

- Under the situation that neither STID nor TSTID is assigned to the AMS, the CDMA allocation A-MAP IE grants the bandwidth to send AAI_RNG-RSP message. In D6, HARQ operation is applied to the AAI_RNG-RSP message. If the AAI_RNG-RSP message is allowed to be fragmented, then AI_SN field should be included in the case for DL allocation. However, such complicate HARQ operation may give a big burden to AMS implementation because this communication is performed based on just RAID.

- Proposed Text Changes

[Change the following text, at line 46 in page 584, section 16.3.6.5.2.4.7, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.2.4.7 CDMA Allocation A-MAP IE

…

Table 858 specifies the fields of CDMA Allocation A-MAP IE used by ABS for UL allocation of bandwidth allocation for bandwidth request or ranging procedures.

Through CDMA Allocation A-MAP IE, ABS allocates UL resources to a user that requested bandwidth using a random ranging preamble code or BR preamble code. Additionally, in contention-based random access ranging procedure, ABS transmits AAI_RNG-RSP message through a DL burst assigned by CDMA Allocation A-MAP IE to an AMS that has no STID or TSTID assigned. That is, ABS grants UL bandwidth for the transmission of BR with STID header in bandwidth request procedure, and for the transmission of AAI_RNG-REQ message or AAI_MSG-ACK in ranging procedure. Other messages and standalone headers shall not be signaled using CDMA Allocation A-MAP IE in UL allocation.

The maximum number of the UL HARQ retransmission is set to the default value defined in 16.2.14.2.2. Non-adaptive HARQ retransmission is used in UL allocation, and thus HARQ retransmission control information cannot be changed during retransmission process. the transmission of CDMA Allocation A-MAP IE is permitted only for the signaling of initial HARQ subpacket.

The UL HARQ burst signaled by the CDMA Allocation A-MAP IE is always transmitted using vertical encoding with Mt=1 as the MIMO encoder format and QPSK as the modulation scheme.

In addition to UL allocation, ABS transmits AAI_RNG-RSP message through a DL burst assigned by CDMA Allocation A-MAP IE to an AMS in contention-based random access ranging procedure, where neither STID nor TSTID is assigned to the AMS yet.

The maximum number of the DL HARQ retransmission is set to the default value defined in 16.2.14.2.1.

The DL HARQ burst signaled by the CDMA Allocation A-MAP IE is always transmitted using SFBC as the MIMO encoder format and QPSK as the modulation scheme.

-------------------------------------------------End proposed text----------------------------------------------------------------