| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications for Update of S-SFH (16.2.24) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Hyunkyu Yu, Hyunjeong Kang, and Ji-Yun Seol Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution provides the clean up text for update of S-SFH. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for Update of S-SFH (16.2.24)

Hyunkyu Yu, Hyunjeong Kang, and Ji-Yun Seol

Samsung Electronics

- Introduction

This contribution provides the clarification on the update of S-SFH.

(1) S-SFH applying offset

- Current D6, it is not clear how S-SFH applying offset is processed.

- If S-SFH applying offset indicates which S-SFH change count should be considered in the current superframe, there exists ambiguity in FDD uplink. That is, at the first frame in uplink where applying offset is changed from 1 to 0, it is unclear which system information should be applied (previous one or current one?).

- We recommend S-SFH applying offset indicating the behavior in the next frame. S-SFH applying offset indicates AMSs which S-SFH change count needs to be considered for applying the S-SFH contents in the next superframe.

(2) Example figures

- We suggest including figures which depict the overall process of S-SFH update.

Removed text

Added text

- Proposed Text Changes

[Remedy #1, Modify the text in line 45, page 421, section 16.2.24 as follows,]

------------------------------------------------Start proposed text----------------------------------------------------------------

The S-SFH subpacket (SP) IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP3 IE) are transmitted by ABS at the scheduled period superframe of each SP, respectively, with different periodicity (i.e., TSP1 < TSP2 < TSP3). This SP scheduling periodicity information is specified in Table 836 and is transmitted in the S-SFH SP3 IE. Otherwise, an AMS may implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE. According to scheduling periodicity, superframe number where SPx is transmitted is also defined in 16.3.6.2.1.2.

Every superframe, the ABS transmits P-SFH IE containing the S-SFH scheduling information, S-SFH change count, S-SFH SP change bitmap, and S-SFH applying offset.

The S-SFH change count shall remain unchanged as long as all of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH SP IEs remain unchanged. The S-SFH change count can be changed only at the specific superframe number (SFN) that satisfies SFN modulo S-SFH change cycle = 0, where the S-SFH change cycle is defined as following formula,

S-SFH change cycle = Rate of change of S-SFH Info  the periodicity of S-SFH SP1 IE / 20ms

and the "Rate of change of S-SFH Info" and "the periodicity of S-SFH SP1 IE" are indicated in S-SFH SP3 IE.

The S-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH IEs change. The changed S-SFH SP IE(s) shall be transmitted at the scheduled superframes of period corresponding to each S-SFH SP IE.

If the AMS determines that the "S-SFH change count" field in P-SFH has not changed, then the AMS determines that it has up to date information.

The ABS shall indicate any changes of S-SFH IE at the specific super-frame number (SFN) that satisfies SFN modulo S-SFH change cycle = 0.

S-SFH change cycle = Rate of change of S-SFH Info  the periodicity of S-SFH SP1 IE / 20ms

Where the "Rate of change of S-SFH Info" and "the periodicity of S-SFH SP1 IE" are indicated in S-SFH SP3 IE.

Each bit of the S-SFH SP change bitmap in P-SFH IE indicates the changing status of the corresponding S-SFH SP IE in association with the S-SFH change count. The bit # 0 (LSB), bit #1 and bit #2 (MSB) are mapped to S-SFH SP1 IE, S-SFH SP 2 IE and S-SFH SP3 IE, respectively. If any of the values (except MSB of superframe number in S-SFH SP1 IE) of in an S-SFH SP IE are changed, the bit corresponding to the changed S-SFH SP IE is set to 1. The value of the S-SFH SP change bitmap can be changed only when the S-SFH change count is changed.

The S-SFH SP IE(s) associated with the incremented S-SFH change count shall be transmitted at their scheduled superframes, starting from the superframe where S-SFH change count is changed.

Once the contents in S-SFH SP IEs are changed, all of the values (except MSB of superframe number in S-SFH SP1 IE) in S-SFH SP IEs shall remain unchanged during one or multiples of S-SFH change cycle, until the S-SFH change count in P-SFH IE is changed again.

The changed S-SFH SPx IE (SP1, SP2 and/or SP3) is transmitted at its corresponding scheduled superframes, respectively during the S-SFH change cycle. No other S-SFH SP IE change shall be transmitted during the S-SFH change cycle.

The S-SFH applying offset in the P-SFH IE is used to indicate which S-SFH change count shall be considered for applying the system parameters in the S-SFH SPx IEs. According to the S-SFH applying offset, if the S-SFH applying offset is set to 0, the AMS shall use the system parameters of S-SFH SPx IEs associated with the current S-SFH change count. Otherwise, if the S-SFH applying offset is 1, the AMS shall store the changed parameters of S-SFH SP IEs associated with the current S-SFH change count, but use the parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16).

The ABS shall set the S-SFH applying offset to 1 for a number of superframes after the S-SFH change count is incremented by 1.

To guarantee enough time for AMSs to receive the changed S-SFH SPx IE(s), the changed contents are applied from the specific SFN according to which S-SFH SPx IE is changed, as below.

S-SFH applying offset shall be set to 1 until the changed S-SFH SP1 IE and the changed S-SFH SP2 IE are transmitted 2 times at their regularly scheduled transmission times and the changed S-SFH SP3 IE is transmitted once, i.e, S-SFH applying offset shall be set to 0 in the next superframe after the changed S-SFH SP1 IE and the changed S-SFH SP2 IE are transmitted 2 times and the changed S-SFH SP3 IE is transmitted once.

- If the S-SFH SP1 IE is changed, the changed contents is applied in the next superframe after the changed S-SFH SP1 IE is transmitted 2 times at its regularly scheduled times.

- If the S-SFH SP2 IE is changed, the changed contents is applied in the next superframe after the changed S-SFH SP2 IE is transmitted 2 times at its regularly scheduled times.

- If the S-SFH SP3 IE is changed, the changed contents is applied in the next superframe after the changed S-SFH SP3 IE is transmitted once at its regularly scheduled times.

If more than one S-SFH SPx IEs are changed, their changed contents shall be applied at once from the last superframe among the superframes indicated in the corresponding cases above.

The S-SFH applying offset in the P-SFH IE is used to explicitly indicate which S-SFH change count shall be considered for applying the contents of S-SFH SPx IEs in the next superframe. If the S-SFH applying offset is set to 0, in the next superframe, the AMS shall use the S-SFH contents associated with the currently transmitted S-SFH change count. Otherwise, if the S-SFH applying offset is 1, in the next superframe, the AMS shall use the S-SFH contents associated with previously transmitted S-SFH change count (=(the current S-SFH change count -1) modulo 16).

For all S-SFH SPx IEs indicated in the S-SFH change bitmap, their changed contents shall be applied at the same superframe. When more than one S-SFH SPx IEs change at the same time, the new information of these S-SFH SPx IEs are applied at the same time.

According to Based on the S-SFH change count, S-SFH SP change bitmap, S-SFH change cycle, and S-SFH applying offset, the AMS knows if it is needed to decode S-SFH SP IEs in the current superframe to update the system parameters broadcasted within the S-SFH SP IEs, and the AMS may decide to disregard the S-SFH SP IEs with the presumption of there being no change in S-SFH SP IEs compared to that already stored in the AMS.

The AMS shall compare the values of each S-SFH change count in the last received current P-SFH IE and with that stored last, the last stored P-SFH IE whenever it receives P-SFH IE. whenever the validity check of the last stored S-SFH SP IEs is required.

a) If there is no difference of two S-SFH change counts, the AMS may not decode S-SFH IE in the superframe during a certain period where S-SFH change count remains unchanged.

b) Else if the difference of two S-SFH change counts is one, the AMS shall update the S-SFH SP IE(s) whose bit in the S-SFH SP change bitmap is set to 1.

c) Else if the difference of two S-SFH change counts is greater than one, the AMS shall update all S-SFH SP IEs.

In case of b) and c), the AMS shall save the S-SFH change count., and shall receive the S-SFH SP IE(s) at their scheduled superframe.

If the S-SFH SP IE indicated by S-SFH scheduling information corresponds to the changed S-SFH SP IE as indicated in the S-SFH change bitmap, the AMS shall decode the following S-SFH SP IE and store the parameters in association with the current S-SFH change count. The AMS shall store and update all the changed S-SFH SP IEs associated with the increment of S-SFH change count at the scheduled superframe for each S-SFH SP IE.

Figure xxx shows the examples of S-SFH update procedure. This figure considers the following assumptions:

- The periodicity of SP1 IE, SP2 IE and SP3 IE are 40ms, 80 ms and 160 ms, respectively

- Rate of change of S-SFH info is 16, i.e., S-SFH change cycle is 32.

- ‘No’ means that no SPx IE is scheduled.

(a) SP1 IE is changed

(b) SP1 IE and SP3 IE are changed

(c) SP3 IE is changed

Figure xxx. Illustrations of S-SFH update procedure

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy #2, Modify the text in line 28, page 539, section 16.3.6.5.1.1 as follows,]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 838 – P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| … | … | … |
| S-SFH applying offset | 1 | Indicate the associated S-SFH change count in this the next superframe 0b0: Use S-SFH SPx IEs associated with the current S-SFH change count 0b1: Use S-SFH SPx IEs associated with (the current S-SFH change count - 1) modulo 16 |
| … | … | … |

…

S-SFH applying offset

A one bit flag that indicates AMSs which S-SFH change count shall be considered for applying the S-SFH contents in the next superframe the system parameters in the S-SFH SPx IEs. If the S-SFH applying offset is set to '0', the AMS shall use the S-SFH contents the system parameters of S-SFH SPx IEs associated with the current S-SFH change count in the next superframe. Otherwise, if the S-SFH applying offset is '1', the AMS shall use the S-SFH contents the parameters associated with previous S-SFH change count (= (the current S-SFH change count -1) modulo 16) in the next superframe. If the S-SFH contents associated with previous S-SFH change count is are not available, the AMS shall acquire the relevant S-SFH contents by decoding the S-SFHs carried in their corresponding scheduled superframes.

-------------------------------------------------End proposed text----------------------------------------------------------------