| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification to AAI_DSx-REQ (Section 16.2.3.47) |  |
| Date Submitted | 2010-09-15 |  |
| Source(s) | Hyunkyu Yu, Seho Kim, Sangheon Kim, Hyunjeong Kang Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution proposes the modification to AAI_DSx-REQ. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification to AAI_DSx-REQ (Section 16.2.3.47)

Hyunkyu Yu, Seho Kim, Sangheon Kim, Hyunjeong Kang

Samsung Electronics

- Introduction

- HARQ channel mapping:

In uplink transmission, bandwidth allocation for the delay-sensitive traffic (e.g. VoIP) needs to be explicitly mapped to a flow. Otherwise, other service flows may use those resources, degrading the performance of that flow (VoIP). To guarantee the performance, we suggest mapping HARQ channels to a flow in DSx message. Note that persistent allocation cannot be always used for VoIP transmission, so we recommend adopting HARQ channel mapping concept, which is more general than just mapping persistent allocation to a flow.

- Padding PDU indicator:

Generally, if an ABS assigns the resource to AMS though an AMS has no data burst to be transmitted, the AMS is supposed to transmit the special data burst called padding PDU which consists of the first byte of 0xF0 and the remaining bytes of 0x00 according to padding rule of section 16.2.4.7. However, in terms of power saving problem in an AMS, the option that an AMS does not transmit the padding PDU can be considered. Therefore, this contribution proposes to add the parameter indicating whether or not an AMS transmits the padding PDU.

- Proposed Text Changes

[Remedy-1: Add the following text, at line 23 in page 377, section 16.2.12.8, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.12.8 Service Flow/Convergence sublayer parameters

…

| … | … | … |
| --- | --- | --- |
| Authorization Token | variable | Authorization token that is used for authorizing the QoS for one or service flows generated by AMS-initiated higher level service flow creation or modification procedures. Refer to 11.13.32 |
| Number of HARQ Channels | 5 | Indicates the number of HARQ channels which are assigned to the flow. |
| fullNumHarqChArr[0…31] |  |  |
| A) HARQ Channel Mapping | 5 | Indicates the index (ACID) of each HARQ channel. |
| ROHC Parameter Payload | variable | Refer to Table 784 |
| … | … | … |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Add the following text, at line 6 in page 382, after section 16.2.12.8.6, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.12.8.7 HARQ channel mapping

This parameter specifies the set of HARQ channels that shall be used for carrying data on this service flow. The absence of this parameter in any of AAI_DSA-REQ or AAI_DSC-REQ relevant for the flow means that this flow may use all HARQ channels. If the set of HARQ channels are mapped to the specific service flow, this set of HARQ channels shall be assigned to that flow for data transmission with higher priority.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-3: Add the following text, at line 29 in page 213, section 16.2.3.47.1, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.3.47.1 AAI-DSA-REQ

…

Table 737 – AAI-DSA-REQ

| M/O | Attributes/Array of attributes | Size (bits) |  | Value/Notes | Conditions |  |
| --- | --- | --- | --- | --- | --- | --- |
| … | … | … |  | … | … |  |
| O | N.6) Vendor-specific PHS parameters | variable |  | Compound |  |  |
| O | Number of HARQ Channels | 5 | Indicates the number of HARQ channels which are assigned to the flow. |  |  |  |
| O | fullNumHarqChArr[0…31] |  |  |  |  |  |
| O | A) HARQ Channel Mapping | 5 | Indicates the index (ACID) of each HARQ channel. |  |  |  |
| O | ROHC Parameter | variable |  |  |  |  |
| … | … | … | … |  | … |  |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-4: Add the following text, at line 34 in page 225, section 16.2.3.47.4, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.3.47.4. AAI_DSC-REQ

…

Table 740 – AAI_DSC-REQ

| M/O | Attributes/Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | N.6) Vendor-specific PHS parameters | variable | Compound |  |
| O | Number of HARQ Channels | 5 | Indicates the number of HARQ channels which are assigned to the flow. |  |
| O | fullNumHarqChArr[0…31] |  |  |  |
| O | A) HARQ Channel Mapping | 5 | Indicates the index (ACID) of each HARQ channel. |  |
| O | ROHC Parameter | variable |  |  |
| … | … | … | … | … |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-5: Add the following text, at line 29 in page 263, section 16.2.4.7, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.4.7 Padding

Within the data burst, the unused portion shall be initialized to a known state. If the size of the unused portion is only one byte then the unused byte is set to 0xF0. If the size of the unused portion is greater than or equal to two bytes, the unused first byte is set to 0xF0 and the remaining unused bytes are set to 0x00.

When the ABS grants UL burst to the AMS and the AMS has no data pending to be transmitted:

- If femto or macro hotzone ABS grants UL bandwidth in unsolicited manner, the AMS may not transmit any padding bytes.

- Otherwise, the AMS shall transmit a MAC PDU only with padding bytes on the UL grant.

-------------------------------------------------End proposed text----------------------------------------------------------------