| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Default TTI Support (Section 16.3.6.5) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Hyunkyu Yu, Ji-Yun Seol, Seho Kim Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution proposes to include A-MAP relevance field in UL basic assignment A-MAP IE to support default TTI in D<U. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Default TTI Support (Section 16.3.6.5)

Hyunkyu Yu, Ji-Yun Seol, Seho Kim

Samsung Electronics

- Introduction

In D6, only persistent allocation and group resource allocation can support default TTI when the number of DL subframes is smaller than the number of UL subframes (D<U). This is mainly for supporting delay-sensitive services such as VoIP.

When considering ranging channels and bandwidth request channels, because those control channels do not occupy the whole duration of UL subframes, the resource wastage may happen. Default TTI bursts can be allocated in those resources via PA or GRA, but it may not an efficient way. In 16m only mode, the case of D<U may be excluded from the main scenario. But it is different in mixed mode case.

Thus, we propose supporting default TTI using UL basic assignment A-MAP IE.

- Proposed Text Changes

[Remedy-1: Change the following text, at line 37 in page 565, section 16.3.6.5.2.4.2, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 852 – UL Basic Assignment A-MAP IE*

| Syntax | Size (bit) | Description/ notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bits |
| } |  |  |

------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Change the following text, at line 53 in page 589, section 16.3.6.5.2.4.9, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 860 – UL Persistent Allocation A-MAP IE*

| Syntax | Size (bit) | Description/ notes |
| --- | --- | --- |
| UL Persistent A-MAP IE () { |  |  |
| … | … | … |
| If (Allocation period == 0b00) { |  |  |
| Resource index | 11 | … |
| Long TTI indicator | 1 | … |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 6 | … |
| Reserved | 16 | … |
| Reserved | 15 | Reserved bits |
| … | … | … |
| Long TTI indicator | 1 | … |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP |
| HFA | 3 | HARQ Feedback Allocation |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b00: 2 0b01: 3 0b10: 4 0b11: 8 |
| Initial_ACID | 4 | … |
| If (D < U) { |  | D: number of DL AAI subframes, U: number of UL AAI subframes |
| Allocation relevance | 1 | 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } else { |  |  |
| Reserved | 1 | Reserved bits |
| } |  |  |
| Reserved | 1 | Reserved bits |
| } |  |  |
| } |  |  |

------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-3: Change the following text, at line 16 in page 592, section 16.3.6.5.2.4.10, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 861 – Group Resource Allocation A-MAP IE*

| Syntax | Size (bit) | Description/ notes |
| --- | --- | --- |
| Group_Resource_Allocation_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Group Resource Allocation A-MAP IE |
| if (UL Allocation && D <U){ |  | If group corresponds to UL allocations and the number of DL AAI subframes (D) is less than the number of UL AAI subframes (U) |
| Allocation Relevance | 12 | 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region Indicates the location of UL subframe relevant to this A-MAP. 0b00: the first UL subframe relevant to this A-MAP 0b01: the second UL subframe relevant to this A-MAP 0b10: third UL subframe relevant to this A-MAP 0b11: N.A |
| } |  |  |
| … | … | … |

------------------------------------------------End proposed text----------------------------------------------------------------