| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications for support of 8 streams in DL MU-MIMO in IEEE P802.16m/D6 |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Sangheon Kim, Taeyoung Kim, Jaewon Cho, Heewon Kang Samsung Electronics | Sangheon.kim@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D6 |  |
| Abstract | This contribution proposes change for 8 stream support in DL MU-MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification to support 8 streams of DL MU-MIMO 
in IEEE P802.16m/D6

Samsung Electronics

- Introduction

Downlink MU-MIMO mode has the limitation of the total number of streams by four, while SU-MIMO supports 8 streams. In practical environment, however, MU-MIMO is more feasible to have 8 streams than SU-MIMO which requires 8 receive antennas and spatial rank of 8. Additionally, under the general condition that AMSs have 2 or 4 receive antennas, 8 streams should be supported in MU-MIMO in order to obtain the peak rate. Therefore, this contribution proposes the modification for support of 8streams in downlink MU-MIMO. To support 8streams in DL MU-MIMO, Si field in DL basic assignment A-MAP IE needs to be modified. Since there are already several reserved cases in Si field, we can use those without using any additional bit.

- Simulation Results

We evaluated the performance gain of the 8stream MU-MIMO over 4stream MU-MIMO by system level simulation. Simulation condition is shown in the following table. According to the proposed text change, TNS of 5, 6, and 7 cannot be indicated. Therefore, 8stream pilot pattern and overhead are assumed even for the cases that 5 to 7 AMSs are assigned.

- Table 1 Simulation Condition

| Parameter | Value |
| --- | --- |
| Channel configuration | Uma ( ITU-R M.2135) |
| Velocity | 3km |
| ISD | 500m |
| Cell configuration | 19 cells, 3 sector |
| System Bandwidth | 10 MHz |
| Frequency Reuse | 3/3/3 |
| Number of User | 20 |
| BS Tx Antenna | ULA 8 antennas with 0.5 Lamda spacing |
| MS Rx Antenna | 2 antennas with 0.5 Lamda spacing |
| Permutation Type | SLRU |

Table 2 shows the simulation results. Although the 8stream pilot overhead is assumed for the cases of “TNS≥5”, we can see that 8stream MU-MIMO provides the higher average throughput than 4stream MU-MIMO. For the transformed codebook mode, 8stream MU-MIMO shows about 5.2% gain over 4stream MU-MIMO. For the sounding case assuming ideal channel estimation, we can obtain 12.6% higher throughput. Therefore, we recommend that 8stream MU-MIMO should be supported.

- Table 2 Simulation Results

|  | - Transformed codebook |  | - Sounding (ideal) |  |
| --- | --- | --- | --- | --- |
|  | - Max 4 streams | - Max 8 streams | - Max 4 streams | - Max 8 streams |
| - Sector throughput (Mbps) | - 39.19 | - 41.218 | - 50.565 | - 56.942 |
| - Edge throughput (Kbps) | - 1013.73 | - 1015.89 | - 1316.1 | - 1288.38 |

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D6, May. 2010.

Text Proposal

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

<Modify the section 16.3.6.5.2.4.1 on line 35, page 556 as shown >

_______________________________Start of the proposed text change #1 _____________________________

# 16.3.6.5.2.4.1 DL basic assignment A-MAP IE

Table 851 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

A-MAP IE Type: Defines the structure of the assignment A-MAP IE for the bits in the assignment A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between assignment A-MAP IEs used for the UL/DL, and assignment A-MAP IEs used for resource allocation and control signaling. Additional A-MAP IE types are reserved for future use.

Mt : Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

Si: Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission. For Co-MIMO transmission to two users, the ABS shall indicate to one AMS a total number of streams equal to 3 using one of the values 0b01000, 0b01001 or 0b01010, in order to ensure that the AMS will perform channel estimation using one stream of the non-interlaced pilot pattern with 3 streams.

| Table 851 - DL basic assignment A-MAP IE* |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Description/Notes |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt ≤ Nt Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 2 | Reserved bits |
| } else if(MEF == 0b10) { |  | Parameters for multi-layer encoding |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b00000: 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: 2 streams with PSI=stream0 and other modulation information not available 0b00100: 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: 2 streams with PSI=stream1 and other modulation information not available 0b01000: 3 streams with PSI=stream0 0b01001: 3 streams with PSI=stream1 0b01010: 3 streams with PSI=stream2 0b01011: 4 streams with PSI=stream0 0b01100: 4 streams with PSI=stream1 0b01101: 4 streams with PSI=stream2 0b01110: 4 streams with PSI=stream3 0b01111: 3 streams with PSI=stream0 and stream1 0b10000: 4 streams with PSI=stream0 and stream1 0b10001: 4 streams with PSI=stream2 and stream3 0b10010~0b11111: n/a 0b10010: 8streams with PSI=stream0 0b10011: 8streams with PSI=stream1 0b10100: 8streams with PSI=stream2 0b10101: 8streams with PSI=stream3 0b10110: 8streams with PSI=stream4 0b10111: 8streams with PSI=stream5 0b11000: 8streams with PSI=stream6 0b11001: 8streams with PSI=stream7 0b11010: 8streams with PSI=stream0 and stream1 0b11011: 8streams with PSI=stream2 and stream3 0b11100: 8streams with PSI=stream4 and stream5 0b11101: 8streams with PSI=stream6 and stream7 0b11110: reserved 0b11111: reserved |
| } else { |  |  |
| Reserved | 5 | Reserved bits |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 1 | Reserved bits |
| } |  |  |

_______________________________End of the proposed text change #1 ______________________________

<Modify Table 856 in the section 16.3.6.5.2.4.5 on line 1, page 577 as shown >

_______________________________Start of the proposed text change #2 _____________________________

# 16.3.6.5.2.4.5 Feedback Allocation A-MAP IE

Table 856 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels(including both PFBCH and SFBCH) to an AMS. …..

| Table 856 – Feedback Allocation A-MAP IE* |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Description/Notes |
| Feedback_Allocation_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Feedback Allocation A-MAP IE |
| Channel Index | 6 | Feedback channel index within the UL fast feedback control resource region (Dependent on LFB,FPi defined in 16.3.8.3.3.2) |
| … | … | … |
| if (MFM ≠ 1) { |  |  |
| MaxMt | Variable 1 or 2 | Variable number of bits – depends on number of transmit antennas Nt If Nt = 2: (Any MFM) 0b0:1 0b1:2 If Nt = 4: (Any MFM) 0b00:1 0b01:2 0b10:3 0b11:4 If Nt = 8: (SU-MIMO, MFM 0,2,3,4) 0b00:1 0b01:2 0b10:4 0b11:8 If Nt = 8: (MU-MIMO, MFM 5,6,7) 0b00:1 0b01:2 0b10:3 0b11:4 If Nt = 8: (MU-MIMO, MFM 6) 0b00:1 0b01:2 0b10:4 0b11:8 |
| } |  |  |
| if (MFM == 2,3,5,6) { |  |  |
| Feedback Format | 2 |  |
| } elseif (MFM = 4,7){ |  |  |
| Feedback Format | 1 |  |
| } |  |  |
| … | … | … |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

_______________________________End of the proposed text change #2 ______________________________

<Modify the section 16.3.7.1.3 on line 52, page 616 as shown >

_______________________________Start of the proposed text change #3 _____________________________

# 16.3.7.1.3 Downlink MIMO modes

There are six MIMO transmission modes for unicast DL MIMO transmission as listed in Table 872.

| Table 872 - Downlink MIMO modes |  |  |  |
| --- | --- | --- | --- |
| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| Mode 0 | OL SU-MIMO (Tx diversity) | SFBC | non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL MU-MIMO (SM) | ME | non-adaptive |
| Mode 4 | CL MU-MIMO (SM) | ME | adaptive |
| Mode 5 | OL SU-MIMO (Tx diversity) | CDR | non-adaptive |

.

The allowed values of the parameters for each DL MIMO mode are shown in Table 873.

| Table 873 - DL MIMO parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |
| MIMO mode 4 | 4 | 2 and 1* | 3 | 1 | 2 |
|  | 4 | 2 and 1** | 4 | 1 | 3 |
|  | 4 | 2 | 4 | 1 | 2 |
|  | 8 | 2 and 1* | 3 | 1 | 2 |
|  | 8 | 2 and 1** | 4 | 1 | 3 |
|  | 8 | 2 | 4 | 1 | 2 |
|  | 8 | 1 | 8 | 1 | 8 |
|  | 8 | 2 and 1*** | 8 | 1 | 7 |
|  | 8 | 2 and 1**** | 8 | 1 | 6 |
|  | 8 | 2 and 1***** | 8 | 1 | 5 |
|  | 8 | 2 | 8 | 1 | 4 |
| MIMO mode 5 | 2 | 1/2 | 1 | 2 | 1 |
|  | 4 | 1/2 | 1 | 2 | 1 |
|  | 8 | 1/2 | 1 | 2 | 1 |

* 2 streams to one AMS and 1 stream to another AMS, with 1 layer each.

** 2 streams to one AMS and 1 stream each to the other two AMSs, with 1 layer each.

*** 2 streams to one AMS and 1 stream each to the other six AMSs, with 1 layer each.

**** 2 streams each to two AMSs and 1 stream each to the other four AMSs, with 1 layer each.

***** 2 streams each to three AMSs and 1 stream each to the other two AMSs, with 1 layer each.

Mt refers to the number of MIMO streams transmitted to one AMS with MIMO modes 0, 1, 2 and 5.

Mt refers to the total number of MIMO streams transmitted to multiple AMS on the same RU with MIMO modes 3 and 4.

_______________________________End of the proposed text change #3 ______________________________

<Modify Table 878 in section 16.3.7.2.5.4 on line 3, page 627 as shown >

- _______________________________Start of the proposed text change #4 _____________________________

Table 878 - DL MIMO control parameters

| Parameter | Description | Value | Notes |
| --- | --- | --- | --- |
| Broadcast Information |  |  |  |
| Nt | Number of transmit antennas at the BS | 2,4,8 | Indicated in S-SFH (system information) SP3 IE |
| OL_Region | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and the number of MIMO streams (1 or 2) | Refer to AAI_SCD message (16.2.3.29) | Indicated in AAI_SCD Message |
| BC_SI | Rank-1 base codebook subset indication for interference mitigation with PMI coordination | BitMAP: 8bits if Nt = 2 16bits if Nt = 4, 8 | Rank-1 codebook element restriction or recommendation information It shall be ignored if Codebook_mode = 0b00, 0b01 or 0b10 It is indicated in AAI_DL_IM_Message Its usage is enabled by Codebook_mode or Codebook_coordination. |
| Unicast Information |  |  |  |
| MEF | MIMO encoder format | SFBC Vertical encoding Horizontal encoding Multi-layer encoding | MIMO encoder format. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| Mt | Number of MIMO streams in transmission | 1 to 8 | Number of MIMO streams in the transmission. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| SI | Index of allocated pilot MIMO stream | 1 to 48 | SI shall be indicated if MEF is HE ME Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent A-MAP IE |
| MFM | MIMO feedback mode | Refer to Table 877 | To decide the feedback content and related MS processing Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| MaxMt | Maximum number of MIMO streams | If Nt = 2(any MFM) 0b0:1 0b1:2 If Nt = 4(any MFM) 0b00:1 0b01:2 0b10:3 0b11:4 If Nt = 8(SU-MIMO MFM 0,2,3,4): 0b00:1 0b01:2 0b10:4 0b11:8 If Nt = 8(MU-MIMO MFM 5,6,7): 0b00:1 0b01:2 0b10:3 0b11:4 If Nt = 8(MU-MIMO MFM 6): 0b00:1 0b01:2 0b10:4 0b11:8 | If MFM indicates a SU feedback mode: the maximum STC rate scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users total streams for all users scheduled on each RU Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| Codebook_subset | Coedebook subset type for CL MIMO modes 2 and 4 | Base codebook or codebook subset | Depending on the MFM and Codebook_subset, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| Codebook_mode | Coedebook feedback mode for CL MIMO modes 2 and 4 | 0b00: base mode with codebook coordination disabled 0b01: transformation mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: base mode with codebook coordination enabled | This field specifies the codebook feedback mode. If codebook coordination is enabled by setting Codebook_mode to 0b11 or by setting Codebook_coordinattion to 0b1, and if the AMS reports STC rate equal to 1, then the AMS shall find the rate-1 PMI from the codebook entries broadcasted in BC_SI in AAI_DL_IM Message. |

- _______________________________End of the proposed text change #4 ______________________________