| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Structure of A-Preamble (16.3.6.1) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Sangheon Kim, Sung-Eun Park, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. Yang-Seok Choi Intel Corporation | sangheon.kim@samsung.com yang-seok.choi@intel.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D6 Section: 16.3.6.1 A-Preamble |  |
| Abstract | The contribution proposes the text changes to the structure of A-Preamble of the 802.16m draft amendment, IEEE P802.16m/D6. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to structure of A-Preamble 
(16.3.6.1)

Sangheon Kim, Sung-Eun Park, Jaeweon Cho, Heewon Kang 
Samsung Electronics Co., Ltd.

Yang-Seok Choi

Intel Corporation

- Introduction

The contribution proposes the text for changing the structure of A-Preamble to be included in the 802.16m amendment [1]. As shown in the following Figure1, D6 [1] specifies that LBS beacon signal of D-LBS zone is transmitted at the position of the last Secondary A-preamble in the pattern of synchronization signals S-P-S-S, where S and P mean the Secondary A-Preamble and the Primary A-Preamble, respectively. That is, if D-LBS zone is activated, the last SA-Preamble is replaced by LBS beacon signal and an AMS has to use the first two SA-Preambles to detect IDcell.

Figure1. D-LBS zone allocation

The information on whether or not D-LBS zone is activated is supposed to be broadcasted in SFH or AAI_SCD message. Therefore, even if D-LBS zone is not activated, an AMS which is trying network entry process and searching preambles has no information on D-LBS zone, and the AMS has no choice but to assume that the last SA-Preamble does not exist. Consequently, the usage of the last SA-Preamble is very limited even in a non-LBS zone mode and we propose to remove the last SA-Preamble as shown in the following Figure2.

Figure2. The proposed structure of A-preamble

- References

[1] IEEE P802.16m/D6, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” May 2010.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Change the text from line 39~49 of page 497 as follows]

There are two types of Advanced Preamble (A-Preamble): primary advanced preamble (PA-Preamble) and secondary advanced preamble (SA-Preamble). One PA-Preamble symbol and three two SA-Preamble symbols exist within the superframe. The location of the A-Preamble symbol is specified as the first symbol of frame except for the last frame. PA-Preamble is located at the first symbol of second frame in a superframe while SA-Preamble is located at the first symbol of remaining three frames the first and the third frame. Figure 511 depicts the location of A-Preamble symbols.

---------------------------------------------------  Text End  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-2: Change the Figure 511 in page 498 as follows]

Figure 511─Location of the A-Preamble

---------------------------------------------------  Text End  ---------------------------------------------------

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-3: Change the Figure 513 in page 500 as follows]

Figure 513─A-Preamble transmission structure supporting WirelessMAN-OFDMA

---------------------------------------------------  Text End  ---------------------------------------------------