| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification to UL Persistent Allocation A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Sangheon Kim, Hyunkyu Yu, Seho Kim, Jaewon Cho, Heewon Kang Samsung Electronics | xxx@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D6 Section: 16.3.6.5.2.4.9 UL Persistent allocation A-MAP IE |  |
| Abstract | The contribution proposes the modifications to UL Persistent allocation A-MAP IE of the 802.16m draft amendment, IEEE P802.16m/D6. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification to UL Persistent Allocation A-MAP IE (Section 16.3.6.5)

Sangheon Kim, Hyunkyu Yu, Seho Kim, Jaewon Cho, Heewon Kang

Samsung Electronics

- Introduction

This contribution provides four technical modifications in the UL persistent allocation A-MAP IE as follows.

- The field of TNS indicating 1stream

- Deleting the sentence about the compulsion to use long TTI in the case that the number of DL AAI subframes is less than the number of UL AAI subframes

- The field of N_ACID supporting a single ACID

- The field of ‘Allocation period’ supporting one frame period

First of all, TNS field should be modified to indicate “1 stream” in order to support SU-MIMO with a single stream. Unlike other assignment A-MAP IE, TNS field is also used for indicating SU-MIMO mode by being set to the same value as Mt. Therefore, SU vertical encoding with a single stream including SIMO is indicated by setting ‘TNS=1stream’ and ‘Mt =1stream’ simultaneously. The second is to delete the sentence compelling to use long TTI in a specific case. Because the field of ‘Allocation Relevance’ was added in the previous meeting, the compulsion is not needed anymore. The third modification is for the values of N_ACID. In the current D6 [1], the minimum number of ACIDs per persistent allocation is specified to two. But, when we consider the case that channel condition is good, such as a small cell or a cell-center user, one or two retransmissions are enough and we do not have to waste two or more ACIDs. Therefore, in this contribution, we suggest the use of a single ACID for PA. In addition to a user having a good channel condition, ‘N_ACID=1’ can be used for a cell-edge user which cannot transmit both the retransmission and the new initial transmission, simultaneously. In this case, ‘N_ACID=1’ provides the functions of limiting the retransmission and providing a simple operation by excluding the processes to assign the resource and transmit the additional assignment A-MAP for the retransmission. Finally, we propose the shorter allocation period of one frame. Regarding this modification, we note that the persistent allocation can be used for the purpose of latency reduction by pre-assigning some portion of resources to a user without bandwidth request process. Therefore, assigning the resource at every frame is expected to provide the shorter latency.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D6, May. 2010.

Text Proposal

<Modify the section 16.3.6.5.2.4.9 on line 17, page 589 as shown >

_______________________________Start of the proposed text change________________________________

- 16.3.6.5.2.4.9 UL Persistent allocation A-MAP IE

The UL persistent allocation A-MAP IE is specified in Table 860.

Table 860 - UL Persistent Allocation A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE(){ | - | - |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period ==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 1 frames 0b10: 4 2 frames 0b11: 8 4 frames |
| if (Allocation Period ==0b00){ | - | - |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL subframes for TDD |
| HFA | 6 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |
| Reserved | 16 | Reserved bits |
| } else if (Allocation Period != 0b00){ | - | - |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 1 | Number of streams in transmission (Mt ≤ Nt ), up to 2 streams per AMS supported Nt: Number of transmit antennas at the AMS 0b0: 1 stream 0b1: 2 streams |
| TNS | 2 | Total number of streams in the LRU for CSM 0b00: reserved 1stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if (TNS > Mt){ | - | - |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4 streams: 0b00, 0b01, 0b10, 0b11 |
| } |  |  |
| else if (TNS == Mt) { |  |  |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding |
| Reserved | 1 | Reserved bits |
| } | - | - |
| PF | 1 | Precoding Flag 0b0: non adaptive precoding 0b1: adaptive precoding using the precoder of rank Mt of AMS’s choice |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b00: 21 0b01: 32 0b10: 43 0b11: 84 |
| Initial_ACID | 4 | Initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| If (D<U) { |  | D: number of DL AAI subframes, U: number of UL AAI subframes |
| Allocation Relevance | 1 | 0b0: Allocation in the first UL subframes relevant to an A-MAP region 0b1: Allocation in the second UL subframes relevant to an A-MAP region |
| } else { | - | - |
| Reserved | 1 | Reserved bits |
| } | - | - |
| Reserved | 1 | Reserved bits |
| } | - | - |
| } | - | - |

- *A 16 bit CRC is generated based on the randomized contents of the UL Persistent A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 849.

- ________________________________End of the proposed text change_________________________________