| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text clean up on Relay Frame Structure (Section 16.6.3) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Youngbin Chang, Jaeweon Cho Samsung Electronics Jinyoung Chun, Jin Sam Kwak LG Electronics | E-mail: yb.chang@samsung.com E-mail: jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot Opening, P802.16m / Topic: Relay (section 16.6.3) |  |
| Abstract | Proposed Text on Relay Frame Structure |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D7 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text clean up on Relay Frame Structure

Youngbin Chang, Jaeweon Cho

Samsung Electronics

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

This contribution proposes technical and editorial changes and a number of clarifications to the current amendment text describing frame structure supporting ARS operation and to be included in the 802.16m amendment draft [1]. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. In this contribution, we propose the clarification text of frame configuration for ARS operation.

The following criteria are used to define frame configuration for ARS operation:

- Frame configuration for ARS should be transparent to AMS attaching ABS.

- Ratio of Access and Relay zone should be decided not to change HARQ timing in section 16.2.14.2.2

- References

- IEEE P802.16m/D5, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the P802.16m/D6

Black text: existing D6 text

Red and strike out text: for deletion

Blue and underline text: for addition

============================= Start of the text #1==============================

[Note to Editor – Modify the text in section 16.6.3.1 Basic frame structure supporting ARS in page 826 as follows]

16.6.3.1 Basic frame structure supporting ARS

The advanced air interface supports two hop data transmission between an ABS and an AMS using an intermediate ARS. Figure 599 shows an example of the basic frame structure for system supporting ARSs. When an ARS is deployed it shall use the same OFDMA signal parameters (defined in Table 779) as its serving ABS. The ABS and ARS superframes shall be time aligned and shall consist of the same number of frames and AAI subframes. Every ARS superframe shall contain a superframe header (SFH). The SFH transmitted by the ARS shall have the same location and format as the SFH transmitted by the ABS. The ARS preambles (SA-Preamble and PA-Preamble) shall be transmitted synchronously with superordinate ABS preambles.

When ARSs are supported the ABS frame is divided on AAI Access zone and AAI Relay zone. The AAI Access zone position precedes the AAI Relay zone position inside the frame. The duration of the AAI Access zone and AAI Relay zone may be different in DL and UL directions. The zone configuration of AAI Access zone and AAI Relay zone is informed to the ARS by the ABS in AAI_ARS-CONFIG-CMD RS_config-CMD message.

The ABS frame AAI Access zone shall consist of AAI DL Access zone and AAI UL Access zone, and AAI Relay zone shall consist of AAI DL Relay zone and AAI UL Relay zone. The ABS frame AAI Access zone shall be used for communication with AMSs only. The ABS frame AAI Relay zone shall be used for communication with ARSs and may be used for communication with AMSs. In the AAI DL Relay zone the ABS shall transmit to its subordinate ARS and in the AAI UL Relay zone the ABS shall receive transmissions from its subordinate ARS.

The ARS frame AAI Access zone shall consist of AAI DL Access zone and AAI UL Access zone and AAI Relay zone shall consist of AAI DL Relay zone and AAI UL Relay zone. The ARS frame AAI Access Zone shall be used for communication with AMSs only. In the AAI DL Relay zone the ARS shall receive transmissions from its superordinate ABS and in the AAI UL Relay zone the ARS shall transmit to its superordinate ABS.

The position of AAI DL/UL relay zone is indicated for AMS using AAI_System Configuration Descriptor message when AAI_Relay_zone_AMS_allocation_indicator is equal to 0.

The frame configuration of AAI DL/UL Access zone and relay zone in the frame is decided when ARS or AMS receives frame configuration index in S-SFH SP1. Some of frame configurations are not supported for ARS operation. The possible frame configurations for ARS operation are shown in table zzz (see section 16.6.3.2.3). The ratio of AAI DL/UL Access zone and relay zone is shown in table zzz.

In each ARS frame, the relay transmit to receive transition interval (R-TTI) may be inserted in order to make allowances for ARSTTG and RTD between the ARS and its superordinate station.

In each ARS frame, the relay receive to transmit transition interval (R-RTI) may be inserted in order to make allowances for ARSRTG and RTD between the ARS and its superordinate station.

The duration and the positions of R-TTI and R-RTI are defined in FDD (see 16.6.3.2.1) and TDD (see 16.6.3.2.2), respectively. The each R-TTI and R-RTI shall be equal or less than one OFDMA symbol.

============================= End of the text #1==============================

============================= Start of the text #2==============================

[Note to Editor – Modify the text in section 16.6.3.2.1 FDD frame structure in page 828 as follows]

16.6.3.2.1 FDD frame structure

The FDD frame shall be constructed on the basis of the basic frame structure defined in 156.3.3.

An ARS supporting FDD mode shall communicate with the ABS using full duplex FDD mode and it shall be able to simultaneously support half duplex and full duplex AMSs operating on the same RF carrier.

An ARS in FDD systems shall use DL carrier frequency (FDL) for receiving from the ABS in the AAI DL Relay zone and shall use UL carrier frequency (FUL) for transmission to the ABS in the AAI UL Relay zone.

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted between two ARS frames. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. In ARS DL frame, R_IdleTime shall be same as IdleTime of ABS. In ARS UL frame, the duration of R_IdleTime shall be less or equal to the duration of IdleTime of ABS. The duration of the R_IdleTime for the ARS UL frame is signaled by the ARS to its AMSs through the AAI_SCD message. The ARS UL frame may be time-advanced for a Tadv Tadv interval referring to the start of the ABS UL frame. The duration of Tadv Tadv is calculated according to the following equation:

Tadv= IdleTime−R_IdleTime									(318)

When an ARS switches transceiver states from transmit to receive or from receive to transmit, a DL ARS radio frame in FDD system may have an R-TTI between AAI Access and AAI Relay zone and an R-RTI between AAI Relay zone and AAI Access zone in the next DL ARS frame. The location of the R-TTI is the last OFDM symbol of the last AAI subframe of AAI Access zone and the location of the R-RTI is the last OFDM symbol of the last AAI subframe of the AAI Relay zone. The durations of R-TTI and R-RTI in DL for each ARS shall be calculated by following equations:

R-TTI =0 if RTD2≥ARSTTGTs if RTD2<ARSTTG								(319)

R-RTI = 0 if IdleTime−RTD2≥ARSRTGTs if IdleTime−RTD2<ARSRTG							(320)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table 762802>>.

If the R-TTI is present (i.e. its duration is equal to Ts), then the ARS DL Access zone AAI subframe with the R-TTI is formed by type-3 or type-1 AAI subframe when the corresponding ABS DL Access zone AAI subframe is type-1 or type-2 AAI subframe, respectively. If there is no R-TTI (i.e. its duration is equal to zero), the AAI subframes in the DL Access zone at ARS are the same as those at ABS.

When an ARS switches transceiver states from receive to transmit or from transmit to receive, an UL ARS radio frame in FDD system may have a R-RTI between AAI Access and AAI Relay zone and a R-TTI between AAI Relay zone and AAI Access zone in the next UL ARS frame. The location of the R-RTI is the first OFDM symbol of the first AAI subframe of AAI Relay zone and the location of R-TTI is the last OFDM symbol of the last AAI subframe of AAI Relay zone. The duration of R-RTI and R-TTI in UL for each ARS shall be calculated by following equations:

R−RTI=0 if Tadv≥ARSRTG+RTD2Ts if Tadv<ARSRTG+RTD2							(321)

R−TTI=0 if R_IdleTime+RTD2≥ARSTTGTs if R_IdleTime+RTD2<ARSTTG						(322)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table  762802>>.

Figure 600— illustrates an example frame structure with ARS support for FDD mode, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8. All transition intervals R-TTI and R-RTI in the figure example are equal to the duration of one OFDMA symbol.

============================= End of the text #2==============================

============================= Start of the text #3==============================

[Note to Editor – Modify the text in section 16.6.3.2.2 TDD frame structure in page 830 as follows]

16.6.3.2.2 TDD frame structure

The TDD frame shall be constructed on the basis of the basic frame structure defined in 156.3.3.

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted before the switching point from DL to UL. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor AAI_SCD message. The ARS UL frame may be advanced (Tadv) referring to the start of ABS UL frame. The duration of Tadv Tadv is calculated according to the following equation:

Tadv=TTG−R_IdleTime									(323)

The duration of R_IdleTime shall be less or equal to the TTG. In each ARS radio frame, the RTG interval shall be inserted before the switching point from UL to DL.

An ARS radio frame in TDD system shall have an R-TTI transition interval in DL between the AAI Access zone and the AAI Relay zones. The location of the R-TTI is the last OFDM symbol of the last AAI subframe of the AAI DL Relay zone. The duration of the R-TTI in DL for each ARS shall be calculated by following equation:

R-TTI =0 if RTD2≥ARSTTGTs if RTD2<ARSTTG								(324)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table  762802>>.

If the R-TTI is present (i.e. its duration is equal to Ts), then ARS DL Access zone AAI subframe with the R-TTI is formed by type-3 or type-1 AAI subframe when the corresponding ABS DL Access zone AAI subframe is type-1 or type-2 AAI subframe, respectively. If there is no R-TTI (i.e. its duration is equal to zero), the AAI subframes in the DL Access zone at ARS are the same as those at ABS.

An ARS radio frame in TDD system shall have an R-RTI transition interval in UL between AAI Access and AAI Relay zones. The location of the R-RTI is the first OFDM symbol of the first AAI subframe of the AAI UL Relay zone. The duration of the R-RTI in UL for each ARS shall be calculated by the following equation:

R-RTI = 0 if Tadv−RTD2≥ARSRTGTs if Tadv−RTD2<ARSRTG							(325)

wWhere the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table  762802>>.

Figure 601— illustrates an example frame structure with ARS support for TDD mode D:U = 5:3, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8. The number of AAI subframes allocated to the Relay zone in DL direction is two and in UL direction is one. The duration of R-TTI and R-RTI is equal to the duration of one OFDMA symbol.

============================= End of the text #3==============================

============================= Start of the text #4==============================

[Note to Editor – Add the text after section 16.6.3.2.2 TDD frame structure in page 831 as follows]

16.6.3.2.3 Set of frame configurations for ARS operation

ABS or ARS informs sets of the frame configurations and indexing for AMS in S-SFH SP1 IE in Table 807, 808 and 809. Some sets of the frame configuration and indexing are not used when ABS supports ARS. If basic frame structure in TDD or FDD is decided, the ratio of access zone and relay zone is also decided by table zzz..

Table zzz - Frame Configuration for ARS operation

| Bandwidth (MHz) | CP | Frame configuration index | DL Access zone : DL relay zone ratio | UL Access zone : DL relay zone ratio | Support of ARS MIMO Midamble transmission | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 5/10/20 | 1/16 | 0 | 3:3 | 1:1 | Yes |  |
| 5/10/20 | 1/16 | 1 | 3:2 | 2:1 | Yes |  |
| 5/10/20 | 1/16 | 2 | 2:2 | 2:2 | Yes |  |
| 5/10/20 | 1/16 | 3 | 2:1 | 3:2 | Yes |  |
| 5/10/20 | 1/16 | 4 | 4:4 | 4:4 | Yes |  |
| 5/10/20 | 1/8 | 0 | 3:3 | 1:1 | Yes |  |
| 5/10/20 | 1/8 | 1 | 3:2 | 2:1 | Yes |  |
| 5/10/20 | 1/8 | 2 | 3:1 | 3:1 | No |  |
| 5/10/20 | 1/8 | 4 | 4:4 | 4:4 | Yes |  |
| 5/10/20 | 1/8 | 17 | 3:3 | 1:1 | Yes |  |
| 5/10/20 | 1/8 | 18 | 3:2 | 2:1 | No |  |
| 5/10/20 | 1/4 | 3 | 4:3 | 4:3 | Yes |  |
| 8.75 | 1/16 | 0 | 2:3 | 1:1 | Yes |  |
| 8.75 | 1/16 | 1 | 2:2 | 2:1 | Yes |  |
| 8.75 | 1/16 | 2 | 2:1 | 3:1 | Yes |  |
| 8.75 | 1/16 | 3 | 3:3 | 3:3 | Yes |  |
| 7 | 1/16 | 2 | 4:2 | 4:2 | Yes | . |
| 7 | 1/8 | 2 | 3:2 | 3:2 | Yes |  |
| 7 | 1/4 | 2 | 3:2 | 3:2 | Yes |  |

============================= Start of the text #4==============================

============================= Start of the text #5==============================

[Note to Editor – Add new subsection 16.6.3.3.3 Downlink mapping of data subcarriers to the section 16.6.3.3 Relay Downlink PHY structure in page 736]

16.6.3.3.3 Downlink data subcarrier mapping

When the AAI_Relay_zone_AMS_allocation_indicator is set to 1, (i.e. ABS is allowed to transmit to AMSs and ARSs in AAI DL Relay zone), The downlink physical structure for both data and pilot described in 16.3.5 is used for DL Access and Relay zone at ABS and ARS frame. When R-RTI is equal to Ts in FDD frame structure, ARS decodes all data and pilot sub-carriers in the last subframe of AAI Relay zone as the last symbol of the subframe is punctured.

When the AAI_Relay_zone_AMS_allocation_indicator is set to 0, (i.e. ABS is only allowed to transmit to ARSs in AAI DL Relay zone), The downlink physical structure for both data and pilot described in 16.3.5 is used for DL Access and Relay zone at ABS frame. When ARS MIMO-Midamble or R-RTI in AAI DL Relay zone are allocated in the type-1 AAI subframe, all data and pilot subcarrier mapping of the type-1 AAI subframe shall be converted to type-3 AAI subframe. When MIMO-Midamble or R-TTI are allocated in the type-2 AAI subframe, all data and pilot subcarrier mapping of the type-2 AAI subframe shall be converted to type-1 AAI subframe.

============================= End of the text #5==============================

============================= Start of the text #6==============================

[Note to Editor – Modify the text in section 16.6.3.4 Downlink Control Structure in page 814 as follows]

16.6.3.4 Downlink Control Structure

In the AAI DL Access zone of the ABS frame and ARS frame, all the DL control channels described in 16.3.6 are reused.

In the AAI DL Relay zone, only the A-MAP control channel described in 16.3.6 is reused for communication between the ABS and the ARS.

When the AAI_Relay_zone_AMS_allocation_indicator is set to 1, the DL control channels described in 16.3.6 are reused for DL Access and Relay zone at ABS and ARS frame. When R-RTI is equal to Ts in FDD frame structure, ARS decodes all control channel subcarriers in the last subframe of AAI Relay zone as the last symbol of the subframe is punctured.

When the AAI_Relay_zone_AMS_allocation_indicator is set to 0, the DL control channels described in 16.3.6 is used for DL Access and Relay zone at ABS frame. When RS MIMO-Midamble or R-RTI in AAI DL Relay zone are allocated in the type-1 AAI subframe, all control channels of the type-1 AAI subframe shall be converted to type-3 AAI subframe. When MIMO-Midamble or R-TTI are allocated in the type-2 AAI subframe, all all control channels of the type-2 AAI subframe shall be converted to type-1 AAI subframe.

============================= End of the text #6==============================

============================= Start of the text #7==============================

[Note to Editor – Add the field in table 716, line 42, page 157 as follows:]

| M/O | Attributes/Array of attributes | Size (bits) | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| O | RELAY_Support_Parameters | 16m_Relay_zone_AMS_allocation_indicator | 1 | 0 or 1 | Relay support parameters |
|  |  | R_Idle_Time | 14 | 0-16384 (0.01 us granularity) | Relay support parameters |

============================= End of the text #7==============================