| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for modification of IoT value signaling (16.2.3.31) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Jeongho Park, Ji-Yun Seol, Hokyu Choi Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D6 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for modification of IoT value signaling (16.2.3.31)

Jeongho Park, Ji-Yun Seol, Hokyu Choi

Samsung Electronics

- References

[1] IEEE 802.16m/D6

- Introduction and Problem Definition

Currently uplink target SINR range is -9~40dB considering both of control and data channel. And UL IoT range is 0~63.5dB. For extreme case, two frequency resource from one AMS has different tone power which range is -9~103.5dB.

For uplink FFR, usual target operation IoT value is around 10dB or less. And IoT difference between frequency partitions would not exceed to 10dB. Power boosted frequency partition could have a few dB lower IoT level and power de-boosted frequency partitions could have a few dB higher IoT level.

So it would be better to modify signaling method for IoT information of frequency partitions.

- Proposed Text

Remedy #1:page 162, Modify line 17~50 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

16.2.3.31 AAI_UL Noise and Interference Level Broadcast Message

An AAI_ULPC_NI broadcast NI IoT (interference over themal noise) values for the UL power control.

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | UlNiInfoSet | iotSounding | 7 | IoT value of sounding, quantized in 0.5 dB steps as IoT level from 0 dB(0x00) to 63.5 dB(0x3F). | Present when sounding is used |
| O | UlNiInfoSet | iotFP0 | 7 | IoT value of Frequency Partition #0, quantized in 0.5 dB steps as IoT level from 0 dB(0x00) to 63.5 dB(0x3F). | Present when FP0 is used |
| O | UlNiInfoSet | iotFP1 | 75 | Differential IoT value of Frequency Partition #1 from Frequency Partition #0, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB -9dB(0x00) to 6.5dB(0x1F). | Present when FP1 is used |
| O | UlNiInfoSet | iotFP2 | 75 | Differential IoT value of Frequency Partition #2 from Frequency Partition #0, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB -9dB(0x00) to 6.5dB(0x1F). | Present when FP2 is used |
| O | UlNiInfoSet | iotFP3 | 75 | Differential IoT value of Frequency Partition #3 from Frequency Partition #0, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB -9dB(0x00) to 6.5dB(0x1F). | Present when FP3 is used |

For the open-loop power control, UL noise and interference level IoT level shall be broadcast to AMSs in the given ABS coverage by ABS. All the UL noise and interference noise level are quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB.

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #2:page 878, Modify line 14~26 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

AAI-ULPC-NI ::= SEQUENCE {

messageType				 	OCTET STRING (SIZE(1)),

-- syntax error UlNiInfoSet ::= SEQUENCE {

ulNiInfoSet SEQUENCE {

iotSounding					INTEGER (0..127) OPTIONAL,

iotFP0 					INTEGER (0..127) OPTIONAL,

iotFP1						INTEGER (0..12731) OPTIONAL,

iotFP2 					INTEGER (0.. 12731) OPTIONAL,

iotFP3 					INTEGER (0.. 12731) OPTIONAL}

}

-------------------------------------------------Text Change END -----------------------------------------------------