| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Feedback Polling A-MAP IE text clean-up (Section 16.3.6.5.2.4.11) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Eugene Visotsky, Bill Hillery, Fred Vook, Mark Cudak Motorola Inc. | eugenev@motorola.com |
| Re: | Category: P802.16m/D6 comments for sb_16m Area: Section 16.3.6.5.2.4.11 (Feedback Polling A-MAP IE) |  |
| Abstract | Description of Feedback Polling A-MAP IE signaling (Section 16.3.6.5.2.4.11) is not clear and leaves many feedback operation details ambiguous. This contribution proposes text changes in Section 16.3.6.5.2.4.11 in order to clarify Feedback Polling A-MAP IE operation. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Feedback Polling A-MAP IE text clean-up

(Section 16.3.6.5.2.4.11)

Eugene Visotsky, Bill Hillery, Fred Vook, Mark Cudak

Motorola, Inc.

# Introduction

Description of Feedback Polling A-MAP IE signaling (Section 16.3.6.5.2.4.11) is not clear and leaves many feedback operation details ambiguous. This contribution proposes text changes in Section 16.3.6.5.2.4.11 in order to clarify Feedback Polling A-MAP IE operation.

# Proposed Text Changes in IEEE P802.16m/D6

[Modify Section 16.3.6.5.2.4.11, beginning on page 593, as follows:]

16.3.6.5.2.4.11 Feedback Polling A-MAP IE

The information element shown in Table 862 is used by the ABS to schedule MMO MIMO feedback transmission by the AMS. In response to Feedback Polling A-MAP IE, The the AMS sends the MIMO feedback using a MAC control message or a MAC signaling header either AAI_SingleBS_MIMO_FBK message or MIMO Feedback Header (MFH) or Correlation Matrix Feedback Header (CMFH), depending on the requested feedback content and number of transmit antennas at the ABS.

When the Polling_sub_type bit is set to 0b0, a dedicated UL allocation is included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Polling_sub_type bit is set to 0b1, no dedicated UL allocation is included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall transmit in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE in the same AAI subframe with feedback periods designating the same transmission frames.

The AMS shall send the feedback in a MAC signaling header MIMO Feedback Header when the following feedback information wideband information for any combinations of MIMO feedback modes 0, 4 and 7 with q = 0 is requested.

•wideband information for any combinations of MIMO feedback modes 0, 4 and 7 with q = 0. The AMS shall send the feedback for all other MFM combinations in a MAC control message AAI_SingleBS_MIMO_FBK message .

The coefficients of the quantized transmit correlation matrix should shall be feedback fed back in Correlation Matrix Feedback Header (CMFH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet burst and if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should shall be feedback fed back in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas. Specifically, when long and short feedback periods are activated (p > 0 and q > 0) and coincide, long and short period feedback information shall be combined and transmitted in the AAI_SingleBS_MIMO_FBK message.

[Modify Section 16.3.6.5.2.4.11, beginning on page 603 line 52, as follows:]

For MFM 0 or 1: When p > 0, q = 0, the wideband CQI and wideband STC rate shall be transmitted using short feedback period. p > 0 and q > 0 or p = 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: When p > 0, q = 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using short feedback period. p > 0 and q > 0 or p = 0 and q > 0 shall not be set at the same time with MFM 2 or 5.