| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on update of S-SFH IEs (Section 16.2.24) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | sb_16m |  |
| Abstract | The current text defines that the ABS may update S-SFH IE only in certain intervals. It is unclear if this restriction applies to the ABS communicating that an update will happen (ie. Changing the change count) or to the ABS actually sending the updated S-SFH IEs. |  |
| Purpose | Adoption of proposed text by P802.16m TG. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on update of S-SFH IEs (Section 16.2.24)

Stavros Tzavidas

Motorola

# Introduction

The current text defines that the ABS may update S-SFH IE only in certain intervals. It is unclear if this restriction applies to the ABS communicating that an update will happen (ie. Changing the change count) or to the ABS actually sending the updated S-SFH IEs.

More specifically, section 16.2.24, p. 421, line 65 specifies that the ABS shall “indicate” any changes of SFH-IE at specific superframes (those that satisfy SFN modulo “S-SFH change cycle” == 0). It is unclear if this means that the ABS is allowed to change the S-SFH change count in these frames or if the change count can be changed at any time but the new versions of the S-SFH IE (those that changed and caused the change count to be incremented) can only be sent at these specific superframes.

# Proposed Text

[Modify paragraph 16.2.24, p. 421, line 65,  as indicated below:]

16.2.24 Update of S-SFH IEs

The S-SFH subpacket (SP) IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP3 IE) are transmitted by ABS at the scheduled period of each SP, respectively, with different periodicity (i.e., TSP1 < TSP2 < TSP3). This SP scheduling periodicity information is transmitted in the S-SFH SP3 IE. Otherwise, an AMS may implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE.

Every superframe, the ABS transmits P-SFH IE containing the S-SFH scheduling information, S-SFH change count, S-SFH SP change bitmap, and S-SFH applying offset. The S-SFH change count shall remain unchanged as long as all of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH SP IEs remain unchanged. The S-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH IEs change. The changed S-SFH SP IE(s) shall be transmitted at the scheduled superframes of period corresponding to each S-SFH SP IE.

If the AMS determines that the "S-SFH change count" field in P-SFH has not changed, then the AMS determines that it has up to date information.

The ABS shall send the new values of any updated indicate any changes of S-SFH IE only at the specific super-frame number(SFN) that satisfies SFN modulo S-SFH change cycle = 0.