| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Change of Relay FDD Frame Structure for HARQ Timing (16.3.6) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics Youngbin Chang, Jaeweon Cho Samsung Electronics | jiny.chun@lge.com yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot |  |
| Abstract | This contribution provides the change of Relay FDD frame structure for HARQ timing. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Change of Relay FDD Frame Structure for HARQ Timing (16.3.6)

Jinyoung Chun, Jin Sam Kwak

LG Electronics

Youngbin Chang, Jaeweon Cho

Samsung Electronics

- Introduction

When ABS supports ARS, ABS-AMS link shall be operated regardless of ARS existence. But Relay HARQ operation can be different between relay and macro, especially in FDD. And when cell specific mapping is ‘on’, that is, ‘AAI_Relay_zone_AMS_allocation_indicator = 0’ in FDD, AMS served by ABS transmits and receives in Access zone and can’t follow the FDD HARQ timing as Figure A. Also AMS served by ARS has the same problem.

Figure A. An example of FDD HARQ timing when ABS supports ARS

Therefore we suggest a solution that Relay zone precedes Relay zone inside the UL FDD frame and the ratio of Access Zone and Relay Zone is fixed to 4:4.

- Text proposal in the P802.16m/D7

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 53 of page 826 as below:]

When ARSs are supported the ABS frame is divided on AAI Access zone and AAI Relay zone. The AAI Access zone position precedes the AAI Relay zone position inside the TDD frame and FDD DL frame. The AAI Relay zone position precedes the AAI Access zone position inside the FDD UL frame. The duration of the AAI Access zone and AAI Relay zone may be different in DL and UL directions. The zone configuration of AAI Access zone and AAI Relay zone is informed to the ARS by the ABS in RS_config-CMD message.

[Modify the text in line 61 of page 828 as below:]

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted between two ARS frames. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. In ARS DL frame, R_IdleTime shall be same as IdleTime of ABS. In ARS UL frame, the duration of R_IdleTime shall be less or equal to the duration of IdleTime of ABS. The ARS UL may be time-advanced or time-retreated for a Tadv interval referring to the start of the ABS UL frame. The duration of Tadv is calculated according to the following equation:

[Modify the text in line 39 of page 829 as below:]

When an ARS switches transceiver states from receive to transmit or from transmit to receive, an UL ARS radio frame in FDD system may have a R-RTI between AAI Access in the previous UL ARS frame and AAI Relay zone and a R-TTI between AAI Relay zone and AAI Access zone in the next UL ARS frame. The location of the R-RTI is the first OFDM symbol of the first AAI subframe of AAI Relay zone and the location of R-TTI is the last OFDM symbol of the last AAI subframe of AAI Relay zone. The duration of R-RTI and R-TTI in UL shall be calculated by following equations:

[Modify the figure 603 in line 1 of page 830 as below:]

Figure 603 – Example of ARS FDD frame structure with G=1/8 in 5/10/20MHz

-----------------------------------------------   Text End  -----------------------------------------------------------