| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Allocation Method of MIMO Midamble in Relay Zone (16.3.6) |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics Youngbin Chang, Jaeweon Cho Samsung Electronics Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | jiny.chun@lge.com yb.chang@samsung.com alexander.maltsev@intel,com alexey.khoryaev@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot |  |
| Abstract | This contribution provides the allocation method of MIMO midamble in Relay zone. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Allocation Method of MIMO Midamble in Relay Zone (16.3.6)

Jinyoung Chun, Jin Sam Kwak

LG Electronics

Youngbin Chang, Jaeweon Cho

Samsung Electronics

Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov

Intel Corporation

- Introduction

In IEEE 802.16m [1], if ABS sends a MIMO midamble in relay zone for ARS, ABS’s frame structure should be changed. When If AAI_Relay_zone_AMS_allocation_indicator is ‘1, the AMS served by ABS can not decode control and data burst in the sub-frame where RS MIMO midamble is used. Then AMS has to know the changed frame structure and it means ARS affects normal ARS operation which does not support ARS operation.

Two problems can be an issue.

- AMS served by ABS can not decode control and data burst in the sub-frame where RS MIMO midamble is used.

- Long TTI can not be allocated in the subframe where RS MIMO midamble is used.

We propose two options to solve the problem here.

- Text proposal in the P802.16m/D7

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 37 of page 828 as below:]

When an ARS is supported, the long TTI allocations in ABS-ARS link shall span the entire AAI Relay zone in any DL or UL directions and the long TTI allocations in ARS-AMS link shall span the entire AAI Access zone in any DL or UL directions. When AAI_Relay_zone_AMS_allocation_indicator is equal to 0, the long TTI allocations in ABS-AMS link shall not be supported span the entire AAI Access zone in any DL or UL directions. When AAI_Relay_zone_AMS_allocation_indicator is equal to 1, the long TTI allocations in ABS-AMS link shall span the entire DL or UL subframes in a TDD frame or occupy 4 AAI subframes for both DL and UL in a FDD frame.

-----------------------------------------------   Text End  -----------------------------------------------------------

- References

[1] IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”