| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Ranging Opportunity Index in the AAI_HO-CMD message (16.2.3.11) |  |
| Date Submitted | 2010-07-11 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | sb_16m Section 16.2.3.11 AAI_HO-CMD |  |
| Abstract | This contribution provides a clarification on Ranging Opportunity Index in the AAI_HO-CMD message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on Ranging Opportunity Index in the AAI_HO-CMD message (16.2.3.11)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides a clarification on the field ‘Ranging Opportunity Index’ in the AAI_HO-CMD message.

The opportunity_index is used in RAID of CDMA allocation IE when BS responds to the ranging code. The RAID is defined as follows in pp. 556, D6: RAID = (LSB 5bits of superframe number | frame_index | preamble_code_index | opportunity_index).

If the ranging opportunity is allocated by broadcast manner, i.e., allocated either by SFH or Broadcast Assignment A-MAP IE, all MSs can count the exact ranging opportunity index for each ranging opportunity. So, everything is clear.

However, if the ranging opportunity is allocated in unicast manner, i.e., allocated by the AAI_HO-CMD message, only the MS which receives the AAI_HO-CMD message can know the existence of unicast ranging opportunity. This causes ambiguity of the ranging opportunity index.

In the following figure, there are four ranging opportunities. Two are broadcasted (by SFH and Broadcast Assignment A-MAP IE). But yellow two ranging opportunities are not visible to other MSs because it assigned in unicast manner. In this case, MS A indexes RCH1 as 0, RCH3 as 1, and RCH2 as index 2. On the other hand, MS B indexes RCH1 as 0, RCH2 as 1, and RCH 4 as 2. And all other MSs index RCH1 as 0, RCH2 as 1. These ambiguity needs to be clarified.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Update the field Ranging Opportunity Index in Table 691 in line 56, page 109 in D6}

16.2.3.11 AAI_HO-CMD

…

Table 691— AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | Ranging opportunity index | 3 | Indicates the ranging opportunity index of the dynamic ranging channel used in the RAID. Ranging opportunity index shall be assigned by the target ABS. The target ABS shall assign unique ranging opportunity index whcih is not overlapped with other ranging channel in the allocated frame. | Shall be included when Action Time refers to a frame where a dynamic ranging channel is allocated |
| O | Subframe index | 2 | Indicates the subframe index of the allocated ranging opportunity | Shall be included when Action Time refers to a frame where a dynamic ranging channel is allocated. The subchannel of a dynamic rangin channel is same as the ranging channel allocated by SFH. |
| … |  |  |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------