| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | R1 BS information in the AAI_NBR-ADV message (16.2.3.12) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011: I EEE 802.16 Working Group Letter Ballot #31 Section 16.2.11.1.1 Contention-based random access bandwidth request |  |
| Abstract | This contribution suggests add optional R1 BS information in the AAI_NBR-ADV message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

R1 BS information in the AAI_NBR-ADV message (16.2.3.12)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution suggests add optional R1 BS information (BS ID and basic physical parameters in the AAI_NBR-ADV message.

To support 16m only ABS to R1 BS, the AAI_NBR-ADV message should include legacy information such as BS ID and basic physical parameters (e.g., preamble index, channel bandwidth and CP length).

Moreover, these values can be added in mixed-mode ABS as well. It is the ABS's decision whether to include 'NBR-ADV offset/interval in LZone' or 'basic R1 BS information' which is proposed in this proposal.

# Proposed Text

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2.3.12 AAI_NBR-ADV

…

{Update Table 692 in line 18, page 117 in D6}

Table 692—AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV, and field Number of neighbor R1 BS is not present. |
| O | NBR-ADV interval: | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV, and field Number of neighbor R1 BSs is not present. |
| O | Number of neighbor R1 BSs | 8 | The number of neighbor R1 BS | It shall be included when the existence of neighbor R1 BS if NBR-ADV offset and NBR-ADV interval fields are not present. |
| O | Neighbor BSID | 48 | Neighbor Base Station ID | It shall be included per each neighbor R1 BS if Number of neighbor R1 BSs is present. |
| O | Preamble index/Subchannel Index | 8 | Defines the PHY-specific preamble for each neighbor R1 BS. Bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. Bit 6 to bit 0 represent the preamble index of the neighbor R1 BS. | It shall be included per each neighbor R1 BS if Number of neighbor R1 BSs is present. |
| O | PHY mode ID | 16 | Specifies the PHY parameters of each neighbor R1 BS, including channel bandwidth, FFT size, cyclic prefix, and frame duration, defined in 11. 18. 2 | It shall be included per each neighbor R1 BS if Number of neighbor R1 BSs is present. |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------