| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification on the range of Initial Sleep/Final Sleep/Listening Window (section 16.2.3.24) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Yeongmon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | “IEEE 802 Sponsor Ballot” |  |
| Abstract | The contribution proposes modification on the range of Initial Sleep/Final Sleep/Listening Window to use full range of Initial Sleep/Final Sleep/Listening Windows. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification on the range of Initial Sleep/Final Sleep/Listening Window

Yeongmoon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

- Introduction

To use full range of Initial Sleep/Final Sleep/Listening Windows, we need to redefine them.

- Text to change in D6

-------------------------------  Text Start  ---------------------------------------------------

[Modify the table 710 on the page 145, line 50 as follows:]

| Table 710—Parameters for AAI_SLP-REQ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| M/O |  | Attributes/Array of attributes | Size(bits) | Value/Note | Conditions |
| … |  | … | … | … | … |
| O | M | Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 | When (operation == 0b01 ¦¦ operation == 0b10) |
| O | M | Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 | When (operation == 0b01 ¦¦ operation == 0b10) |
| O | M | Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 | When (operation == 0b01 ¦¦ operation == 0b10) |
| … |  | … |  | … | … |
| O | O | New Initial Sleep Cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). Value: 0~31 New Initial Sleep Cycle = Value + 1 | When (operation == 0b01 ¦¦ operation == 0b10) && (NSCF == 0b10) |
| … |  | … |  | … | … |

[Modify the table 711 on the page 149, line 14 as follows:]

| M/O | Attributes/Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 | (mandatory) When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
| O | Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 | (mandatory) When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
| O | Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 | (mandatory) When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation == 0b01 ¦¦ Operation == 0b10) |
| … | … |  | … | … |
| O | New Initial Sleep Cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). Value: 0~31 New Initial Sleep Cycle = Value + 1 | (optional) When (Response_Code == 0b00 ¦¦ Response_Code == 0b01) && (Operation != = 0b01 ¦¦ Operation == 0b10) && (NSCF == 0b10) |
| … | … |  | … | … |

-------------------------------  Text End  ---------------------------------------------------