| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_RNG-REQ Message and ASN.1 Clean Up (16.2.3.1) |  |
| Date Submitted | 2010-07-12 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the ranging request message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_RNG-REQ Message and ASN.1 Clean Up (16.2.3.1)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_RNG-REQ message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

- Ranging Purpose Indication is represented using the ASN.1 CHOICE to avoid plenty of optional parameters and to save some encoding bits

- Clarification on the presence of CSGID list.

Proposed Text

Remedy 1:

In the P802.16m/D6, page 75, line 8, modify Table 679 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 679—AAI_RNG-REQ Parameters

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | AMSID* |  | 48 | It's the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry in the advanced network mode, without its STID/DID which the ABS/Paging Controller assigns. |
| O | MAC version |  | 8 | See 11.1.3 |  |
| O | AMS MAC address |  | 48 | AMS’s real MAC address | In the legacy network mode it shall be included when the AMS is attempting network entry, location update, network reentry or un coordinated handover. |
| O | Ranging Purpose Indication |  | 4 | 0b0000 = it indicates that the AMS is currently attempting HO reentry, or, in combination with a Paging Controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If this value is set and Paging Controller ID is absent, it indicates that the AMS is currently attempting HO reentry. If this value is set and Paging Controller ID is present, it indicates that the AMS is attempting network reentry from idle mode to the ABS. 0b0001 = it indicates that the AMS is initiating the idle mode location update process, or, in combination with CRID, it indicates that the AMS is initiating DCR mode extension. If this value is set and CRID is absent, it indicates that the AMS is initiating the idle mode location update process. If this value is set and CRID is present, it indicates that the AMS is initiating DCR mode extension. 0b0010 = ranging request for emergency call setup. When this value is set, it indicates AMS action of Emergency Call process. 0b0011 = it indicates that the AMS is attempting to perform location update due to a need to update service flow management encodings for E-MBS flows. 0b0100 = it indicates that the AMS is initiating location update for transition to DCR mode from idle mode. 0b0101 = it indicates that the AMS is currently attempting reentry from DCR mode, coverage loss or detection of the different ABS restart count. 0b0110 = it indicates that the AMS is currently attempting network reentry from a Legacy BS 0b0111 = it indicates that the AMS is currently experiencing “femto interference” 0b1000 - 0b1111 = reserved | It shall be included when the AMS is attempting to perform reentry, HO, location update or DCR mode extension. |
| O | Serving BSID |  | 48 | The BSID of the AMS's previous serving ABS before incurring a coverage loss, or the BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry. In case of performing Direct HO, this is the BSID of the previous serving Legacy BS. |
| O | CRID |  | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or It shall be included when the AMS detects the restart count in AAI_SCD message different from old one save in AMS. |
| O | STID |  | 12 | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform the uncoordinated HO reentry |
| O | Previous Basic CID |  | 16 | The Basic CID which the AMS used in the previous serving BS | It shall be included when the AMS is attempting to perform Direct HO reentry |
| O | Paging Controller ID |  | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. | To be included when the AMS is attempting to perform reentry from Idle Mode or location update. In the legacy network mode, DID shall not be included, and the ABS performs a mapping for paging parameters between AAI air interface and legacy network interface. |
| O | Deregistration Identifier (DID) |  | 12 | The ID which the AMS is assigned for idle mode and currently maintains. |  |
| O | PGID |  | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| O | Paging Cycle |  | 4 | PAGING_CYCLE applied to the AMS |  |
| O | Paging Offset |  | 6 | PAGING_OFFSET applied to the AMS |  |
| O | New Paging Cycle Change |  | 4 | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| O | Power Down Indicator |  | 1 | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS is attempting to perform location update due to power down |
| O | AK_COUNT |  | 16 | The AMS's current value of the AK_COUNT, which is used to generate the security keys in the target ABS. | It shall be included during reentry, secure Location Update or HO |
| O | AMS Mobility Information |  | 2 | 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved | It may be included when the AMS is attempting to perform the location update |
| O | CSGID(s) |  |  | One or more identifiers for one or more CSG(s) | Sent by AMS to aid quick CSG membership detection by Femto ABS as well as to aid ABS reselection. May be included when AMS is performing HO reentry or idle mode reentry. |
| O | CMAC Tuple | PMK SN | 4 | The sequence number of PMK used to derive the current AK. | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication. |
|  |  | CMAC_PN | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
|  |  | CMAC value | 64 | The CMAC value derived from the whole message. |  |
| O | List of FIDs, and Change Count of each FID |  |  | List of FIDs which the AMS currently has, and Change Count of each FID per Each FID SEQUENCE {fid INTEGER (0 .. 15),fidchangecount INTEGER (0 .. 15)} | Shall be included if Ranging Purpose Indication value is set to 0b0000 and Paging Controller ID is absent, |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, replace the text from page 861 line 20 to page 862 line 43 with the following ASN.1 code:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- Ranging Request

AAI-RNG-REQ ::= SEQUENCE {

amsidHash 			MACAddress 			OPTIONAL,

macProtocolVersion 	MACProtocolVersion 	OPTIONAL,

rangingPurpose 		RangingPurpose 		OPTIONAL,

servingBsid 		BsIdentifier 		OPTIONAL,

crid 				CRID 				OPTIONAL,

previousBasicCID 	CID 				OPTIONAL,

pagingControllerID 	PagingControllerID 	OPTIONAL,

deregistrationID 	DID 				OPTIONAL,

pagingGroupID 		PGID 				OPTIONAL,

pagingCycle 		PagingCycle 		OPTIONAL,

pagingOffset 		PagingOffset 		OPTIONAL,

pagingCycleChange 	PagingCycle 		OPTIONAL,

powerDownIndicator 	Supported 			OPTIONAL,

nonceAms 			Nonce 				OPTIONAL,

nonceAbs 			Nonce 				OPTIONAL,

akCount 			AKCount 			OPTIONAL,

amsMobility 		ENUMERATED {slow, medium, fast} 	OPTIONAL,

csgList 			SEQUENCE (SIZE(0..999)) OF CSGID,	--++++what is the correct upper bound?

cmac 				CMAC 				OPTIONAL,

changeCounts 		SEQUENCE (SIZE(0..999)) OF SEQUENCE {--++++what is the correct upper bound?

fid 				FID,

changeCount 		FIDChangeCount

},

...

}

MACAddress 			::= BIT STRING (SIZE(48))

MACAddressMask 		::= MACAddress

IPv4Address 		::= OCTET STRING (SIZE(4))

IPv6Address 		::= OCTET STRING (SIZE(16))

IPPortNumber 		::= INTEGER (0..65535)

MACProtocolVersion 	::= INTEGER (0..255)

RangingPurpose ::= ENUMERATED {

attemptingHOReentryOrNetworkReentry,

idleModeLocationUpdateOrDCRModeExtension,

emergencyCallSetup,

locationUpdateEMBSFlows,

locationUpdateTransitionToDCRMode,

attemptingReentryFromDCRModeOrCoverageLossOrBadRestartCount,

attemptingReentryFromLegacyBS,

...

}

CRID 				::= BIT STRING (SIZE(72))

STID 				::= BIT STRING (SIZE(12))

CID 				::= BIT STRING (SIZE(16))

PagingControllerID 	::= MACAddress

DID 				::= BIT STRING (SIZE(10))

PGID 				::= BIT STRING (SIZE(16))

PagingCycle 		::= INTEGER (0..15)

PagingOffset 		::= INTEGER (0..15)

Nonce 				::= BIT STRING (SIZE(64))

AKCount 			::= INTEGER (0..65535)

CSGID 				::= MACAddress

CMAC ::= SEQUENCE {

pmkSN PMKSN,

cmacPN CMACPN,

cmacValue CMACValue

}

PMKSN 				::= INTEGER (0..15)

CMACPN 				::= BIT STRING (SIZE(24))

CMACValue 			::= BIT STRING (SIZE(64))

FID 				::= INTEGER (0..15)

FIDChangeCount 		::= INTEGER (0..15)

Supported 			::= BOOLEAN

-------------------------------------------------------------------------------

-- AAI-RNG-REQ

-------------------------------------------------------------------------------

AAI-RNG-REQ ::= SEQUENCE {

msId				CHOICE {

msIdLegacy		MSIDLegacy,

msIdAdvanced	MSIDAdvanced

}									OPTIONAL,

rngPurpose			CHOICE {

rngPurpose0		RngPurpose0,

rngPurpose1		RngPurpose1,

rngPurpose2		RngPurpose2,

rngPurpose3		RngPurpose3,

rngPurpose4		RngPurpose4,

rngPurpose5		RngPurpose5,

rngPurpose6		RngPurpose6,

rngPurpose7		RngPurpose7

}									OPTIONAL

}

MSIDLegacy ::= SEQUENCE {

macAddr		AMSID

}

MSIDAdvanced ::= SEQUENCE {

amsIDHash	AMSIDHash,

macVer		MACVersion

}

-- 0b0000: HO reentry or idle mode reentry

RngPurpose0 ::= CHOICE {

hoReentry		RP-HOReentry,

idleReentry		RP-IdleReentry

}

RP-HOReentry	::= SEQUENCE {

servingBSID			ABSID,

stid				STID				OPTIONAL,

previousBasicCID	BasicCID			OPTIONAL,

akCount				AKCount,

fidInfo				FIDInfo,

csgIDList			SEQUENCE (SIZE (1..maxCSGIDs)) OF CSGID		OPTIONAL,

cmacTuple			CMACTuple

}

FIDInfo	::= SEQUENCE {

fid				INTEGER(0..15),

changeCount		INTEGER(0..15)

}

RP-IdleReentry	::= SEQUENCE {

pcid			PagingControllerID,

pgid			PagingGroupID,

pagingCycle		PagingCycle,

pagingOffset	PagingOffset,

did				DeregistrationID							OPTIONAL,

akCount			AKCount,

csgIDList		SEQUENCE (SIZE (1..maxCSGIDs)) OF CSGID		OPTIONAL,

cmacTuple		CMACTuple

}

-- 0b0001: idle mode location update or

--         DCR mode extension

RngPurpose1 ::= CHOICE {

locUpdate		RP-LocationUpdate,

dcrExtension	RP-DCRExtension

}

RP-LocationUpdate ::= SEQUENCE {

pcid				PagingControllerID,

pgid				PagingGroupID,

pagingCycle			PagingCycle,

pagingOffset		PagingOffset,

did					DeregistrationID		OPTIONAL,

newPagingCycle		PagingCycle				OPTIONAL,

powerDownIndicator	BOOLEAN					OPTIONAL,

akCount				AKCount					OPTIONAL,

mobilityInfo		MobilityInfo			OPTIONAL,

cmacTuple			CMACTuple				OPTIONAL

}

RP-DCRExtension	::= NULL

-- 0b0010: emergency call setup

RngPurpose2 ::= NULL

-- 0b0011: location update due to a need to update

-- service flow management encodings for E-MBS flows.

RngPurpose3 ::= SEQUENCE {

}

-- 0b0100: location update for transition to DCR mode from idle mode.

RngPurpose4 ::= NULL

-- 0b0101: network reentry from DCR mode, coverage loss

-- or detection of the different ABS restart count.

RngPurpose5 ::= SEQUENCE {

crid			CRID

}

-- 0b0110: network reentry from a Legacy BS

RngPurpose6 ::= SEQUENCE {

servingBSID		ABSID

}

-- 0b0111: experiencing ¡§femto interference¡¨

RngPurpose7 ::= NULL

-----------------------------------------------------  Text End  -----------------------------------------------------------------