| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_RNG-RSP Message and ASN.1 Clean Up (16.2.3.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the ranging response message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_RNG-RSP Message and ASN.1 Clean Up (16.2.3.2)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_RNG-RSP message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

Proposed Text

Remedy 1:

In the P802.16m/D6, page 79, line 59, modify Table 680 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 680—AAI_RNG-RSP Parameters

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | Ranging Abort |  | 1 | Set to 1 when an ABS rejects the AMS | Present when an ABS rejects an AMS |
| O | Ranging Abort Timer |  | 16 | Timer defined by an ABS to pro-hibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1-65535, In units of sec-onds | It shall be included when Ranging Abort is set to 1. |
| O | Temporary STID |  | 12 | Used for AMS identification until STID is assigned to the AMS during registration proce-dure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. STID is presented in the AAI_RNG-RSP mes-sage during uncon-trolled HO, NW reentry or Zone switching in case that the AAI_RNG-RSP is encrypted |
| O | AMSID*/MAC address |  | 48 | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent.In the legacy network mode, the actual MAC address of the AMS is used instead. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. |
| O | STID |  | 12 | AMS identification to be used in the target ABS | It shall be included in the AAI_RNG-RSP message during uncon-trolled HO, NW reentry or Zone switching in case that the AAI_RNG-RSP is encrypted |
| O | CRID |  | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are cur-rently maintained | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform net-work reentry/location update/zone-switch with assigning a new CRID or it shall be included when the AMS detects the restart count in AAI_SCD message dif-ferent from old one save in AMS. |
| O | Emergency Service FID |  | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | It shall be included when an ABS sends an AAI_RNG-RSP in response to an AAI_RNG-REQ with Ranging Purpose Indi-cation set to code 0b0010 when an AMS performs initial ranging. |
| O | Location Update Response |  | 4 | 0x00= Success of Location Update 0x01= Failure of Location Update0x02 = Reserved 0x03=Success of location update and DL traffic pending0x04 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x05 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x06~0xF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP mes-sage in response to an AAI_ RNG-REQ mes-sage used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| O | Paging Cycle |  | 4 | New Paging Cycle of the AMS 0b0000 - 0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed |
| O | Paging Group ID |  | 16 | New PGID of the AMS |  |
| O | Paging Offset |  | 6 | New PAGING_OFFSET of AMS |  |
| O | Paging Controller ID |  | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Suc-cess of Idle Mode Loca-tion Update) and Paging Controller ID has changed |
| O | Deregistration Identifier (DID) |  | 12 | The new DID which the AMS shall maintain in idle mode. |  |
| O | Paging carrier information |  | 6 | physical carrier index | It may be included only if the Location Update Response is set to 0x00(Success of Idle Mode Location Update) |
| O | HO Process Optimization |  | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control mes-sage may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentica-tion phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines. Identi-fies reentry process MAC con-trol messages that may be omitted during the current HO attempt due to the availability of MS service and operational con-text information obtained by means that are beyond the scope of this standard, and the MS ser-vice and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC control message responses as indicated in HO process opti-mization. | It shall be included when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reen-try process MAC con-trol messages that may be omitted during the current HO attempt |
| O | Activation Deadline |  | 6 | LSB bits of Superframe number after the AAI_RNG-RSP mes-sage to confirm theactivation of pre-assigned secondary carrier by sending the AAI_CM-IND message. | It shall be included when the target ABS has pre-assigned the secondary carriers through AAI_HO-CMD. |
| O | Neighbor station measurement report indicator |  | 1 | 1: Perform Neighbor station measurement report. | Identifies Neighbor sta-tion measurement report is required during cur-rent network entry for ARS |
| O | FID_update | SFID | 32 | SFID which need to update existing flow. | It shall be included if the ABS needs to update AMS's existing flows. FIDs which are not appeared in this field shall be regarded as guranteed by the ABS. |
|  |  | Update or delete | 1 | 0: delete the flow1: updated QoS Info exists |  |
|  |  | Updated QoS Info | Note: refer to DSA message | All therules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsu-lated in this field. |  |
| O | Unsolicited bandwidth grant indica-tor |  | 1 | 1 : an ABS should allocate UL bandwidth for transmission or retransmission of MAC mes-sages without request from AMS during network entry | It shall be included when AMS is attempt-ing network entry |
| O | CLC-INFO |  | Note: refer to CLC messages | The information of co-located coexistence response (Table 693) | It shall be included when the AMS is attempting network reentry after HO if the AMS has any Type I or II CLC class active before handover. The Information Type field of CLC-INFO shall be set to 2 to indicate it provides CLC response information. |
| O | Redirection Info |  | 48 | ABSID, preamble index and center frequency for one or more neighbor ABSs of the serving ABS | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry (due to various reasons such as high serving ABS load, non existence of CSG mem-bership etc) |
| O | Ranging Request bit |  | 1 | 1 : AMS shall send the AAI_RNG-CFM message after a successful periodic ranging | It shall be included when serving ABS per-forms coverage loss detection procedure (16.2.26) |
| O | Invalidity indication for FIDs |  | 1 | 0b0: FIDs information which was included in the AAI_RNG-REQ message are all valid0b1: Any mismatched FID infor-mation found | Shall be included when the AMS performs HO network reentry |
| O | List of invalid FIDs |  | List of FIDs which have invalidchange count | Shall be included if 'Invalidity indication for FIDs' is set to 1. ABS and AMS may exchange the AAI_DSx-mes-sages for invalid FIDs. |  |
| O | SAID_update_bitmap |  | 16 | Bitmap for indicating the spe-cific FID(s) that are being updated to SAID of AES-CTR. |  |
| O | Num of E-MBS Zone ID to update |  | 3 | Number of E-MBS ID to update in the target ABS | Presented only if the Location Update Response = 0x00 (Suc-cess of Idle Mode Loca-tion Update)and network re-entry for HO procedure if it needs to update. |
| O | fullNumE-MBSIdtoUpdateArr[1..8] |  |  |  |  |
| O | A) Service_flow_update_indicator |  | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included.0b0: "Service Flow Update Bit-map + New_E-MBS ID and FID"0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" | Present only if Num of E-MBS Zone ID to update > 0 |
| O | B) E-MBSZone ID |  | 7 | The E-MBS Zone ID to update | Present only if Num of E-MBS Zone ID to update > 0 |
| O | C) New E-MBS Zone ID |  | 7 | The new E-MBS Zone ID which the AMS shall maintain in any state on being served E-MBS. | Present only if the new E-MBS Zone is served in the target ABS. |
| O | D) Physical Carrier Index |  | 6 | The physical carrier index of the target ABS that supports the new E-MBS Zone. | Present only if the new E-MBS Zone ID is served on a different carrier from the previ-ous one at the target ABS. |
| O | E) Service Flow Update Bitmap |  | 16 | E-MBS Service Flow update bit-map where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. | Presented only if Service_flow_update_indicator is set to 0b0. |
| O | F) fullServiceFlowUpdateBitmap-Arr[0...16] |  |  |  |  |
| O | F.1) New_E-MBS ID and FID |  | Vari-able(16xN) | New E-MBS IDs and FIDs cor-responding to the Service Flow Update Bitmap where N is the number ones in the Service Flow Update Bitmap, which are used in the new E-MBS Zone sup-ported by the target ABS. | Presented only if Service_flow_update_indicator is set to 0b0. |
| O | G) Number of E-MBS ID_FID Map-pings (M) |  | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to update | Presented only if Service_flow_update_indicator is set to 0b1. |
| O | H) fullEMBSIdFidMappin-gArr[0...16] |  |  |  |  |
| O | H.1) Current_E-MBS ID and FID and New_E-MBS ID and FID |  | Vari-able(32xM) | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update. Based on the value of Number of E-MBS ID_FID Mappings | Presented only if Service_flow_update_indicator is set to 0b1. |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, replace the text from page 862 line 46 to page 864 line 40 with the following ASN.1 code:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- Ranging Response

AAI-RNG-RSP ::= SEQUENCE {

rangingStatus RangingStatus,

temporarySTID STID OPTIONAL,

amsidHash MACAddress OPTIONAL,

stid STID OPTIONAL,

crid CRID OPTIONAL,

locationUpdateResp LocationUpdateResponse OPTIONAL,

pagingCycle PagingCycle OPTIONAL,

pagingGroupID PGID OPTIONAL,

pagingOffset PagingOffset OPTIONAL,

pagingControllerID PagingControllerID OPTIONAL,

deregistrationID DID OPTIONAL,

pagingCarrier PhysicalCarrierIndex OPTIONAL,

hoProcessOptimization HOProcessOptimization OPTIONAL,

nbrMeasurementReport ENUMERATED {doNotPerform, perform} OPTIONAL,

nonceAMS Nonce OPTIONAL,

nonceABS Nonce OPTIONAL,

fidUpdateList SEQUENCE (SIZE(0..999)) OF FIDUpdate,

--++++what is the correct upper bound?

unsolicitedBwInd ENUMERATED {doNotAllocate, allocate} OPTIONAL,

clcInfo CLCInfo OPTIONAL,

redirectionInfo RedirectionInfo OPTIONAL,

--++++the table says "for one or more";

--++++ what does that mean?

rangingRequest ENUMERATED {

doNotSendAaiRngCfm,

sendAaiRngCfm

} OPTIONAL,

invalidFids SEQUENCE (SIZE(0..16)) OF FID,

--++++perhaps a bitmap would be better here?

saidUpdateBitmap 		BIT STRING (SIZE(0..16)) 	OPTIONAL,

embsZoneIdentifier 		MbsZoneIdentifier 			OPTIONAL,

carrier 				PhysicalCarrierIndex 		OPTIONAL,

flowUpdate CHOICE {

new SEQUENCE {

mbsServiceFlowUpdateBitmap BIT STRING (SIZE(1..16)),

--++++what is the correct upper bound?

mstidsAndFIDs NULL OPTIONAL

--++++what is the correct type definition?

},

update SEQUENCE {

mappingOfCurrentMSTIDAndFID NULL

--++++what is the correct type definition?

}

} OPTIONAL,

...

}

RangingStatus ::= ENUMERATED {success, abort, continue}

LocationUpdateResponse ::= ENUMERATED {

locationUpdateSuccessful,

locationUpdateFailed,

dlTrafficPending,

allowDCRModeInitiationOrExtension,

rejectDCRModeInitiationOrExtension,

...

}

PhysicalCarrierIndex ::= INTEGER (0..63)

LogicalCarrierIndex ::= INTEGER (0..7)

HOProcessOptimization ::= BIT STRING {

omitSbcMessages (0),

omitPkmAuthenticationPhase (1),

omitRegMessages (2),

omitFullStateTransfer (3)

} (SIZE(4))

FIDUpdate ::= SEQUENCE {

sfid SFID,

updateOrDelete ENUMERATED {	update, delete },

updatedQoSInfo QosInfo

}

SFID ::= BIT STRING (SIZE(32))

QosInfo ::= SEQUENCE {

--++++what is the correct type definition?

}

CLCInfo ::= SEQUENCE {

--++++what is the correct type definition?

}

RedirectionInfo ::= SEQUENCE {

bsid BsIdentifier

--++++what is the correct type definition?

}

MbsZoneIdentifier ::= INTEGER (0..127)

EmbsZoneIdentifier ::= MbsZoneIdentifier

-------------------------------------------------------------------------------

-- AAI-RNG-RSP

-------------------------------------------------------------------------------

AAI-RNG-RSP ::= SEQUENCE {

rangingStatus				RangingStatus,

stID						CHOICE {

temporarySTID	TemporarySTID,

stid			STID

}									OPTIONAL,

amsIDHash					AMSIDHash							OPTIONAL,

locationUpdateResponse		LocationUpdateResponse				OPTIONAL,

--pagingInfo					PagingInfo							OPTIONAL,

hoProcessOptimization		BIT STRING (SIZE (4))				OPTIONAL,

nbrMeasurementReport		BOOLEAN								OPTIONAL,

--zoneSwitchInfo				ZoneSwitchInfo						OPTIONAL,

redirectionInfoList			SEQUENCE (SIZE(1..maxRedirectInfos)) OF NeighborBSInfo	OPTIONAL,

unsolicitedBWIndicator		BOOLEAN								OPTIONAL,

--	clcInfo						CLCInfo								OPTIONAL,

crID						BIT STRING (SIZE (16))				OPTIONAL,

rangingRequestBit			BOOLEAN								OPTIONAL

}

LocationUpdateResponse ::= ENUMERATED {

success,

failed,

success-DL-traffic-pending,

dcr-mode-initiation-allowed

}

-----------------------------------------------------  Text End  -----------------------------------------------------------------