| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_RNG-ACK Message and ASN.1 Clean Up (16.2.3.3) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the ranging acknowledgement message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_RNG-ACK Message and ASN.1 Clean Up (16.2.3.3)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_RNG-ACK message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

- Unicast indicator is represented using ASN.1 CHOICE so that we don't need a single explicit bit for this purpose.

- The 3-bit Number of frame identifiers, which means one AAI_RNG-ACK responses to ranging opportunities carried by at most 8 frames, does not make any sense. Besides, The 4-bit RNG-ACK Bitmap indicates this message can indicate at most 4 ranging opportunities, which means 1 frame (if one frame carries at most maximum 4 ranging opportunities) or 2 frames (if one frame carriers at most maximum 2 ranging opportunities). However, there is very little gain to make AAI_RNG-ACK response to 2 frames. Therefore, we suggest to remove Number of frame identifiers and have AAI_RNG-ACK response to one frame only.

- Number of received codes is not needed because when the received code list is represented using ASN.1 SEQUENCE, the encoder will automatically generates a preceding integer to indicate the number of list elements.

- Clarification on the conditions of the presence of Ranging Preamble Code Index and Dedicated ranging preamble code indicator.

- Adjustment Parameter Indicator (API) is not needed because when the adjustment parameters are qualifies using ASN.1 OPTIONAL, the encoder will generate a preceding bitmap to indicate the presence/absence of each optional parameter.

Proposed Text

Remedy 1:

In the P802.16m/D6, page 85, line 4, modify Table 681 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 681—AAI_RNG-ACK Parameters

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Unicast indication |  | 1 | 0b0 : for broadcast 0b1: for unicast | N.A. |
| O | Number of frame identifiers (N_Frames) |  | 3 | The number of frame identifiers included in this RNG-ACK message. | It shall be included only when the Unicast indication is 0b0. |
| O | Frame identifier |  | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1. The 2 least significant bits of the superframe number 2. The frame index within the superframe (ranging from 0b00 to 0b11). | It shall be included only when the Unicast indication is 0b0. |
| O | RNG-ACK Bitmap |  | 4 | Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. | It shall be included only when Unicast indication =0b0 |
| O | Number of received codes |  | 6 | The number of ranging preamble code indices detected in this corresponding ranging opportunity. | It shall be included for each bit = 0b1 in the RNG-ACK Bitmap only when Unicast indication =0b0 |
| O | Ranging responses[0..32] | Ranging preamble Code Index | 6 | Ranging preamble code index received in this ranging opportunity. | It shall be included for each received ranging preamble code when Unicast indication = 0. Shall be included for the specific user when Unicast indication =1 |
|  | Ranging responses[0..32] | Dedicated ranging preamble code indicator | 1 | Used to indicate whether the received ranging preamble code is one of RP codes for the purpose of dedicated ranging or not.0b0 = RP code for the purpose of contention-based normal ranging0b1 = RP code for the purpose of dedicated ranging | It shall be included for each received ranging preamble code when Unicast indication = 0. Shall be included for the specific user when Unicast indication =1. |
|  | Ranging responses[0..32] | Ranging Status | 2 | Used to indicate whether ranging preamble code is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue | It shall be included for each received ranging preamble code index. Or it shall be included for the specific user when Unicast indication =1 |
|  | Ranging responses[0..32] | Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | It shall be included for each received ranging preamble code index when Ranging status is 0b00 or 0b10.Or it shall be included for the specific user when Unicast indication =0b1 and Ranging status is 0b00 or 0b10. |
|  | Ranging responses[0..32] | Timing offset adjustment | 5 | Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs | It shall be included for each received ranging preamble code index when API bit #0 = 0b1 Or it shall be included for the specific user when Unicast indication =0b1 and API bit #0 = 0b1 |
|  |  | Power level adjustment | 4 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the MS shall apply to its current transmission power for initial ranging. The bit size represents power level correction in the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. | It shall be included for the specific user when RNG-ACK bitmap=1 and number of received codes > 0 and and API bit #1 = 1and Ranging Status = 0b10. |
|  | Ranging responses[0..32] | Power level adjustment | 3 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the MS shall apply to its current transmission power for initial ranging. The bit size represents power level correction in the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #1 = 1 Or it shall be included for the specific user when Unicast indication =1 and API bit #1 = 1 |
|  | Ranging responses[0..32] | Frequency offset adjustment | 9 | Frequency offset adjustment Relative change in transmission frequency. The correction is 2% of the subcarrier spacing () multiplied by the 9-bit number interpreted as a signed integer | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #2 = 1 |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, replace the text from page 864 line 43 to page 865 line 28 with the following ASN.1 code:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- Aggregated CDMA Ranging Acknowledge

AAI-RNG-ACK ::= SEQUENCE {

unicastIndication ENUMERATED {unicast, broadcast},

frameIdentifiers SEQUENCE (SIZE(0..999)) OF	ShortFrameIdentifier, --++++what is the correct upper bound?

rangingCodes RangingCodeDetectionBitmap OPTIONAL,

rangingResponses SEQUENCE (SIZE(0..999)) OF RangingResponse, --++++what is the correct upper bound?

...

}

ShortFrameIdentifier ::= SEQUENCE {

superframe INTEGER (0..3),

frameIndex INTEGER (0..3)

}

RangingCodeDetectionBitmap ::= BIT STRING (SIZE(1..999)) --++++what is the correct upper bound?

RangingResponse ::= SEQUENCE {

rangingPreambleCodeIndex 		RangingPreambleCodeIndex,

rangingStatus 					RangingStatus,

adjustmentParameterIndication 	AdjustmentParameterIndication,

timingOffsetAdjustment 			TimingOffsetAdjustment

}

RangingPreambleCodeIndex ::= INTEGER (0..63)

AdjustmentParameterIndication ::= BIT STRING {

timeOffsetAdjustment (0), powerLevelAdjustment (1), frequencyOffsetAdjustment (2)

} (SIZE(3))

TimingOffsetAdjustment ::= INTEGER (0..31)

--

-- AAI-RNG-ACK

--

AAI-RNG-ACK ::= SEQUENCE {

unicastIndication		CHOICE {

broadcastRngAck		BroadcastRngAck,

unicastRngAck		ReceivedRngCodeInfo

}

}

BroadcastRngAck ::=	SEQUENCE {

frameIdentifier		FrameIdentifier,

rngAckBitmap			BIT STRING {

rngOpp1(0), rngOpp2(1), rngOpp3(2), rngOpp4(3)

},

rngOppsList			SEQUENCE (SIZE (1..maxRngOpps)) OF RangingOppStatus	OPTIONAL

}

maxRngOpps	INTEGER ::= 4

FrameIdentifier ::= SEQUENCE {

superFrameNumber		INTEGER (0..3),

frameIndex			INTEGER (0..3)

}

RangingOppStatus ::= SEQUENCE {

receivedCodesList	SEQUENCE (SIZE(1..maxReceivedCodes)) OF ReceivedRngCodeInfo

}

ReceivedRngCodeInfo ::= SEQUENCE {

rngCodeIndex				RangingCodeIndex,

dedicatedRngCodeIndicator	BOOLEAN,

rngStatus					CHOICE {

success		AdjustmentParameters,

abort		NULL,

continue		AdjustmentParameters

}

}

RangingCodeIndex	::= INTEGER(0..63)

RangingStatus 		::= ENUMERATED { success(0), abort(1), continue(2) }

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment		INTEGER (0..31)			OPTIONAL,

powerLevelAdjustment		INTEGER (0..7)			OPTIONAL,

frequencyOffsetAdjustment	INTEGER (0..511)		OPTIONAL

}

-----------------------------------------------------  Text End  -----------------------------------------------------------------