| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_HO-IND Message and ASN.1 Clean Up (16.2.3.9) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the handover indication message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_HO-IND Message and ASN.1 Clean Up (16.2.3.9)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_HO-IND message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

- Attribute HO Event Code is represented using ASN.1 CHOICE.

- SFH mismatch indication represented using ASN.1 BOOLEAN in HandoverCancel instead of being a standalone HO event code.

An analysis on the encoded bit string size using Unaligned PER to the proposed ASN.1 message format is also presented in the Appendix.

Proposed Text

Remedy 1:

In the P802.16m/D6, page 106, line 4, modify Table 689 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 689—AAI_HO-IND Parameters

| M/O | Attribute/Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | HO Event Code | 32 | Used to distinguish AAI_HO-IND among different scenarios 0b000: Target ABS selection in case of multiple candidate T-ABSs in HO-CMD. The serving ABS does not meet the "ABS unreachable" triggers. 0b001: All target ABSs in AAI_HO-CMD are unreachable. The AMS includes a new target ABS that was not included in AAI_HO-CMD. The serving ABS does not meet the "ABS unreachable" triggers. 0b010: The serving ABS meets the "ABS unreachable" triggers. The AMS will attempt network re-entry at the indicated T-ABS. 0b011: HO cancel due to SFH mismatch; the AMS does not have the latest SFH for any of the T-ABSs included in HO-CMD. The serving ABS does not meet the "ABS unreachable" triggers. 0b100: HO cancel due to reasons different than SFH mismatch (see description of code 0b011). The serving ABS does not meet the "ABS unreachable" triggers. | N/A |
| O | Target ABS ID | 48 | N/A | Shall be included when HO Event Code = 0b000 or 0b001 or 0b10 |
| O | Physical_Carrier_Index Target Physical Carrier Index | 6 | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be included when the target ABS is a multi-carrier ABS and HO Event Code = 0b000, 0b01 or 0b10 |
| O | Physical carrier index at the serving ABS Serving Physical Carrier Index | 6 | Recommended carrier index of the AMS to be used for network reentry to the target ABS | May be included when AMS's multicarrier capability = 0b10, HO Reentry Mode = 1 and the target carrier is different from the serving carrier Presented only when HO Event Code = 0b00 , 0b01 or 0b10 |
| O | SFH mismatch indication | 1 | 1: the AMS does not have the latest SFH for any of the T-ABSs included in the AAI_HO-CMD message; 0: otherwise | Shall be included when HO Event Code = 0b11 |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, page 865, line 29, insert the following ASN.1 code into Annex P.2 MAC Control Message Definitions:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AAI-HO-IND ::= SEQUENCE {

hoEventCode	CHOICE {

targetABSSelection		TargetABSSelection,		-- 0b00

targetABSUnreachable	TargetABSSelection,		-- 0b01

servingABSUnreachable	TargetABSSelection,		-- 0b10

handoverCancel			HandoverCancel			-- 0b11

}

}

TargetABSSelection ::= SEQUENCE {

targetABSID			INTEGER (0..7),

targetPhyCarrierID		PhysicalCarrierIndex		OPTIONAL,

servingPhyCarrierID		PhysicalCarrierIndex		OPTIONAL

}

HandoverCancel ::= SEQUENCE {

sfhMismatch			BOOLEAN					OPTIONAL

}

PhysicalCarrierIndex 	::= INTEGER (0..63)

-----------------------------------------------------  Text End  -----------------------------------------------------------------

Appendix. Analysis on the Encoded Bit String Size

Maximum size: 3 bytes (Unaligned PER)

value MAC-Control-Message ::=

{

--TYPE INFORMATION: SEQUENCE

--LEVEL NUMBER: 1

--OFFSET: 0,0

message --choice value:

--TYPE INFORMATION: CHOICE

--LEVEL NUMBER: 1.1

--OFFSET: 0,0

--choice index: <.0100> (index = 4)

aaiHoInd :

{

--TYPE INFORMATION: SEQUENCE

--LEVEL NUMBER: 1.1.1

--OFFSET: 0,4

hoEventCode --choice value:

--TYPE INFORMATION: CHOICE

--LEVEL NUMBER: 1.1.1.1

--OFFSET: 0,4

--choice index: <00> (index = 0)

targetABSSelection :

{

--TYPE INFORMATION: SEQUENCE

--LEVEL NUMBER: 1.1.1.1.1

--OFFSET: 0,6

--preamble: <11>

--bit #0 = 1: 'targetPhyCarrierID' is present

--bit #1 = 1: 'servingPhyCarrierID' is present

targetABSID 7,

--TYPE INFORMATION: INTEGER (0..7)

--LEVEL NUMBER: 1.1.1.1.1.1

--OFFSET: 1,0; LENGTH: 0,3

--contents: <.111>

targetPhyCarrierID 63,

--TYPE INFORMATION: INTEGER (0..63) OPTIONAL

--LEVEL NUMBER: 1.1.1.1.1.2

--OFFSET: 1,3; LENGTH: 0,6

--contents: <11111.1>

servingPhyCarrierID 63

--TYPE INFORMATION: INTEGER (0..63) OPTIONAL

--LEVEL NUMBER: 1.1.1.1.1.3

--OFFSET: 2,1; LENGTH: 0,6

--contents: <111111>

}

}

}

--PDU padding: <0>

--TOTAL LENGTH: 3,0

Minimum size: 1 bytes (Unaligned PER)

value MAC-Control-Message ::=

{

--TYPE INFORMATION: SEQUENCE

--LEVEL NUMBER: 1

--OFFSET: 0,0

message --choice value:

--TYPE INFORMATION: CHOICE

--LEVEL NUMBER: 1.1

--OFFSET: 0,0

--choice index: <.0100> (index = 4)

aaiHoInd :

{

--TYPE INFORMATION: SEQUENCE

--LEVEL NUMBER: 1.1.1

--OFFSET: 0,4

hoEventCode --choice value:

--TYPE INFORMATION: CHOICE

--LEVEL NUMBER: 1.1.1.1

--OFFSET: 0,4

--choice index: <11> (index = 3)

handoverCancel :

{

--TYPE INFORMATION: SEQUENCE

--LEVEL NUMBER: 1.1.1.1.1

--OFFSET: 0,6

--preamble: <0>

--bit #0 = 0: 'sfhMismatch' is absent

}

}

}

--PDU padding: <0>

--TOTAL LENGTH: 1,0