| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_HO-REQ Message and ASN.1 Clean Up (16.2.3.10) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the handover request message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_HO-REQ Message and ASN.1 Clean Up (16.2.3.10)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_HO-REQ message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

- Report Metric is removed because PER will generate bits to indicate the existence of either CINR mean or RSSI mean or both.

- N_New_ABS_Index and N_New_ABS_Full are removed because when the neighbor ABS list is represented using ASN.1 SEQUENCE, the number of neighbor ABSs will be generated by the PER.

- Relative delay is removed to be consistent with the agreement in Session#67.

Proposed Text

Remedy 1:

In the P802.16m/D6, page 107, line 4, modify Table 690 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 690—AAI_HO-REQ Parameters

| M/O | Attribute/Array of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Report Metric | 8 | Bitmap indicator of trigger metrics that the AMS reports in this message Bit #0: BS CINR mean Bit #1: BS RSSI mean Bits #2-7: Reserved; shall be set to zero | N.A. |
| M | AAI_NBR-ADV Change count | 8 | AAI_NBR-ADV change count last received from the serving ABS | N.A. |
| M | N_New_ABS_Index | 8 | Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. | N.A. |
| O | N_New_ABS_Full | 48 | The number of neighboring ABSs to be are identified by full considered for handover, which is 48 bit ABSID |  |
| O | Neighbor_ABS_Index | 8 | ABS index corresponds to AAI_NBR-ADV. | Shall be included when N_New_ABS_Index is not 0. for each listed ABS which is presented in the AAI_NBR-ADV message. |
| O | Neighbor_ABS_ID | 48 | The 48 bits neighbor ABSID | Shall be included when N_New_BS_Full is not 0. for each listed ABS which is absent in the AAI_NBR-ADV message. |
| O | ABS CINR mean | 8 | CINR mean of the new ABS | ShallMay be included for each listed ABS. if Report Metric Bit #0 is set to 0 |
| O | ABS RSSI mean | 8 | RSSI mean of the new ABS | ShallMay be included for each listed ABS. if Report Metric Bit #1 is set to 1 |
| O | Relative delay | 8 | - | Shall be included if Report Metric Bit #2 is set to 1 |
| O | Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when AMS supports MC mode=0b010 or 0b011 or 0b100 |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, page 865, line 29, insert the following ASN.1 code into Annex P.2 MAC Control Message Definitions:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

--

-- AAI-HO-REQ

--

AAI-HO-REQ ::= SEQUENCE {

nbrAdvChangeCount				INTEGER(0..255),

newABSList					NewABSList			OPTIONAL,

carrierPreassignmentIndicator		BOOLEAN				OPTIONAL

}

NewABSInfo ::= SEQUENCE {

newABSIndex	CHOICE {

neighborABSID	NeighborABSIndex,

fullABSID		ABSID

},

cinrMean		CINRMean			OPTIONAL,

rssiMean		RSSIMean			OPTIONAL

}

NewABSList		::= SEQUENCE (SIZE(1..maxNewABSIndices)) OF NewABSInfo

NeighborABSIndex	::= INTEGER(0..255)

CINRMean			::= INTEGER(0..255)

RSSIMean			::= INTEGER(0..255)

ABSID 			::= BIT STRING (SIZE (48))

maxNewABSIndices	INTEGER ::= 64

-----------------------------------------------------  Text End  -----------------------------------------------------------------