| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_HO-CMD Message and ASN.1 Clean Up (16.2.3.11) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the handover command message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_HO-CMD Message and ASN.1 Clean Up (16.2.3.11)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_HO-CMD message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

- HO Reentry Mode is represented using the ASN.1 CHOICE to save some bits for the encoded bit string.

- CDMA_RNG_FLAG is represented using the ASN.1 CHOICE to save some bits for the encoded bit string.

- Dedicated Ranging Code Flag is not needed because the ASN.1 encoder will automatically generate bits to indicate the presence/absence of the dedicated CDMA ranging code.

- Carrier Preassignment Indication is not needed because PER will automatically generate bits to indicate the presence/absence of the pre-assigned secondary carrier information.

- N_Preassigned_Carriers is not needed because when the list of pre-assigned carrier IDs is represented using ASN.1 SEQUENCE, the encoder will automatically generate the number of list elements.

- Proposed the following clarifications:

  - HO Reentry Interleaving Interval and HO Reentry Iteration apply only to Mode 0b00 (HO command).

  - Ranging opportunity index should be a target ABS-specific parameter.

  - Pre-allocated Basic CID applies to handover (Mode=0b00) to a legacy BS or LZone of a mix-mode ABS only, i.e. does not apply to M-to-LZone switching within the same ABS, and should be a target BS-specific parameter.

Proposed Text

Remedy 1:

In the P802.16m/D6, page 108, line 4, modify Table 691 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 691—AAI_HO-CMD Parameters

| ﻿M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. | N.A. |
| O | HO Reentry Mode | 1 | 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). | Shall be included when Mode = 0b00 or 0b10 Shall be included when Mode = 0b01. Shall not be set to 1 if AMS does not support EBB (according to AAI_REG-REQ). |
| O | HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when Mode = 0b00 and HO Reentry Mode is set to 1 |
| O | HO Reentry Iteration | 3 | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when Mode = 0b00 and HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Shall be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 | The time when the serving BS discards MS's context. | Shall be included when Mode = 0b00 or 0b01. |
| O | targetBSID | 48 | - | Shall be included for each target ABS when Mode=0b00 |
| O | Preamble Index | 10 |  | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 |  | Shall be included for each target ABS when Mode=0b00 |
| O | Action Time | 8 | Mode=0b00 Action Time included in this message is the absolute frame number at the serving ABS. When CDMA_RNG_FLAG is set to 1, it shall be set to the frame where either a normal or dynamic ranging channel is present. Mode=0b01 This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. If CDMA_RNG_FLAG is set to 0, pre-assigned STID at the target BS shall be assigned. | Shall be included for each target ABS when Mode=0b00 |
| O | offsetData | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | Presents when CDMA_RNG_FLAG=0 |
| O | offsetControl | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | Presents when CDMA_RNG_FLAG=0 |
| O | Dedicated Ranging Code Flag | 1 | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | May be included for each target ABS when CDMA_RNG_FLAG=1 Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity. | Shall be included for each target ABS when Action Time refers to a frame where a dynamic ranging channel is allocated |
| ﻿ ﻿O | Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 6 | Physical carrier index of the recommended Target ABS | Shall be included for each target ABS when Mode = 0b00 and target ABS is not included in AAI_NBR-ADV message or is multi-carrier ABS |
| O | Channel bandwidth | 8 | Channel bandwidth of recommended target ABS in units of 125 KHz. | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | CP length | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | Carrier Preassignment Indication | 1 | 0: no pre-assigned secondary carrier 1: information of pre-assigned secondary carriers by the target ABS is included in this message; | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 3 | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| ﻿O | Pre-assigned secondary carrier information |  | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preambile Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre-allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target legacy BS. | May be included for each legacy target BS when HO (Mode = 0b00) to a LZone or a legacy BS |
| O | SFH delta information | Variable | Delta encoding with reference to the serving ABS. | May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |
| O | S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information. | Shall be included for each target ABS when SFH delta information is included |
| O | Physical carrier index at the serving ABS | 6 | Recommended carrier index of the AMS to be used for network reentry to the target ABS | May be included when AMS's multicarrier capability= 0b10 , HO Reentry Mode = 1 and the target carrier is different from the serving carrier |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, page 865, line 29, insert the following ASN.1 code into Annex P.2 MAC Control Message Definitions:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

--

-- AAI-HO-CMD

--

AAI-HO-CMD ::= SEQUENCE {

mode		CHOICE {

hoCmd 		HandoverCommand, 	-- 0b00

zsCmd		ZoneSwitchCommand, 	-- 0b01

hoReject		NULL				-- 0b10

}

}

HandoverCommand ::= SEQUENCE {

hoReentryMode					CHOICE {

bbeHO	NULL,

ebbHO	EBBHOInfo

},

disconnectTimeOffset			INTEGER(0..255),

resourceRetainTime				INTEGER(0..255),

targetBSList					SEQUENCE (SIZE(1..maxNewABSIndices)) OF TargetBSInfo

}

ZoneSwitchCommand ::= SEQUENCE {

zoneSwitchMode					BOOLEAN,

resourceRetainTime				INTEGER(0..255),

actionTime					INTEGER(0..3),

preambleIndexLZone				PreambleIndexLZone

}

EBBHOInfo ::= SEQUENCE {

hoReentryInterleavingInterval	INTEGER(0..255),

hoReentryIteration				INTEGER(0..255)			OPTIONAL,

servingPhyCarrierID				PhysicalCarrierIndex		OPTIONAL

}

TargetBSInfo ::= SEQUENCE {

targetBSID				ABSID,

preambleIndex				PreambleIndex,

faIndex					INTEGER(0..255),

actionTime				INTEGER(0..255),

cdmaRngFlag				CHOICE {

offsetInfo		OffsetInfo,		-- skip CDMA ranging

dedicatedRngInfo	DedicatedRngInfo	-- perform CDMA ranging

},

serviceLevelPrediction		ServiceLevelPrediction,

rngOpportunityIndex			INTEGER(0..7)					OPTIONAL,

targetPhyCarrierID			PhysicalCarrierIndex			OPTIONAL,

channelBandwidth			INTEGER(0..255)				OPTIONAL,

cpLength					CPLength						OPTIONAL,

sfhDeltaInfo				SFHDeltaInfo					OPTIONAL,

preassignedCarrierInfo		PreassignedCarrierInfo			OPTIONAL,

preallocatedBasicCID		BasicCID						OPTIONAL

}

BasicCID 			::= INTEGER(0..65535)

CPLength 			::= ENUMERATED { one-eighth(0), one-sixteenth(1), one-fourth(2) }

PreambleIndex 		::= INTEGER(0..1023)

PreambleIndexLZone	::= INTEGER(0..65535)

OffsetInfo ::= SEQUENCE {

offsetData		INTEGER(0..127),

offsetControl		INTEGER(0..127)

}

DedicatedRngInfo ::= SEQUENCE {

dedicatedCDMARngCode	CDMARngCodeIndex		OPTIONAL

}

CDMARngCodeIndex	::= INTEGER(0..31)

SFHDeltaInfo ::= SEQUENCE {

s-SFHChangeCount	INTEGER(0..15)

-- SFH delta information to be included here

}

ServiceLevelPrediction ::= ENUMERATED {

noServiceAvailable(0),

partiallyServiceAvailable(1),

allServiceAvailable(2),

noServiceLevelPrediction(3)

}

PreassignedCarrierInfo ::= SEQUENCE {

carrierStatusBitmap		BIT STRING (SIZE(maxPreassignedCarriers)),

physicalCarrierIndexList	SEQUENCE (SIZE(1..maxPreassignedCarriers)) OF PhysicalCarrierIndex

}

-----------------------------------------------------  Text End  -----------------------------------------------------------------