| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_NBR-ADV Message and ASN.1 Clean Up (16.2.3.12) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D6) |  |
| Abstract | Text proposal on the neighbor advertisement message and its ASN.1 format |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_NBR-ADV Message and ASN.1 Clean Up (16.2.3.12)

Kelvin Chou, I-Kang Fu, Yih-Shen Chen, Alex Hsu and Paul Cheng

MediaTek Inc.

Introduction

This contribution proposes the some clarifications and the ASN.1 format for the AAI_NBR-ADV message. In order to minimize the number of PER encoded bits, the following modifications are proposed:

- BS number M is not needed because when the BS information list is represented using ASN.1 SEQUENCE, the encoder automatically generates a preceding integer to indicate the number of list elements.

- Number of Carriers is not needed because when the list of physical carrier indices is represented using ASN.1 SEQUENCE, the encoder automatically generates a preceding integer to indicate the number of list elements.

- Control Bitmap is not needed because the ASN.1 encoder will automatically generate the bitmap for the same purpose.

- Clarifications on the SFH subpacket information.

Proposed Text

Remedy 1:

In the P802.16m/D6, page 114, line 31, modify Table 692 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 692—AAI_NBR-ADV Parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Count | 3 | NBR-ADV Change Count | N.A. |
| M | Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101-0b111: reserved | N.A. |
| M | Total Number of AAI_NBR-ADV Segments | 4 | Total number of segments of AAI_NBR-ADV for this cell type | N.A. |
| M | AAI_NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type | N.A. |
| M | BS number M | 8 | Total number of BSs to be included in this AAI_NBR-ADV segment | N.A. |
| M | Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index. | N.A. |
| O | List of PHY carrier indices | 6x64 (maxi-mum 64 phy indices) | Refer to the physical carrier index in AAI_Global-Config message | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range start | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range end | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| M | BSID | 48 | - | Shall be included for each neighbor BS |
| M | MAC protocol versions | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 10 defined for 16m. | Shall be included for each neighbor BS |
| M | CP time | 2 | CP time of the BS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each neighbor BS |
| M | Number of carriers | 3 | Number of carriers of the BS | Shall be included for each neighbor BS |
| M | SA-PREAMBLE index | 10 | - | Shall be included for each carrier of each neighbor BS |
| M | A-PREAMBLE transmit power | 8 | - | Shall be included for each carrier of each neighbor BS |
| M | Physical carrier index | 6 | Refer to the physical carrier index in AAI_Global-Config message | Shall be included for each carrier of each neighbor BS |
| M | S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS | Shall be included for each carrier of each neighbor BS |
| M | SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier which transmits this AAI_NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macro cell ABS, the SFH_encoding_format shall be either 0b00 or 0b01 or 0b11. | Shall be included for each neighbor BS |
| O | Control_bitmap | 4 | Each bit maps to one SFH subpacket Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: reserved | Shall be included for each neighbor ABS when SFH_encoding_format is 0b00. |
| O | SFH Subpkt 1 | Variable depending on FFT size | - | Shall May be included for each carrier of each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #0 is set to 1 |
| O | SFH Subpkt 2 | Variable depending on FFT size | - | Shall May be included for each carrier of each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #1 is set to 1 |
| O | SFH Subpkt 3 | variable | - | Shall May be included for each carrier of each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #2 is set to 1 |
| O | Delta information | Var. | Delta encoding, w.r.t. the reference carrier the current carrier transmitting this message if SFH_encoding format = 01, the preceding carrier if SFH_encoding_format=11) | Shall be included for each carrier of each neighbor BS if SFH_encoding_format is set to 0b01 or 0b11 |
| O | Neighbor-specific trigger | Var. | Optional neighbor-specific triggers with encoding defined in Table764774- Trigger description | May be included for each neighbor BS |
| O | NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | NBR-ADV interval: | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV. |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D6, page 865, line 29, insert the following ASN.1 code into Annex P.2 MAC Control Message Definitions:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-------------------------------------------------------------------------------

-- AAI-NBR-ADV

-------------------------------------------------------------------------------

AAI-NBR-ADV ::= SEQUENCE {

changeCount				INTEGER (0..7),

cellType					ENUMERATED {macro, micro, macro-hotzone, femto, relay},

totalNumberOfSegments		INTEGER (0..15),

segmentIndex				INTEGER (0..15),

startingABSIndex			INTEGER (0..255),

-- For ABS type whose system info are not included in AAI_NBR-ADV

cellTypeInfo				CellTypeInfo		OPTIONAL,

neighborBSInfoList			SEQUENCE (SIZE(1..maxNeighborBSs)) OF NeighborBSInfo

}

CellTypeInfo ::= SEQUENCE {

phyCarrierIDList	SEQUENCE(SIZE(1..maxPhyIndices)) OF PhysicalCarrierIndex,

startIDCell		IDCell,

endIDCell			IDCell

}

NeighborBSInfo ::= SEQUENCE {

bsID					ABSID,

macVersion			MACVersion,

cpLength				CPLength,

carrierInfoList		SEQUENCE (SIZE(1..maxCarriers)) OF CarrierInfo,

specificTrigger		Triggers							OPTIONAL,

nbrAdvOffset			INTEGER(0..15)						OPTIONAL,

nbrAdvInterval			INTEGER(0..15)						OPTIONAL

}

CarrierInfo ::= SEQUENCE {

saPreambleIndex		PreambleIndex,

aPreambleTxPower		INTEGER(0..255),

physicalCarrierIndex	PhysicalCarrierIndex,

sfhChangeCount 			INTEGER (0..15),

sfhEncFmt 				CHOICE {

fullSubpkt			SFHSubpacket,

deltaInfoCurrentCxr		DeltaInfoCurrentCarrier,

noSFHIncluded			NULL,

deltaInfoPrecedingCxr	DeltaInfoPrecedingCarrier

}

}

Triggers ::= SEQUENCE {

-- Optional neighbor-specific triggers with encoding defined in Table774 Trigger description

-- Exact content TBD

}

ABSID 				::= BIT STRING (SIZE (48))

MACVersion 			::= INTEGER (0..255)

CPLength 				::= ENUMERATED { one-eighth(0), one-sixteenth(1), one-fourth(2) }

PreambleIndex 			::= INTEGER(0..1023)

PhysicalCarrierIndex 	::= INTEGER (0..63)

SFHSubpacket	::= SEQUENCE {

sfhSubpkt1		SFHSubPacket1	OPTIONAL,

sfhSubpkt2		SFHSubPacket2	OPTIONAL,

sfhSubpkt3		SFHSubPacket3	OPTIONAL,

}

SFHSubPacket1	::= SEQUENCE {

-- content TBD

}

SFHSubPacket2	::= SEQUENCE {

-- content TBD

}

SFHSubPacket3	::= SEQUENCE {

-- content TBD

}

DeltaInfoCurrentCarrier	::= SEQUENCE {

-- content TBD

}

DeltaInfoPrecedingCarrier ::= SEQUENCE {

-- content TBD

}

-----------------------------------------------------  Text End  -----------------------------------------------------------------