| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MC-REQ/RSP Message and ASN.1 Clean Up (16.2.3.52) |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D6 |  |
| Abstract | The existing ASN.1 code needs further clean up |  |
| Purpose | For TGm discussion and adoption into P802.16m/D7 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MC-REQ/RSP Message and ASN.1 Clean Up

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#55, page#214 of P802.16m/D6]

16.2.3.52 AAI_MC-REQ (multicarrier Request) Message

The Multi-Carrier Request Message (AAI_MC-REQ) is sent by an AMS to an ABS to request the list of Assigned Carriers. The AMS cannot send the AAI_MC-REQ message until it receives the AAI_Global-Config message and AAI_MC-ADV message from its serving ABS. According to the multi-carrier configuration supported by serving ABS, which is indicated in AAI_MC-ADV, the AMS shall determine the subset of carriers which it can simultaneously support under its hardware capability. Then AMS will send AAI_MC-REQ message to the serving ABS and include sets of physical carrier index to inform ABS this information.

Table 755—AAI_MC-REQ message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| Global_Support | 1 | Indicates whether the AMS can simultaneously process all the carriers supported in by serving ABS simultaneously 0b0: The AMS can only process a subset of the carriers supported in by serving ABS 0b1: The AMS can simultaneously process all carriers supported in by serving ABS |
| If (Global_Support==0){ |  |  |
| Number of Candidate Combinations (N) | 42 |  |
| For (i=0, i<N; i++) { |  |  |
| Number of Candidate Assigned Carrier (Nc) | 3 |  |
| For (j=0, j<Nc; j++) { |  |  |
| Physical Carrier Index | 64 | The carriers AMS can simultaneously support process |
| } |  |  |
| DL/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. |
| } |  |  |
| } |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0b0 = not supported 0b1=support |

Table 756—Contents of AAI_MC-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Support | 1 | Indicates whether the AMS can simultaneously process all the carriers supported in by serving ABS simultaneously 0b0: The AMS can only process a subset of the carriers supported in by serving ABS 0b1: The AMS can simultaneously process all carriers supported in by serving ABS |  |
| O | Number of candidate combinations | 42 | Indicates the number of carrier combination AMS can support simultaneously process | Shall be present when Global Support is zero |
| O | Number of candidate assigned carrier | 3 | Indicates the number of carriers AMS can support simultaneously process | Shall be present when the number of candidate combinations is not zero |
| O | Physical carrier index | 64 | Indicates the carriers AMS can simultaneously support process | Shall be present when the number of candidate assigned carrier is not zero |
| O | DU/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. | Shall be present when the number of candidate combinations is not zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |

16.2.3.53 AAI_MC-RSP (multicarrier Response) Message

Based on information provided by the AMS in the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message:

Table 757—AAI_MC-RSP message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| Global_Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |
| If (Global_Assign == 0) { |  |  |
| Number of Assigned Carriers (N) | 3 | Number of Assigned Secondary Carriers. |
| For (i=0; i<N; i++) { |  |  |
| Physical carrier index | 64 | The index refers to a Physical carrier index in AAI_MC-ADV message. Logical carrier index is assigned implicitly in the order of assigned physical carrier index |
| DL/UL indicator | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b1: The carrier is assigned for DL only. |
| Support of data transmission over guard sub-carrier | 1 | 0b0 = not supported 0b1 = support |
| } |  |  |
| } |  |  |

Table 758—Contents of AAI_MC-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers | 3 | Indicates the number of carriers to be assigned to the AMS | Shall be present when the Global Assign is zero |
| O | Physical carrier index | 64 | Indicates the carriers to be assigned to the AMS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index | Shall be present when the Global Assign is zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |
| O | DU/UL indic | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b1: The carrier is assigned for DL only. | Shall be present when the Global Assign is zero |

The AAI_MC-RSP (multicarrier Response) Message is typically sent to the AMS in response to the AAI_MC-REQ message, but it may also be sent by the ABS to an AMS to update the list of assigned carriers in unsolicited manner.

[Adopt the following text modifications from line#11, page#308 of P802.16m/D6]

The AAI_MC-RSP (multicarrier Response) Message is typically sent to the AMS in response to the AAI_MC-REQ message, but it may also be sent by the ABS to an AMS to update the list of assigned carriers in unsolicited manner.

[Adopt the following text modifications from line#36, page#896 of P802.16m/D6]

-- AAI_MC-REQ message

-- Multicarrier Request Message

AAI-MC-REQ ::= SEQUENCE {

}

-- AAI_MC-RSP message

-- Multicarrier Response Message

AAI-MC-RSP ::= SEQUENCE {

}

---------------------------------------------------------Start of the Text-----------------------------------------------------------