| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_CM-CMD/IND Message and ASN.1 Clean Up (16.2.3.54) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D6 |  |
| Abstract | The existing ASN.1 code needs further clean up |  |
| Purpose | For TGm discussion and adoption into P802.16m/D7 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_CM-CMD/IND Message and ASN.1 Clean Up

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#1, page#218 of P802.16m/D6]

16.2.3.54 Carrier Management Command (AAI_CM-CMD) MAC control message

An AAI_CM-CMD message shall be transmitted by an ABS to initiate a carrier management procedure when ABS and AMS both supports , such as carrier activation/deactivation or and primary carrier change. In order to respond to a received AAI_CM-CMD MAC control message, the AMS shall transmit the AAI_CM-IND MAC control message.

The format of the AAI_CM-CMD MAC control message is shown in Table 759756.

Table 759—Contents of AAI_CM-CMD message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | Indicate the purpose of this message 0b0: secondary carrier management 0b1: primary carrier change | NA |
| O | Indication Type | 2 | Indicate the corresponding secondary carrier is activated or deactivated 00: no action 01: deactivation only 10: activation only 11: both activation and deactivation | Shall be present when Action code is set to 0b0 in AAI_CM-CMD message |
| O | Activation Deadline | 6 | LSB bits of Superframe number after the AAI_CM-CMD is sent for the AMS to confirm the activation of secondary carrier by sending the AAI_CM-IND message | Shall be present when Indication Type is set to 10 or 11 in AAI_CM-CMD message |
| O | Num of target carrier | 3 | Indicate the number of secondary carrier(s) to be activated or deactivated 1..8 | Shall be present when Indication Type is set to 01 or 10 or 11 in AAI_CM-CMD message |
| O | Target carrier index | 3 | Indicate the secondary carrier index to be activated or deactivated 1..8 | Shall be present when Indication Type is set to 01 or 10 or 11 in AAI_CM-CMD message |
| O | Activation of DL/UL | 1 | Indicate the activation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are activated 0b1: DL of the secondary carrier is activated but UL of the secondary carrier is not activated | Shall be present when Indication Type is set to 10 or 11 in AAI_CM-CMD message |
| O | Ranging indicator | 1 | Indicate the periodic ranging is required for the carrier 0b0: no initial or periodic ranging is required for the carrier 0b1: periodic ranging is required for the carrier | Shall be present when Indication Type is set to 10 or 11 in AAI_CM-CMD message or shall be present when Action code is set to 0b1 in AAI_CM-CMD message |
| O | Deactivation of DL/UL | 1 | Indicate the deactivation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are deactivated 0b1: UL of the secondary carrier is deactivated but DL of the secondary carrier is kept active | Shall be present when Indication Type is set to 01 or 11 in AAI_CM-CMD message |
| O | Physical carrier index of Target carrier | 63 | Indicate the physical carrier index of a target carrier for primary carrier change. If the AMS supports multicarrier operation, the carrier shall be one of the assigned carriers 0..763 | Shall be present when Action code is set to 0b1 in AAI_CM-CMD message |
| O | Action Time | 3 | LSB bits of Superframe number at the time to switch to the target carrier The value shall be set to the value more than the Message ACK timer for AAI_CM-CMD message 0..7: LSB of superframe | Shall be present when Action code is set to 0b1 in AAI_CM-CMD message |
| O | Next state of serving primary carrier | 1 | Indicate the deactivation of serving primary carrier after primary carrier change. 0b0: serving primary carrier will be deactivated after primary carrier change. If the AMS does not support carrier aggregation, this field shall be always set to 0b0. 0b1: serving primary carrier is kept active after primary carrier change | Shall be present when Action code is set to 0b1 in AAI_CM-CMD message |

16.2.3.55 Carrier Management Indication (AAI_CM-IND) MAC control message

An AAI_CM-IND message shall be transmitted by an AMS to inform the ABS of the readiness of target carrier when the AMS is instructed to newly activate the target carrier(s) in an AAI_CM-CMD message with action code=0 or when the target carrier in an AAI_CM-CMD message with action code=1 has not been an active carrier.

The format of the AAI_CM-IND MAC control message is shown in Table 760757.

Table 760—Contents of AAI_CM-IND message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | Indicate the purpose of this message Purpose Indication 0b0: secondary carrier management 0b1: primary carrier change | NA |

[Adopt the following text modifications from line#65, page#894 of P802.16m/D6]

-- AAI_CM-CMD message

-- Carrier Management Command Message

AAI-CM-CMD ::= SEQUENCE {

messageType 		OCTET STRING(SIZE(1)),

--Action Code indicating the purpose of the message

actionCode 		ENUMERATED {

secCxrMgmt(0),

priCxrChng(1)

},

-- Indication Type

indicationType 	ENUMERATED {

noAction(0),

deAct(1),

act(2),

actDeAct(3)

} OPTIONAL ,

-- Target Carrier List

tgtCxrMgmt 		TgtCxrLst OPTIONAL,

-- Activation deadline for secondary carrier activation

activeDeadline 	BIT STRING SIZE (6) OPTIONAL,

--Physical Carrier Index of Target Carrier

tgtPhyCxrIdx 		PhyCxrIdx OPTIONAL,

-- Action Time

axnTime 		BIT STRING SIZE (3) OPTIONAL,

-- Next State of serving primary carrier

-- serving carrier DeAct/Act after primary cxr chng

servCxrNxtState 	CxrState OPTIONAL

}

-- Carrier State

CxrState ::= ENUMERATED {

cxrDeAct(0),

cxrAct(1)}

--Physical Carrier Index

PhyCxrIdx ::= INTEGER (0 .. 763)

-- Number of Target Carriers

NumTargetCarrier ::= INTEGER (1..8) -- Range TBD

-- Target Carrier List TgtCxrLst ::= SEQUENCE (SIZE(NumTargetCarrier)) OF TgtCxrInfo

-- Target Carrier Index

TgtCxrIdx ::= INTEGER (1 .. 8)

--Activation-DeActivation Indication

ActDeActInd ::= BIT STRING SIZE(1)

-- Targer Carrier Info

TgtCxrInfo ::= SEQUENCE {

-- Target Carrier Index

tgtCxrIdx 	TgtCxrIdx,

-- Activation of DL - UL

-- 0: Both DL/UL Activated 1: Only DL is Activated

dlulAct 	ActDeActInd OPTIONAL,

-- DeActivation of DL - UL

-- 0: Both DL/UL DeActivated 1: Only UL is DeActivated

dlulDeAct 	ActDeActInd OPTIONAL,

-- Ranging Indicator for Target Carrier

-- 0: No rng reqd 1: Periodic rng reqd

rngInd 	BIT STRING SIZE(1) OPTIONAL

}

-- AAI_CM-IND message

-- Carrier Management Indication Message

AAI-CM-IND ::= SEQUENCE {

messageType 		OCTET STRING(SIZE(1)),

--Action Code indicating the purpose of the message

actionCode 		ENUMERATED {

secCxrMgmt(0),

priCxrChng(1)}

}

---------------------------------------------------------Start of the Text-----------------------------------------------------------