| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MC-ADV Message and ASN.1 Clean Up |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D6 |  |
| Abstract | The existing ASN.1 code needs further clean up |  |
| Purpose | For TGm discussion and adoption into P802.16m/D7 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MC-ADV Message and ASN.1 Clean Up

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#18, page#220 of P802.16m/D6]

16.2.3.56 AAI_MC-ADV (multicarrier Advertisement) Message

The MC ABS which supports the multicarrier operations depicted in 16.2.8 shall periodically broadcast AAI_MC-ADV message for the reception by all AMSs.

Table 761—Parameters for AAI_MC-ADV message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Multicarrier configuration change count | 4 | The value is incremented by 1 upon each update |  |
| M | Serving ABS carrier number | 3 | Indicates the number of carriers the supported by serving ABS carriers |  |
| M | MAC protocol version | 8 | Indicates MAC protocol version that the current carrier supports: it is consistent with REV.2 definition with new MAC protocol version 9 defined for 16m |  |
| O | Physical carrier index | 63 | Indicates the carrier index that the ABS carries except current carrier: the physical carrier index refers to AAI_Global-Config message | Shall be present when Serving ABS carrier number is more than 1 |
| O | SA-Preamble index | 10 | Indicates the SA-Preamble index of the indicated secondary carrier | Shall be present when Serving ABS carrier number is more than 1 |
| O | Paging carrier indicator | 1 | Indicates whether this carrier is a paging carrier or not 0: not paging carrier 1: paging carrier | May be present when multiple carrier operation is applied Serving ABS carrier number is more than 1 |
| O | SFH encoding format | 2 | 0b00: full SFH subpacket and AAI_SCD information 0b01: delta encoding of SFH_Info (SFH subpacket and AAI_SCD) with reference to the information of current carrier which transmits this AAI_MC-ADV message 0b10: delta encoding of SFH_Info (SFH subpacket and AAI_SCD) with reference to the information of preceding carrier 0b11: reserved | Shall be present when Serving ABS carrier number is more than 1 |
| O | S-SFH change count | 4 | Change count of S-SFH SP IE(s) | Shall be present when Serving ABS carrier number is more than 1 |
| O | Change count for AAI_SCD | 4 | Change configuration count of AAI_SCD | Shall be present when Serving ABS carrier number is more than 1 |
| O | Control bitmap | 4 | Each bit maps to one SFH subpacket and AAI_SCD information Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: AAI_SCD | Shall be present when SFH encoding format is set to 0b00 |
| O | SFH subpacket 1 | Variable depending on FFT size | SFH subpacket 1 is included | May Shall be present when SFH encoding format is set to 0b00 and the 1st MSB of control bitmap is set to 1 |
| O | SFH subpacket 2 | Variable depending on FFT size | SFH subpacket 2 is included | May Shall be present when SFH encoding format is set to 0b00 and the 2nd MSB of control bitmap is set to 1 |
| O | SFH subpacket 3 | Variable | SFH subpacket 3 is included | May Shall be present when SFH encoding format is set to 0b00 and the 3rd MSB of control bitmap is set to 1 |
| O | AAI_SCD_Info | Variable | Information from AAI_SCD is included | May Shall be present when SFH encoding format is set to 0b00 and the 4th MSB of control bitmap is set to 1 |
| O | Delta information | Variable | Delta encoding w.r.t. the reference carrier (the current carrier transmitting this message if SFH_encoding_format = 0b01, the preceding carrier if SFH_encoding_format=0b10) | May be present when SFH encoding format is set to 0b01 or 0b10 |

[Adopt the following text modifications from line#61, page#893 of P802.16m/D6]

-- AAI_MC-ADV message

-- MultiCarrier Advertisement Message

AAI-MC-ADV ::= SEQUENCE {

messageType 		OCTET STRING SIZE(1),

--Multi-Carrier Configuration Count

multiCxrCCnt 	INTEGER (1..16),

--Multi-Carrier Configuration

bsCxrInfo 		BsCxrInfoList

mcCfg 		McCfg

}

McCfg ::= SEQUENCE {

-- Serving BS Carrier Number

servBSCxrNum 	INTEGER ( 0 .. 7 ),

macVer 		MacVer,

-- BS Info List

bsInfo 		BsInfoList

}

--BsCxrInfo 1 -> (bsCxrNumber-1)

BsCxrInfoList ::= SEQUENCE (SIZE(1 .. 7)) OF BsCxrInfo

BsCxrInfo ::= SEQUENCE {

macVer 			MacVer,

phyCxrIdx 			PhyCxrIdx,

saPpreambleIdx 		PreambleIndexINTEGER (0 .. 1023),

pagingCrxIndicator	BOOLEAN		OPTIONAL,

sfhCCC 		INTEGER (0..15),

scdCCC 		INTEGER (0..15),

-- SFH Encoding Format

sfhEncFmt 		BIT STRING SIZE(2), CHOICE {

fullSubpkt			SFHSubpacket,

deltaInfoCurrentCxr	DeltaInfoCurrentCrx,

deltaInfoPrecedingCxr	DeltaInfoPrecedingCrx

}

-- Control Bitmap

ctrlBmp 		BIT STRING {

sfhSubPkt1(0),

sfhSubPkt2(1),

sfhSubPkt3(2),

sfhSubPkt4(3)

} SIZE(4) OPTIONAL,

-- SFH subpacket and AAI_SCD Information

sfhPkt1 		SEQUENCE { ??} OPTIONAL,

sfhPkt2 		SEQUENCE {..} OPTIONAL,

sfhPkt3 		SEQUENCE {..} OPTIONAL,

scdInfo SEQUENCE {..} OPTIONAL,

-- Delta Information

deltaInfo 		SEQUENCE { ... } OPTIONAL

}

MacVer ::= SEQUENCE {

verNumber INTEGER (0..255)

}

PhyCxrIdx 		::= INTEGER (0..763)

PreambleIndex 	::= INTEGER (0..1023)

SFHSubpacket	::= SEQUENCE {

sfhSubpkt1		SFHSubPacket1	OPTIONAL,

sfhSubpkt2		SFHSubPacket2	OPTIONAL,

sfhSubpkt3		SFHSubPacket3	OPTIONAL,

scdInfo		SCDInfo		OPTIONAL

}

SFHSubPacket1	::= SEQUENCE {

-- content TBD

}

SFHSubPacket2	::= SEQUENCE {

-- content TBD

}

SFHSubPacket3	::= SEQUENCE {

-- content TBD

}

SCDInfo		::= SEQUENCE {

-- content TBD

}

DeltaInfoCurrentCrx	::= SEQUENCE {

-- content TBD

}

DeltaInfoPrecedingCrx	::= SEQUENCE {

-- content TBD

}

---------------------------------------------------------Start of the Text-----------------------------------------------------------