| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text for MIMO OL region |  |
| Date Submitted | 2010-07-06 |  |
| Source(s) | Yanfeng Guan, Ding Ning, Huiying Fang, Zhaohua Lu ZTE Corporation | (guan.yanfeng, ning.ding, huiying.fang, zhaohua.lu )@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “Contribution in support of a comment on SB_16m”: Category – AWD Comments Area – Chapter 16.2.3 (MAC Control Messages) Chapter 16.3.5 (Downlink physical structure) Chapter 16.3.6 (Downlink Control Structure) Chapter 16.3.7 (Downlink MIMO) |  |
| Abstract | This contribution proposes some modifications to the parameters indicating the OL MIMO region configuration in the AAI System Configuration Descriptor message in Section 15.2.3.30, and also proposes the interpretation rules for these parameters. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 6) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification Text for MIMO OL region

Yanfeng Guan, Ding Ning, Huiying Fang, Zhaohua Lu

ZTE Corporation

- Introduction

In current P802.16m/D6, when the OL region type 0 is enabled, all the DLRUs in FP0 will be used for OL Region Type0. If the primary frequency partition also is located in FP0, the power of OL Region in different ABSs maybe different due to the different boosted power for A-MAP Region of different ABS. So, if different ABS will adjust the transmission power for A-MAPs according to the channel condition, the actual transmission can’t be controlled and then the interference in OL region 0 can’t be stable. At a result of that, OL region type 0 can’t offer a stable interference environment. In order to fix this problem, it is suggested to indicate the DLRU size for OL region type0 in the AAI SCD message.

There are 3 text modifications proposed in this contribution, each in a different section, and appear in the section order in [1]. Modification 1 makes changes to the parameters in the AAI_SCD message in Section 16.2.3.30. Modification 3 adds text to Section 16.3.6.5.1.2 (“S-SFH IE”). Modification 3 adds text to the MIMO Open Loop Region section, describing how the AAI_SCD parameters are to be interpreted to derive the structure of the OL region.

Text proposal for inclusion in the 802.16m amendment

- Proposed Text Modification 1

The OL Region parameters in the AAI System Configuration Description (AAI_SCD) message from line 33 to 54 on Page 157 in Section 16.2.3.30 are proposed to be modified as follows.

----------------------------------------------------------------------------- Begin --------------------------------------------------------------------------

Table 716 – AAI SCD Message

| --- | --- | --- | --- | --- | --- |
| --- | --- | --- | --- | --- | --- |
| O | OL_MIMO_Parameters | OL-Region-Type0_ONDLRU-Size | 14 | 0 or 1 to 15 | OL MIMO parameters |
| O | OL_MIMO_Parameters | OL-Region-Type1_NLRU-Size | 4 | 0 to 15 | OL MIMO parameters |
| O | OL_MIMO_Parameters | OL-Region-Type1_SLRU-Size | 4 | 0 to 15 | OL MIMO parameters |
| O | OL_MIMO_Parameters | OL-Region-Type2_SLRU-Size | 4 | 0 to 15 | OL MIMO parameters |
| --- |  | --- | --- | --- | --- |

---------------------------------------------------------------- End---------------------------------------------------------------

- Proposed Text Modification 2

OL Region Field in Table 841 (“S-SFH SP2 IE format”) on line 21 to 25 on Page 546 of Section 16.3.6.5.1.2 is proposed to be modified as follows.

----------------------------------------------------------- Begin -----------------------------------------------------------------

Table 841－S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| ----- |  |  |
| OLRegion OL Region | 1 | Provides indication about the structure of the MIMO OL Region. Further details in Section OL Region Indication, See 16.3.5.3.1 CRU/DRU Allocation. 0b0: Disable 0b1: Enable |
| ----- |  |  |
| } |  |  |

--------------------------------------------------------------------------- End -------------------------------------------------------------------------

- Proposed Text Modification 3

Change the text from line 63 to 65 on page 622 in Section 16.3.7.2.5.1 (“Open Loop Region”) as followed.

----------------------------------------------------------- Begin -----------------------------------------------------------------

The OL region type 0 is present if OL-Region-Type0-ON is indicated in a downlink broadcast message AAI_SCD message. All DRUs in frequency partition 0 will be in OL region type 0.

The OL region type 0 DLRU is present if OL-Region-Type0-DLRU-Size is greater than 0 in the downlink broadcast message AAI_SCD message. The PRUs allocated to type 0 OL region DLRU are the last (4* OL-Region-Type0-DLRU-Size) PRUs obtained by the mapping equation (191), i.e., PRUFP0(j), where j = LSB,FP0 + LMB,FP0 -(4 * OL-Region-Type0-DLRU-Size)to LSB,FP0 + LMB,FP0-1.

----------------------------------------------------------- End -----------------------------------------------------------------

References

[1] IEEE P802.16m/D6, “Part 16: Air Interface for Broadband Wireless Access Systems”, May 2010.