| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup on Switching to STID from TSTID during network entry(16.2.15.6) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Youngkyo Baek Youngbin Jang Kyeong-Tae Do Ji-Yun Seol HeeKwun Cho Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.15.6” |  |
| Abstract | This contribution proposes switching mechanism to STID from TSTID to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup on switching to STID from TSTID during network entry (16.2.15.6)

Youngkyo Baek, Youngbin Jang,Kyeong-Tae Do,Ji-Yun Seol,HeeKwun Cho

Samsung Electronics

# 1. Introduction

According to 16m/D6[1], in order to support AMS location privacy, ABS assigns two station identifers such as TSTID and STID. The TSTID is assinged by AAI_RNG-RSP during initial ranging procedure and it is used until STID is assigned by AAI_REG-RSP during registration procedure.

However, it may occur that AAI_REG-RSP is not successfully transmitted to the AMS but the ABS regards it as successful transmission. Then the ABS would switch to STID from TSTID but the AMS would still keep TSTID. Finally the link between AMS and ABS is lost.

Hence the switching machanism should guarantee for non-disrupted link connection and so we suggest using an AAI_MSG-ACK message as the following proposed text.

# 2. Text Proposal

Modify the sentences at line 43 page 381 as follows

======================== Start of Proposed Text =====================

The ABS shall allocate and transfer an STID to the AMS through an encrypted AAI_REG-RSP message.

The ABS triggers the AMS to transmit an AAI_MSG-ACK message by setting Polling bit to 1 in MCEH of the AAI_REG-RSP message. Upon receiving the AAI_REG-RSP, the AMS shall send a BR with STID header containing the STID to transmit AAI_MSG-ACK.

The ABS shall discard the temporary STID, which was allocated during initial ranging procedure, is discarded when the ABS recognizes it allocates uplink bandwidth with the STID for the AMS to transmit the AAI_MSG-ACK message that the AMS received the AAI_REG-RSP messages successfully. The AMS shall discard the temporary STID when it receives an Assignment A-MAP IE CRC-masked using the STID that was assigned through the encrypted AAI_REG-RSP message.

The AAI_REG-RSP message contains the CRID, where the network retains the AMS contexts including the CRID and the AMS MAC address.

============================== End of Proposed Text ===============

# 3. References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, MAY 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.