| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up on AAI_RNG message field description (16.2.3.1) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Youngkyo Baek Hyunjeong Kang Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.1” |  |
| Abstract | This contribution proposes cleanup on AAI_RNG message field description to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up on AAI_RNG message field description (16.2.3.1)

Youngkyo Baek, Hyunjeong Kang

Samsung Electronics

# 1. Introduction

AAI_RNG message cleanup is required as follows.

- According to 16m/D6[1], SMS may be transmitted by AAI_RNG-REQ and AAI_RNG-RSP messages in idle mode. But this SMS attribute is not contained in those messages’ field description.

- CMAC tuple should be the last attribute in the control messages to know the position of the CMAC value easily.

- Etc.

# 2. Text Proposal

Modify the table 679 at line 8 page 75 as follows

======================== Start of Proposed Text =====================

Table 679—AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| …… | ….. |  | ….. | ……. | ………… |
| O | CSGID(s) |  |  | One or more identifiers for one or more CSG(s) | Sent by AMS to aid quick CSG membership detection by Femto ABS as well as to aid ABS reselection |
| O | List of FIDs, and Change Count of each FID |  | 72 | List of FIDs which the AMS currently has, and Change Count of each FID perEachFID SEQUENCE { fid INTEGER (0 .. 15), fidchangecount INTEGER (0 .. 15) } | Shall be included if Ranging Purpose Indication value is set to 0b0000 and Paging Controller ID is absent, |
| O | SMS |  | variable | Short Message content up to the size of 140 bytes(table 801) | May be included when short message service contents is sent in idle mode. |
| O | CMAC tuple | PMKSN | 4 | The sequence number of PMK used to derive the current AK. | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication. It shall be the last attribute in the AAI_RNG-REQ message. |
| O |  | CMAC_PN_U | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
| O |  | CMAC value | 64 | The CMAC value derived from the whole message. |  |
| O | List of FIDs, and Change Count of each FID |  | 72 | List of FIDs which the AMS currently has, and Change Count of each FID perEachFID SEQUENCE { fid INTEGER (0 .. 15), fidchangecount INTEGER (0 .. 15) } | Shall be included if Ranging Purpose Indication value is set to 0b0000 and Paging Controller ID is absent, |

============================== End of Proposed Text ===============

Modify the table 680 at line 59 page 79 as follows

======================== Start of Proposed Text =====================

Table 680—AAI_RNG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| …… | ….. |  | ….. | ……. | ………… |
| O | Redirection Info[0..256] | ABSID for neighbor ABS | 48 | ABSID, preamble index and center frequency for one or more neighbor ABSs of the serving ABS | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry (due to various reasons such as high serving ABS load, non existence of CSG membership etc) |
| O |  | preamble index for neighbor ABS | 10 |  |  |
| O |  | center frequency for neighbor ABS | 32 |  |  |
| …… | ….. |  | ….. | ……. | ………… |
| O | SMS |  | variable | Short Message content up to the size of 140 bytes(table 801) | May be included when short message service contents is sent in idle mode. |

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, MAY 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.