| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up for AAI_REG-REQ/RSP messages (16.2.3) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Youngkyo Baek Hyeonjeong Kang Jicheol Lee Jungshin Park Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3” |  |
| Abstract | This contribution provides Clean up AAI_REG-REQ/RSP messages |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up AAI_REG-REQ/RSP messages (16.2.3)

Youngkyo Baek, Hyeonjeong Kang, Jicheol Lee, Jungshin Park

Samsung Electronics

# Introduction

Table 687 and Table 688 describe parameter s to be negotiated by AAI_REG-REQ and AAI_REG-RSP, respectively.

But since some parameters are missing and some boxes are still blank, we suggest clean-up those messages as the proposed texts.

The folowings are list of suggestion.

- For some attributes, modify M/O types and their conditions.

- Include some missing attributes in AAI_REG-REQ/RSP(e.g. The Number of Uplink transport FID Supported, Total number of provisioned service flow, etc.)

- Include some missing attributes which AAI_REG-RSP has, but AAI_REG-REQ does not have.( e.g. Persistent Allocation support, Group Resource Allocation support, HO trigger metric, etc.)

- Remove some attribute(e.g. OFDMA supports,3-step BR etc)

- Move ‘Max Tx power’ to AAI_SBC-REQ message.

# Proposed Text

Modify the sentences and tables, line 1, page 95 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

16.2.3.7 AAI_REG-REQ

An AAI_REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

The AAI_REG-REQ message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

The following parameters may be included in AMS capability negotiation parameters of AAI_REG-REQ.

•AMS initiated aGP Service Adaptation Capability:

0b0: no support

0b1: support

Table 687 - AAI_REG-REQ message Field Descriptions

[Note to editor:

- Fill with‘O’ in the column ‘M/O’ for ‘Host configuration capabilities and parameters’,

- But, Fill with ‘M’ for the other attributes.

- Fill ‘N.A.’ in the column ‘condition’ for each attributes marked as ‘M’ in the column ‘M/O’.]

| M/O | Attributes / Array of attributes |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | AMS MAC address |  |  |  | 48 | AMS’s real MAC address This is used to derive security keys | N.A. |
| … | . |  |  |  | … | …….. | ……. |
| O |  | frame configuration to support legacy(5MHz) |  |  | 15 | If Bit#0 =1,it R1 5MHz supports (TDD only) If Bit#1 =1, R1 10MHz supports If Bit#2 =1, R1 8.75MHz supports If Bit#3 =1, R1 7MHz supports If Bit#4 =1, UL TDM supports If Bit#5 =1, UL FDM supports |  |
| O |  | frame configuration to support legacy(10MHz) |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | MAX Tx Power |  |  | 24 | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. |  |
| … | … | … |  |  | … | … | … |
| O |  | 3-step BR |  |  | 1 | If Bit#0 =1,it supports |  |
| … | … | … |  |  | … | … | … |
| O |  | Capabilities for Handover EBB mode |  | Minimal HO_Reentry_Interleaving_Interval | 1 | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1. |  |
| M | AMS initiated aGP Service Adaptation Capability: |  |  |  | 1 | 0b0: no support 0b1: support | N.A. |
| OM | Convergence sublayer capabilities |  | Classification/PHS options and SDU encapsulation support CS type support |  | 16 | It indicates which CS type(s) the AMS supports( refer to table xxx CS type to support) | N.A. |
| OM | Convergence sublayer capabilities |  | Maximum number of classification rules |  | 2 | Maximum number of simultaneous admitted classification rules that a service flow supports. | N.A. |
| … | . |  |  |  | … | …….. | ……. |
| O | Host configuration capabilities and parameters |  | Host-Configuration-Capability-Indicator IE |  | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message.(One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. 0b0: not support 0b1: support | To be included when Requested-Host-Configurations IE is not included |
| O | Host configuration capabilities and parameters |  | Requested-Host-Configurations IE |  | 1variable | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. | To be included when additional host configurations are required. |
| … | . |  |  |  | … | …….. | ……. |
| M | The Number of Uplink transport FID Supported |  |  |  | 4 | The number of uplink transport FIDs the AMS supports | N.A. |
| M | The Number of Downlink transport FID Supported |  |  |  | 4 | The number of donwlink transport FIDs the AMS supports | N.A. |

Table xxx – CS type to support

| Bit | CS | CS subpart | Traffic constraint |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | Packet (5.2) | IP (5.2.5) | IPv4 Traffic Only |
| 2 | Packet (5.2) | IP (5.2.5) | IPv6 Traffic Only |
| 3 | Reserved |  |  |
| 4 | Reserved |  |  |
| 5 | Reserved |  |  |
| 6 | Reserved |  |  |
| 7 | Reserved |  |  |
| 8 | Reserved |  |  |
| 9 | Reserved |  |  |
| 10 | Reserved |  |  |
| 11 | Reserved |  |  |
| 12 | Reserved |  |  |
| 13 | Reserved |  |  |
| 14 | Packet (5.2) | IP (5.2.5) | None |
| 15 | Packet (5.2) | Multiprotocol (5.2.6) | IPv4 or IPv6 Traffic |

16.2.3.8 AAI_REG-RSP

An AAI_REG-RSP message is transmitted by ABS in response to AAI_ REG-REQ message during initialization.

The AAI_REG-RSP message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

Table 688 - AAI_REG-RSPmessage Field Descriptions

[Note to editor :

- fill ‘O’ in the column ‘M/O’ for ‘Host configuration capabilities and parameters’ and ‘femto BS LDM parameters’ and ‘redirection Info’,

- but, fill with ‘M’ for the other attributes.

- Fill ‘N.A.’ in the column ‘condition’ for each attributes marked as ‘M’ in the column ‘M/O’]

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | STID |  | 48 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. | N.A. |
| MO | CRID |  | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode | To be included when the DCR mode or coverage loss recovery is supported |
|  | A2)Femto ABS LDM parameters |  |  |  | to be included when the femto ABS support LDM |
| O | A.2.1) Start_superframe_offset |  | 9 | The 9 LSB of the start superframe number |  |
| O | A.2.2) Available_interval_length |  | 4 | Length of available interval in units of four frames |  |
| O | A.2.3) Unavailable_interval_length |  | 8 | Length of unavailable intervals in units of four frames |  |
| … | . |  | … | …….. | ……. |
| M | Persistent Allocation support |  | 1 | If Bit#0 =1,it supports | N.A. |
| M | Group Resource Allocation support |  | 1 | If Bit#0 =1,it supports | N.A. |
| M | HO trigger metric |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | N.A. |
| M | AMS initiated aGP Service Adaptation Capability: |  | 1 | 0b0: no support 0b1: support | N.A. |
| … | . |  | … | …….. | ……. |
| OM | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support CS type support | 16 | It indicates which CS type(s) the ABS supports for AMS( refer to table xxx CS type to support) Indicates which classification/PHS options and SDU encapsulation the AMS supports | N.A. |
| OM | Convergence sublayer capabilities | Maximum number of classification rules | 2 | Maximum number of simultaneous admitted classification rules that a service flow supports. | N.A. |
| … | . |  | … | …….. | ……. |
| O | Host configuration capabilities and its parameters | IPv4-Host-Address IE | 32 | The allocated IPv4 Host Address for the AMS. | May be included when AMS indicates its capability of configuring host parameters |
| O | Host configuration capabilities and its parameters | IPv6-Home-Network-Prefix IE | 64 | The allocated IPv6 Home Network Prefix for the AMS. | May be included when AMS indicates its capability of configuring host parameters |
| O | Host configuration capabilities and its parameters | Additional-Host-Configurations IE | variable | Includes additional host configuration options in DHCP Options format | May be included when AMS sends Requested-Host-Configurations IE in AAI_REG-REQ or network decided to configure Additional-Host-Configuration parameters |
| O | Redirection Info[0..256] | ABSID, preamble index and center frequency for one or more neighbor ABS ABSID for neighbor ABS | 48 | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. | May be included when the AMS does not support CSG whitelist capability or does not have any CSGID(s) provisioned in its CSG whitelist |
|  |  | preamble index for neighbor ABS | 10 |  |  |
|  |  | center frequency for neighbor ABS | 32 |  |  |
| M | The Number of Uplink transport FID Supported |  | 4 | The number of uplink transport FIDs the ABS supports | N.A. |
| M | The Number of Downlink transport FID Supported |  | 4 | The number of donwlink transport FIDs the ABS supports | N.A. |
| M | Total number of provisioned service flow |  | 4 | Total number of provisioned service flows to be setup for an AMS | N.A. |
| … | . |  | … | …….. | ……. |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

Add the attribute to the tables 682 , line 16, page 89 as follows.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

Table 682—AAI_SBC-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CAPABILITY_INDEX | 8 | It refers to the "Capability Class" that the AMS can support |  |
| …… | …. | . | ….. | … |
| O | MAX Tx Power | 24 | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. |  |
| …… | …. | . | ….. | … |

----------------------------------------------------- End of Proposed Text ---------------------------------------------------

# References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.