| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Precapability negotiation for zone switch and direct HO (16.2.15.6) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Youngkyo Baek Jungshin Park Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.15.6” |  |
| Abstract | This contribution proposes precapability negotiation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Precapability negotiation for zone switch and direct HO (16.2.15.6)

Youngkyo Baek, Jungshin Park, Jicheol Lee

Samsung Electronics

# 1. Introduction

AMS may perform Zone switch from Mzone to Lzone or direct HO from Mzone to R1BS.

In those cases AMS’s R1 capabilities has to be negotiated at the target zone or target BS. That means additional control message transactions are required besides RNG message transaction because R1 capabilities are different from 16m capability.

But, if the AMS’s R1 capabilities are pre-neogotiated at the serving ABS, additional control message transactions for capability negotiation are not required.

We suggest that when the AMS access to the mixed ABS, if it perform network entry to M zone then it pre-negotiate the AMS capability for WirelessMAN-OFDMA reference system

# 2. Text Proposal

Modify the sentences and table 687 page 96 as follows.

======================== Start of Proposed Text#1 =====================

Table 687 - AAI_REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | AMS MAC address | 48 | This is used to derive security keys |  |
| … | . | … | …….. | ……. |
| O | R1 SBC-REQ capability parameters | variable | This is the TLV encodings of the SBC-REQ message to be negotiated in wirelessMAN-OFDMA R1 reference system | Shall be included when R1 SBC parameters are prenegotiated |
| O | R1 REG-REQ capability parameters | variable | This is the TLV encodings of the REG-REQ message to be negotiated in wirelessMAN-OFDMA R1 reference system | Shall be included when R1 REG parameters are pre-negotiated |

============================== End of Proposed Text #1===============

Modify the sentences and table 688 page 102 as follows.

======================== Start of Proposed Text#2 =====================

Table 688 - AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | STID | 48 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. |  |
| … | . | … | …….. | ……. |
| O | R1 SBC-RSP capability parameters | variable | This is the TLV encodings of the SBC-RSP message to be sent in response to SBC-REQ in wirelessMAN-OFDMA R1 reference system | Shall be included when R1 SBC parameters are prenegotiated |
| O | R1 REG-RSP capability parameters | variable | This is the TLV encodings of the REG-RSP message to be sent in response to REG-REQ in wirelessMAN-OFDMA R1 reference system | Shall be included when R1 REG parameters are pre-negotiated |

============================== End of Proposed Text #2===============

Modify the sentences line 11 page 381 as follows.

======================== Start of Proposed Text#3 =====================

16.2.15.6 Registration

Registration is the process by which the AMS is allowed to enter into the network.

After authorization and key exchange are finished, the AMS informs the ABS of its capabilities and requests the registration for entry into the network by AAI_REG-REQ. If an ABS receives an AAI_REG-REQ, the ABS shall respond with an AAI_REG-RSP.

In the AAI_REG-REQ, the AMS informs the ABS of its capability parameters to be negotiated except those capabilities already negotiated with the ABS through AAI_SBC-REQ/RSP messages. In AAI_REG-RSP, the ABS responds with the accepted capability parameters. If the AMS omits some capability parameters in the AAI_REG-REQ, the ABS considers the AMS follows the default values for those parameters and, if acceptable, the ABS may omit those parameters applying default value in its AAI_REG-RSP.

When the AMS performs network entry in a mixed mode ABS, AMS’s capabilities in WirelessMAN OFDMA R1 reference system can be negotiated during registration procedure. Especially AMS capabilities in SBC and REG message to be negotiated in WirelessMAN OFDMA R1 reference system are negotiated through AAI_REG messages.

============================== End of Proposed Text #3===============

# 3. References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, MAY. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.