| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up on AMSID* derivation(16.2.5.2.1.4) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.5.2.1.4” |  |
| Abstract | This contribution proposes cleanup on AMSID* derivation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up on AMSID* derivation (16.2.5.2.1.4)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

# 1. Introduction

AMSID* is derived based on the MS MAC address and a 64-bit random number NONCE_AMS. The description mentions the NONCE_AMS is the same as the random NONCE_AMS in keyagreement MSG#2 message. However there is no advantange on that binding and the binding leads easily to misunderstanding. (i.e. it looks AMSID* is changed whenever keyagreement occurs, but it’s not true)

Hence, I suggest removing the binding between AMSID* and key agreement procedure.

# 2. Text Proposal

Modify the sentences at page 244, line 43 as follows

======================== Start of Proposed Text#1 =====================

The hanshake procedure (as shown in Figure 400) includes the following steps:

•EAP authentication completes (Authenticator got "EAP Success" from AAA and sent it to AMS). Assuming AMS received the EAP_Success message, both AMS and ABS suppose to have valid AK and derive CMAC keys.

•The ABS sends CMAC'ed AAI_PKM_RSP (Key Agreement MSG#1) to the AMS. The message includes a random NONCE_ABS.

•If the AMS receives MSG#1 without receiving the EAP_Success before it SHALL query the supplicant for the current MSK and SHALL calculate the CMAC tuple based on it. If CMAC verification fails, the MS shall silently discard MSG#1. If CMAC verification is successful, the AMS shall sends AAI_PKM-REQ (handshake MSG#2) using the MSK and derived keys and including the random NONCE_ABS and NONCE_AMS and security negotiation parameters to the ABS. In case of key agreement procedure during network entry, security negotiation parameters are included also in the Key agreement MSG#2. In the case S-SFH Network Configuration bit = 0b0, NONCE_AMS shall be the one used to derive AMSID*. In the case of Legacy Support Mode is True; NONCE_AMS is not used in any key derivation.

============================== End of Proposed Text #1===============

Modify the sentences at page 245, line 13 as follows

======================== Start of Proposed Text#2 =====================

Note that supplying the AMSID to the ABS allows, among other used of AMSID, for the NW elements to calculate AMSID* whenever a new AK needs to be derived from PMK (HO for example).

============================== End of Proposed Text#2 ===============

Modify the sentences at page 270, line 36 as follows

======================== Start of Proposed Text#3 =====================

To protect AMSID the AMS shall define a hash value of the real AMSID (i.e. AMSID*) is defined and transmit it to the ABS by AAI_RNG-REQ during network entry, which is used to derive AK, for the case of S-SFH Network Configuration bit = 0b0 as follows:

AMSID*=Dot16KDF(AMSID|80-bit zero padding, NONCE_AMS, 48)

•NONCE_AMS is a random 648-bit value generated by AMS before sending AAI_RNG-REQ message, and transmitted to ABS during the following Key Agreement 3-way handshake procedure. If the AMS doesn't receive a successful AAI_RNG-RSP from the ABS, the AMS should send another AAI_RNG-REQ with the AMSID* derived from the same NONCE_AMS to the ABS in the followed initial ranging procedure before retries are exhausted. If retries are exhausted, AMS should use another AMSID* derived from a newly generated NONCE_AMS.

============================== End of Proposed Text#3 ===============

# 3. References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, MAY. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.