| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on CRID update method(16.2.3.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.2” |  |
| Abstract | This contribution proposes CRID update scheme to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on CRID update method (16.2.3.2)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

# 1. Introduction

According to 16m/D6[1], CRID is used for MS identification when the AMS performs Network reentry from the coverage loss/DCR mode. And this CRID contains information which network entity manages and AMS’s identifier. Due to some reasons the network entity may be changed so that the CRID has to be refreshed.

We propose a mechanism to refresh the CRID.

# 2. Solution

We suggest using an unsolicited AAI_RNG-RSP containing a new CRID for CRID update.

When the network entity wants to update CRID during network reentry, AAI_RNG-RSP in response to AAI_RNG-REQ is used for CRID update.

Meanwhile, even when the network entity is changed without AAI_RNG transactions and CRID update is required(e.g. anchor authenticatior relocation without re-authentication under assumption that the anchor authenticator manages CRID) we suggest that ABS send unsolicited AAI_RNG-RSP containing a new CRID. For confirmation the ABS requests that AMS respond with AAI_MSG-ACK.

# 3. Text Proposal

Add some description at line 41 page 79 and modify the table 680 at line 49 page 80 as follows

======================== Start of Proposed Text =====================

16.2.3.2 AAI_RNG-RSP

An AAI_RNG-RSP shall be transmitted by the ABS in response to a received AAI_RNG-REQ. HARQ operation is applied to the DL burst including the AAI_RNG-RSP message. In contention-based random access ranging procedure, the AAI_RNG-RSP message is allocated using CDMA Allocation A-MAP IE only when the ABS transmits the AAI_RNG-RSP message to an AMS that has no STID or TSTID assigned.

When ABS sends the AMS an AAI_RNG-RSP in response to an AAI_RNG-REQ if network assigns a new CRID to the AMS then the AAI_RNG-RSP shall contain the new CRID.

When network assigns a new CRID to the AMS separately from NW reentry or location update procedure, ABS sends the AMS an unsolicited encrypted AAI_RNG-RSP containing the new CRID and should set polling bit by 1 in MCEH so that the AMS shall send acknowledgment (i.e., AAI_MSG-ACK) to the AAI_RNG-RSP.

The AAI_RNG-RSP message shall be encrypted and not contain CMAC Tuple, when the ABS notifies the AMS through the HO Process Optimization parameter that the AAI_PKM-REQ/RSP sequence may be omitted for the current HO reentry attempt, or when the ABS wishes to respond to the acknowledged AAI_RNG-REQ message containing a valid CMAC.

In case of Zone Switch procedure from LZone to MZone, the AAI_RNG-RSP message, if specific FID(s) is to be re-mapped to SAID of AES-CTR, the ABS shall include the 'SAID_update_bitmap' in the AAI_RNG-RSP message.

Table 680—AAI_RNG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| …… | ….. | ….. | ……. | ………… |
| O | CRID | 72 | AMS identifier which the network assigns new to the AMS has been assigned for coverage loss or DCR mode and are currently maintained | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform network reentry/location update/zone-switch with assigning a new CRID in response to AAI_RNG-REQ or it shall be included when the network assigns a new CRID unsolicitedly to the AMS detects the restart count in AAI_SCD message different from old one save in AMS. |
| …… | ….. | ….. | ……. | ………… |

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, MAY 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.