| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up on Scanning procedure for Femtocell (sections 16.4.8.1.2) |  |
| Date Submitted | 2010-07-02 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7188 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract | This contribution is to propose text changes for 16.4.8.1.2 |  |
| Purpose | Update P802.16m/D6 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up on Scanning procedure for Femtocell (sections 16.4.8.1.2)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to clarify the procedure for femtocell scanning operation.

- Rationale/Motivation of modifications

The procedure for receiving an accessible list of CSG/OSG femtocells are possible by AAI_SCN-REQ message and AAI_SCN-REP message, which is duplicate and confusing. We suggest to allow AMS to receive an accessible femtocell list by message exchange of AAI_SCN-REQ/RSP messages only.

- References

[1] IEEE P802.16m/D6, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D6, page 795, line 62, add proposed text in section 16.4.8.1.2 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

...

After scanning and identifying the existence of any Femto ABSs, the AMS may report FA, A-Preamble index or BS IDs and measurement results according to the Trigger conditions included in the AAI_SCD message by sending AAI_SCN-REP. The AAI_SCN-REP may contain the neighbor request indication, to which the ABS may unicast an AAI_NBR-ADV that includes a list of femto ABSs which is formed based on the reported FA, A-preamble indexor BSIDs, or the reported measurement

If the AMS decides to perform HO to any Femto ABS, it may request more detailed system information of the detected neighbor Femto ABSs to the serving ABS using an AAI_NBR-REQ message.

--------------------------------------------------------Text End -----------------------------------------------------