| Le Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Carrier switching operation for E-MBS receiving |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Nan Li，Kaiying Lv，Bo Sun Feng Xie，Yuqin Chen ZTE Corporation | E-mail: lv.kaiying@zte.com.cn |
| Re: | Sponsor ballot comment on P802.16m/D6 |  |
| Abstract | This contribution proposes texts on carrier switching operation for E-MBS receiving in connected state. |  |
| Purpose | To be discussed and adopted. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Carrier switching operation for E-MBS receiving

Nan Li, Kaiying Lv, Bo Sun, Feng Xie，Yuqin Chen

ZTE Corporation

# 1. Problem Statement

According to 802.16m/D6, if a carrier-switching enabled AMS subscribes E-MBS service flows transmitted on carriers other than the AMS’s primary carrier, it shall perform carrier switching to receive those E-MBS service flows. However, in EMBS deployment, based on EMBS application design, the AMS doesn’t need to receive and decode all subscribed service flows simultaneously. While in this case, the ABS, in current D6, could not detect when exactly the AMS receives its favorite EMBS service flows and when is possible to schedule unicast services for the AMS. The ABS has to assume that the AMS receive all subscribed service flows and then schedule unicast services for this AMS with lowest scheduling efficiency. Especially for free Broadcast service which does not need any join procedure in application layer, there will be no DSA procedure in the air interface between the ABS and the AMS. Neither can the ABS schedule unicast services efficiently under this scenario.

# 2. Proposed Schemes

We propose that for a carrier switching enabled AMS, which performs primary to secondary carrier switching for E-MBS, before the AMS start receiving EMBS data or the AMS decides to change its current receiving service flows, it shall send the information about which E-MBS service(s) it intends to receive to the ABS before switching.

To provide this information, an E-MBS Connection Bitmap combined with an E-MBS Zone Bitmap could be used. Before switching, the AMS sends an E-MBS Connection Bitmap in an AAI_EMBS-REV-IND message to the ABS. Each bit of the E-MBS Connection Bitmap maps to an E-MBS connection. The LSB of the bitmap presents the E-MBS connection with the lowest E-MBS ID +FID value. The E-MBS service(s) the AMS has subscribed are mapped in ascending order of their E-MBS ID+FID value from LSB to MSB of the bitmap. The corresponding bit will be set to 1 if the AMS is going to receive relative E-MBS service. A value 0 means the AMS does not intend to receive the E-MBS service. An E-MBS Zone Bitmap could also be included to indicate which zone a specified E-MBS connection belongs to. An example of E-MBS Connection Bitmap is shown as follows.

Table 1 AAI_EMBS-REV-IND parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | 0b0: E-MBS Connection Bitmap has been changed 0b1:AMS stops receiving E-MBS service |  |
| O | E-MBS Zone Bitmap | 7 |  | Shall be included if the value of Action code is 0 |
| O | E-MBS Connection Bitmap | 16 |  | Shall be included if the value of Action code is 0 |

After receiving an AAI_EMBS-REV-IND message, the ABS transmits an AAI_EMBS-REV-RSP message in response. An Action Time may be included in the AAI_EMBS-REV-RSP message to indicate the time the ABS allows the AMS to start carrier switching. When the AMS changes the concurrent receiving E-MBS service(s), it sends the message with an updated E-MBS ID +FID list/ E-MBS Connection Bitmap to the ABS.

Meanwhile, the ABS should pre-define a minimal time interval for the AMS to perform carrier switching. After decoding AAI_E-MBS-CFG Message, E-MBS MAP Message or E-MBS service data burst, if the time interval between the current E-MBS relative control message / E-MBS data burst and the next one is shorter than or equal to the minimal time interval, the AMS should not switch back to the primary carrier. And the ABS should not consider the AMS is available on the primary carrier.

# 3. Text Proposal

======================== Start of Proposed Text =====================

[Change text in P848, Line 24, 16.9.2.2 E-MBS Operation in Connected State as follows:]

…

During the transmission of EMBS configuration messages, the E-MBS MAP and the EMBS data to which AMS is subscribed, the AMS with only one transceiver may not be available for signaling exchange with ABS on the primary carrier. The AMS sends an AAI_EMBS-REV-IND message to the ABS to inform the ABS which E-MBS service(s) it intends to receive. After receiving an AAI_EMBS-REV-IND message, the ABS transmits an AAI_EMBS-REV-RSP message in response. An Action Time may be included in the AAI_EMBS-REV-RSP message to indicate the time the ABS allows the AMS to start performing carrier switching. The ABS schedules unicast services for the AMS on its primary carrier based on AAI_EMBS-REV-IND message. When the AMS changes the concurrent receiving E-MBS service(s), it sends the message with an updated E-MBS Connection Bitmap and/or E-MBS Zone Bitmap.

The ABS pre-defines a minimal time interval (e.g., 3 subframes) for indicating whether the AMS needs to perform carrier switching back to primary carrier. After decoding AAI_E-MBS-CFG Message, E-MBS MAP Message or E-MBS service data burst, if AMS detects the time interval between the current E-MBS relative control message / E-MBS data burst and the next one is shorter than or equal to the minimal time interval, the AMS should not switch back to the primary carrier. The ABS should not assume the AMS is available on the primary carrier.

…

[Insert the following text to 16.2.3:]

16.2.3.x AAI_EMBS-REV-IND

The AMS sends an AAI_EMBS-REV-IND message to inform the ABS which E-MBS service(s) the AMS intends to receive. Table XXX shows the parameters for AAI_EMBS-REV-IND message:

Table XXX AAI_EMBS-REV-IND parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | 0b0: E-MBS Connection Bitmap indicating currently receiving services 0b1: AMS stops receiving E-MBS service |  |
| O | E-MBS Zone Bitmap | 7 | Each bit of the bitmap presents an E-MBS zone supported by the ABS. The E-MBS zones are mapped from LSB to MSB in ascending order of the value of Zone ID. For each bit: Value 0: The AMS does not intend to receive E-MBS service in the specific zone. Value 1: The AMS is going to receive E-MBS service in the specific zone. | Shall be included if the value of Action code is 0 |
| O | E-MBS Connection Bitmap | 16 | Each bit of the bitmap presents an E-MBS connection in one E-MBS Zone. The E-MBS service(s) are mapped in ascending order of their E-MBS ID+FID value from LSB to MSB of the bitmap. For each bit: Value 0: The AMS does not intend to receive the E-MBS service. Value 1: The AMS is going to receive relative E-MBS service. | Shall be included if the value of Action code is 0. Shall be included for each E-MBS zone as indicated in E-MBS Zone Bitmap with value of “1”. |

16.2.3.y AAI_EMBS-REV-RSP

The AAI_EMBS-REV-RSP message is transmitted by the ABS in response to an AAI_EMBS-REV-IND message sent by the AMS.

Table XXX AAI_EMBS-REV-RSP parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value/Notes | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 1 | 0b0：confirm AMS’s switching 0b1：refuse AMS to leave the primary carrier or confirm AMS of stopping receiving E-MBS service |  |
| O | Action Time | 4 | Indicates the switching time by frame number the AMS could start carrier switching. | May be included when action code=0b0. |

============================== End of Proposed Text ============================