| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text to clarify the NBR-REQ message in femto cell (16.2.3.36) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Namgi Kim Kyonggi University Soojung Jung, Eunkyung Kim, Jaesun Cha, Hyun Lee, Chulsik Yoon ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: sjjung@etri.re.kr |
| SB: | IEEE 802.16 Sponsor Ballot on P802.16m/D6 |  |
| Abstract | This contribution provides the clarification for the cell bar bit in femto cell |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text to clarify the NBR-REQ message in femto cell (16.2.3.36)

Namgi Kim

Kyonggi University

Soojung Jung, Eunkyung Kim, Jaesun Cha, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

According to "IEEE 802.16m/D6"[1], an AMS may request neighbor Femto ABS list by sending the AAI_NBR-REQ message to the ABS. In the AAI_NBR-REQ message, there are BSID and Request BS Type fields. If the AMS know the BSID of the Femto ABS, it may send the AAI_NBR-REQ message with the BSID field only. If an AMS does not know the BSID of the nearby accessible Femto ABS and wants to know the information of the Femto ABSs corresponding to the specific femto type, it may send the message with the Request BS Type field only. Therefore, the BSID and the Request BS Type fields shall be optional.

Second, the AMS may want to know the CSG-Closed type Femto ABS list only or want to request the CSG-Open and OSG type Femto ABSs simultaneously. However, there is no way to implement these requests in the current draft. Therefore, we propose to divide the Request BS Type into CSG-Closed, CSG-Open, and OSG, and present the Request BS Type field as the bitmap for enabling to request more than two femto types simultaneously.

Third, according to the section 16.4.8.1.2, the AMS may include CSGID(s) of the subscribed CSG femto BSs in the AAI_NBR-REQ message. Therefore, we add the CSGID(s) field at the end of the AAI_NBR-REQ message.

# References

[1] IEEE P802.16m/D6, “Advanced Air Interface (Draft)”, Jun. 2010.

# Proposed Text

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

-------- Start of the text proposal ---------------------------------------------------------------------------------------------

### 16.4.7.3 Femto ABS access restrictions

An AMS may send the AAI_NBR-REQ message to request the AAI_NBR-ADV message. The request should not contain the BSIDs for which full information is already provided in the AAI_NBR-ADV message. The AAI_NBR-REQ message may contain the request type indication to request the neighbor BS information with the specified type.

The AAI_NBR_REQ message shall include following parameters:

- LSB of Requested BS ID: 16 bits

- 16-bit LSB of unique 48-bit identifier of neighbor BS

- Request BS Type: 2bits 4bit bitmap

- 00: CSG Femto ABS

- 01: OSG Femto ABS

- 10-11: reserved

- Bit #0: CSG-Closed Femto ABS

- Bit #1: CSG-Open Femto ABS

- Bit #2: OSG Femto ABS

- Bit #3: reserved

- CSGID(s): One or more identifiers for one or more CSG(s)

Table 722-AAI_NBR-REQ message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M O | BSID | 16 | 16-bit LSB of unique 48-bit identifier of neighbor BS | NA |
| M O | Request BS type | 2 4 | 00: CSG Femto ABS 01: OSG Femto ABS 10-11: reserved Bitmap to indicate Request BS type Bit #0: CSG-Closed Femto ABS Bit #1: CSG-Open Femto ABS Bit #2: OSG Femto ABS Bit #3: reserved | NA |
| O | CSGID(s) |  | One or more identifiers for one or more CSG(s) | NA |

-------- End of the text proposal ----------------------------------------------------------------------------------------------