| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Contention-based Bandwidth Request Random Backoff Parameters (16.2.3.46, 16.2.11.1, 16.2.12.8) |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Anurag Nishad, Abhay Karandikar, Prateek Kapadia | TICET (IIT Bombay) |
| Re: |  |  |
| Abstract | Contention-based Bandwidth Request Random Backoff Parameters are specified in relevant MAC management messages |  |
| Purpose | Accept the revisions proposed in this contribution |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Random Backoff Parameters for Contention-based Bandwidth Request

This document includes the parameters of connection priority in DSx MAC control message in section 16.2.3.46, table 745, at page number 200 of D6. These parameters are copied over from Table 786 (Service Flow Parameters) in Section 16.2.12.8.

[Note to Editor: Please modify the text in section 16.2.3.46.1, 16.2.3.46.2, 16.2.3.46.4 and 16.2.3.46.5 follows]

(Original Text: Black, New Text: Blue, Deleted Text: Red, strike-through)

--------- Start of text to add to Table 745, AAI_DSC-REQ, Section 16.2.46.4--------------

| M/O | Attributes / Arrays of attributes | Size(bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Initial Backoff Window Size | 4 | Window size expressed as a power of 2 | For Uplink only |
| O | Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 | For Uplink only |
| O | Backoff Scaling Factor | 4 | "0010" for Binary Exponential Backoff | For Uplink only |

-------------------------- End of text to add to Table 745, Section 16.2.46.4 ------------------------------------